3GPP TSG-RAN WG2 #131bis	R2-2507619

Prague, Czech Republic, 13th – 17th October 2025

Source:	NTT DOCOMO, INC.

Title:	Initial discussion on introduction of Topology 2

Document for:	Discussion

Agenda Item:	9.2.2

- Introduction

This contribution is to provide our views on kick-off of normative work for introduction of Topology 2.

- Discussion

  - Analysis of need for coordination between UE and gNB

In the Ambient IoT system under Release 20 Topology 2, it is preferable to reuse the A-IoT air interface from Release 19 Topology 1 as much as possible. In particular, regarding the upper layers, all functionalities that were supported by the A-IoT MAC layer in Release 19 should also be supported in Release 20 Topology 2.

Based on this premise, this section analyzes all the messages delivered by the MAC layer in Release 19 Ambient IoT and identifies the information elements that are likely to require message exchange between the UE and gNB when the reader is replaced by a UE.

| Message | Information element | Need for coordination between UE and gNB |
| --- | --- | --- |
| Paging | R2D Message Type | No. |
|  | R2D TBS | No. |
|  | Transaction ID | Since the reader delivers Transaction ID from Correlation ID generated in AIOTF, the gNB needs to forward the Correlation ID received from the CN to the UE reader. This information may not have to be visible by the gNB. |
|  | Paging ID | The gNB needs to forward Paging ID received from the CN to the UE reader. This information may not have to be visible by the gNB. |
|  | D2R Scheduling Info | The gNB needs to allocate radio resources for D2R scheduling by the UE reader. |
|  | K value | No. |
| Access Trigger | R2D Message Type | No. |
| Random ID Response (Msg2 in CBRA) | R2D Message Type | No. |
|  | R2D TBS | No. |
|  | D2R Scheduling Info | The gNB needs to allocate radio resources for D2R scheduling by the UE reader. |
|  | Frequency Index | No. |
|  | Echoed Random ID | No. |
|  | Assigned AS ID | No. |
| R2D Upper Layer Data Transfer | R2D Message Type | No. |
|  | R2D TBS | No. |
|  | AS ID | No. |
|  | D2R Scheduling Info | The gNB needs to allocate radio resources for D2R scheduling by the UE reader. |
|  | Data SDU | The R2D AIoT NAS message generated by AIOTF needs to be delivered to the reader UE via the gNB. |
|  | Received Data Size | No. |
| NACK Feedback | R2D Message Type | No. |
|  | R2D TBS | No. |
|  | AS ID | No. |
| Access Random ID (Msg1 in CBRA) | Random ID | No. |
| D2R Upper Layer Data Transfer | D2R Message Type | No. |
|  | More Data Indication | No. |
|  | Data SDU | The D2R AIoT NAS message received by the UE reader needs to be delivered to AIOTF via the gNB. |

In addition to the above analysis, it should be noted that the radio resources required for the UE reader to transmit R2D messages (Paging, Access Trigger, Random ID Response, R2D Upper Layer Data Transfer, NACK Feedback) also need to be allocated by the gNB to the UE reader.

Proposal 1.	To achieve the same service as Topology 1, following information shall be delivered in DL RRC message between the gNB and the UE reader.

- Radio resources for transmission of following R2D messages: Paging, Access Trigger, Random ID Response, R2D Upper Layer Data Transfer, NACK Feedback.

- Radio resources for transmission of D2R messages to indicate the following R2D messages: Paging, Random ID Response, R2D Upper Layer Data Transfer.

- Correlation ID

- Paging ID

- R2D AIoT NAS message

Proposal 2.	To achieve the same service as Topology 1, following information shall be delivered in UL RRC message between the gNB and the UE reader.

- D2R AIoT NAS message

  - Reader ID

In the operation of Topology 2, it is usual to have multiple UE readers attached to a single gNB. In such cases, there can be situations where the coverage areas of neighboring UE readers overlap. That is, there may be environments in which a device can receive paging messages from multiple UE readers.

Even in the discussions on Release 19 Topology 1, it was recognized that overlapping coverage between readers could occur, but no specific solution was defined for this, and the issue was addressed operationally by assigning different Transaction IDs to neighboring readers.

Unlike Topology 1, where the physical location of gNB readers can be easily managed operationally, Topology 2 involves mobile UE readers, making the adjacency between readers dynamically change.

A potential solution is for AIOTF to continuously track the location information of UE readers and control the assignment of Correlation IDs accordingly. However, this would require the integration of positioning functionality into the Topology 2 network, significantly increasing deployment costs. Moreover, it would necessitate defining interactions between AIOTF and LMF on the CN side, resulting in a substantial impact and making the approach impractical.

As a solution that can be introduced without impacting the CN, the introduction of a reader ID can be considered. In other words, different reader IDs would be assigned to UE readers, and when a device receives a new paging message, it would initiate a new procedure only if the Transaction ID or reader ID is different. This provides a simple resolution.

Proposal 3.	Consider the scenario where neighboring UE readers have overlapped coverage.

Proposal 4.	Introduce Reader ID to differentiate UE readers attaching to the same gNB.

- Conclusions

Proposal 1.	To achieve the same service as Topology 1, following information shall be delivered in DL RRC message between the gNB and the UE reader.

- Radio resources for transmission of following R2D messages: Paging, Access Trigger, Random ID Response, R2D Upper Layer Data Transfer, NACK Feedback.

- Radio resources for transmission of D2R messages to indicate the following R2D messages: Paging, Random ID Response, R2D Upper Layer Data Transfer.

- Correlation ID

- Paging ID

- R2D AIoT NAS message

Proposal 2.	To achieve the same service as Topology 1, following information shall be delivered in UL RRC message between the gNB and the UE reader.

- D2R AIoT NAS message

Proposal 3.	Consider the scenario where neighboring UE readers have overlapped coverage.

Proposal 4.	Introduce Reader ID to differentiate UE readers attaching to the same gNB.

- References

[1]	RP-252894, “Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2,” Huawei, RAN#109.