3GPP TSG-RAN WG2 Meeting #131bis	R2-2507624

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.1.2.2

Source:	Nokia

Title:	Issues related to AI for Air interface feature in TS 38.300

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This document addresses one open issue (Issue-5) discussed in R2-2506958.

# 2	Discussion

## 2.1	Procedures for applicability reporting

In accordance of the CR of TS 38.300 (cf. R2-2506995), the network inquiries about UE capabilities via UECapabilityEnquiry and UE responds with AIML supported capabilities via UECapabilityInformation.  This is illustrated below from the CR:

#### X.Y.2.2 Applicability Reporting For UE-side model, the network provides inference configuration based on UE supported functionalities. The UE reports its applicable functionalities, inapplicable functionalities with its preference to release the configuration and subsequent changes of applicability status of functionalities to network. The basic procedure of applicability reporting is described as in Figure X.Y.2.3-1. <omitted> 1. The network inquires about the UE capability information. 2. The UE indicates its supported AI/ML functionalities to the network via UECapabilityInformation message. <omitted>

The current description implies that applicability reporting requires additional UE capability exchange as part of the procedures. This is not true as AI/ML supported UE is always needs to perform the applicability procedures whenever it receives an inference configuration and inference related parameters via RRCReconfiguration. It is unnecessary to inquiry by the network and the indication of supported AI/ML functionalities by UE each time the applicability procedure is required.

Observation 1: It is unnecessary to retrieve the UE capabilities of supported AI/ML functionalities each time the applicability reporting procedure is performed.

Proposal 1: Remove Step 1 and Step 2 from the basic procedure of applicability reporting in Clause 22.2.3 and add a clarification “NOTE that the supported AI/ML functionalities of the UE are retrieved within the UE capabilities before the procedure”. (TP can be found in the ANNEX.)

# 3	Conclusion

This document has made the following observations and proposal:

Proposal 1: Remove Step 1 and Step 2 from the basic procedure of applicability reporting in Clause 22.2.3 and add a clarification “NOTE that the supported AI/ML functionalities of the UE are retrieved within the UE capabilities before the procedure”. (TP can be found in the ANNEX.)

# Annex: TP for 38.300

### 22.2.3	Applicability Reporting

For UE-side model, the network provides inference configuration based on UE supported functionalities. The UE reports its applicable functionalities, inapplicable functionalities with its preference to release the configuration and subsequent changes of applicability status of functionalities to network. The basic procedure of applicability reporting is described as in Figure 22.2.3-1.

Figure 22.2.3-1: Initial applicability and applicability status change reporting

1.	The network may provide inference configuration with NW-side additional conditions (e.g., associated ID) to UE via CSI report configuration or inference related parameters configuration via OtherConfig.

NOTE X:	The supported AI/ML functionalities of the UE are retrieved within the UE capabilities before this procedure.

2.	The UE determines the applicable AI/ML functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in the UE.

NOTE 1:	If the network does not provide the associated ID, it is up to UE implementation how to determine the applicability.

3.	The UE reports its functionality applicability in RRCReconfigurationComplete message.

4.	When the inference configuration consists of periodic CSI report configuration, upon reporting the applicable functionalities, the UE autonomously activates the applicable AI/ML functionalities. When the inference configuration consists with semi-persistent CSI and/or aperiodic CSI report configuration, upon reporting the applicable AI/ML functionalities, applicable AI/ML functionality activation can be activated by MAC CE/DCI and aperiodic CSI reporting can be activated by DCI.

5.	When the network enables applicability reporting via OtherConfig, and applicability of the functionality changes, the UE can report updated functionality applicability status in UEAssistanceInformation message. When an activated AI/ML functionality becomes inapplicable, the UE does not autonomously deactivate it, but the UE indicates to the network the change in the applicability. Upon reception of UE indication of the functionality becoming inapplicable, the network should deactivate or release this activated functionality.

NOTE 2:	Upon receiving one or more inference configuration(s), UE maintains all the inference configuration(s) no matter the inference configuration is applicable or inapplicable until the network releases it.

During handover, the UE may receive the inference configuration related to the target gNB via handover command. Then the UE reports its applicable/inapplicable AI/ML functionalities to the target gNB after the handover completion.

NOTE 3:	UAI can be sent from the source gNB to the target gNB to exchange applicability reporting referring to the configurations from the source gNB.