3GPP TSG-RAN WG2 Meeting #131-bis	R2-2507633

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.13.3

Source:	Samsung

Title:	SRAP error handling and related TP

Document for:	Discussion & Decision

# Introduction

The basics of SRAP design for R19 multi-hop (MH) Sidelink relay have been finalized, with many key issues now settled and implemented in the running CR agreed at RAN#109 (September 2025). We do however believe that a couple of issues (which ended up not being identified as designated open issues) should be further discussed, and we outline them in this submission.

# Error handling: dealing with errors linked to next hop and previous hop

In the downstream direction, there could be multiple child Relays/UEs, and therefore SRAP packet should be discarded by the U2N Relay UE if the next-hop ID configured for the UE identity matching that of the UE ID field of the received SRAP packet does not match any of the L2 IDs of any of the egress links configured for the U2N Relay UE in question. Essentially, this is the case of unknown/wrong next-hop ID which is not present in legacy systems (or in the initial version of the R19 SRAP specification), i.e. when the next-hop ID configured is not one of the child nodes of the Relay UE in question. Current UE ID error handling looks at the UE ID in the SRAP header, and (in the downstream) compares it to the DST UE. Whereas now (in the MH case), in addition to this check, we need to have a check of the next-hop ID. In other words, even if the match between the DST UE and the SRAP header UE ID is ok (this is the legacy check, and the check performed in the initial version of the R19 SRAP spec), there could be a misconfiguration of the next-hop ID.

Rather than assuming that such an error would be prevented by performing RRC reconfiguration, we believe SRAP spec should capture this as an error case.

Additionally, in legacy specs (single-hop U2N Relaying), the ingress link connects directly to source Remote UE on the upstream, and therefore it indeed makes sense (as is done currently) to perform a check at the Relay UE whether the UE ID of the packet matches sl-LocalIdentity corresponding to L2 ID of the ingress link (which is inherited by the MH case as per the initial version of the SRAP R19 spec). For MH however, the source UE could be several hops away and this check is potentially incorrect/needs enhancing, as the UE ID in the SRAP header on the upstream only refers to SRC ID, and not the previous hop UE ID. There now exist the indirect parent/child, and the direct parent/child. The current (legacy) error handling does not make a distinction between the two. We therefore propose:

Proposal. RAN2 to agree the TP in the Appendix of this document, capturing in the SRAP specification the case of unknown/incorrect next-hop ID on the downstream, and the case of the UE ID of the indirect parent being compared to the sl-LocalIdentity corresponding to L2 ID of the ingress link on the upstream.

(The former assumes the configuration of a next-hop ID for each of the sl-LocalIdentity included in sl-SRAP-ConfigRelay or sl-SRAP-ConfigRelay-ToAddModList, while the latter assumes a parameter referred to as sl-SRAP-ConfigRelayChild [or any other suitable name] is configured comprising configurations for each of the Child UEs of the Relay UE.)

# Appendix: TP to 38.351

## 5.4	Handling of unknown, unforeseen, and erroneous protocol data

For U2N Remote UE (includes Intermediate U2N Relay UE acting as U2N Remote UE), if sl-LocalIdentity and sl-RemoteUE-RB-Identity are both configured, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRemote is received, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

For U2N Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRelay or sl-SRAP-ConfigRelay-ToAddModList is received except in the case where the SRAP Data PDU from SL-RLC1 as specified in TS 38.331 [3] is the first SRAP Data PDU received from a U2N Remote UE, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity included in sl-SRAP-ConfigRelay or sl-SRAP-ConfigRelay-ToAddModList associated with sl-L2IdentityRemote of the ingress link or any of the sl-LocalIdentity included in sl-SRAP-ConfigRelayChild is received by U2N Relay UE, or when the next hop ID configured for the sl-LocalIdentity included in sl-SRAP-ConfigRelay or sl-SRAP-ConfigRelay-ToAddModList corresponding to the UE ID field of the received SRAP packet does not match any of the L2 IDs of any of the egress links configured for the U2N Relay UE the SRAP entity shall:

-	Discard the received SRAP Data PDU.

For U2U Remote UE, if sl-RemoteUE-LocalIdentity and sl-PeerRemoteUE-LocalIdentity are both configured, when an SRAP Data PDU with SRAP header that contains UE ID fields which does not match sl-PeerRemoteUE-LocalIdentity and sl-RemoteUE-LocalIdentity included in any one of the entries in sl-LocalID-PairList is received or when an SRAP Data PDU with SRAP header that contains BEARER ID field which does not match 0/1/2/3 or any of the 5 bits LSBs of slrb-PC5-ConfigIndex used on the end-to-end PC5 connection indicated by sl-RemoteUE-LocalIdentity and sl-PeerRemoteUE-LocalIdentity is received, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

For U2U Relay UE, when an SRAP Data PDU with SRAP header that contains a UE ID (for DST) field which does not match sl-PeerRemoteUE-LocalIdentity included in any one of the entries in sl-LocalID-PairList is received, or when an SRAP Data PDU that contains a UE ID (for SRC) field which does not match the concerned sl-RemoteUE-LocalIdentity corresponding to sl-RemoteUE-L2Identity of the ingress link in any one of the entries in sl-LocalID-PairList with matched sl-PeerRemoteUE-LocalIdentity and sl-PeerRemoteUE-L2Identity is received, or when an SRAP Data PDU with SRAP header that contains BEARER ID field which does not match 0/1/2/3 or any of the 5 bits LSBs of slrb-PC5-ConfigIndex used on the end-to-end PC5 connection indicated by sl-RemoteUE-LocalIdentity and sl-PeerRemoteUE-LocalIdentity is received by U2U Relay UE, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

When any of the U2N Remote UE, the U2N Relay UE, the U2U Remote UE or the U2U Relay UE receives an SRAP Data PDU with invalid or reserved values, the SRAP entity shall:

-	Discard the received SRAP Data PDU.