3GPP TSG-RAN WG2 Meeting #131-bis	R2-2507646

Prague, CZ, Oct 13-17, 2025

Agenda Item:	10.3.2

Source:	THALES, Airbus, Echostar, Novamint, Fraunhofer IIS

Title:	6G Control Plane design aspects for NTN

Document for:	Discussion

# 1	Introduction

The 6G Radio Study Item was approved at the RAN#108 meeting and updated at RAN#109 [1]. The main objectives related to the radio protocol architecture are:

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] a) Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE ca i) Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. […] j) Aim at a harmonized 6G Radio design for TN and NTN, including their integration. […] (3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] […] b) Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] c) Access stratum security aspects, in alignment with requirements from SA3 [RAN2] […]

While in the context of 5G, the support of Non-Terrestrial Networks (NTN) was added in Rel-17 to the NR access protocols, 6G offers the opportunity to design harmonized protocols and procedures between TN and NTN to consider the characteristics of the two access systems. Especially, NTN systems are characterized by a long delay on the user link, depending on the orbit (see [5]), large cell coverage, and constrained satellite capacities (energy, memory, computation).

In this contribution, we share our views on some control plane desirable characteristics for NTN deployments.

# 2	Discussion

## 2.1	Initial Access

The initial access is a critical operation for an efficient satellite communication system. One challenge for NTN is to increase the density of terminals willing to transmit quasi-simultaneously on a limited bandwidth. Another challenge is the access time for the UE since the satellite visibility in the sky is limited to few minutes in low Earth orbit (LEO), there is a risk of an uncompleted procedure if the access time is too important. These two challenges can be solved with a grant-free access protocol that limits the number of RTTs for the initial access [7]. It is also beneficial in terrestrial networks for latency-critical applications.

Proposal 1	The 6G radio access shall be able to support grant-free initial access protocol for NTN. This may also be beneficial for TN under some use cases.

## 2.2	Optimized and flexible control signalling

5G NR introduced an ultra-lean design for control signals compared to LTE, i.e. minimize the “always-on” transmissions. Ultra-lean design significantly improved network energy efficiency in terrestrial networks. Legacy satellite systems usually implement beam-hoping techniques to increase coverage of a given satellite as illustrated below.

Figure 1 - Beam Hoping in satellite networks

In 5G-Advanced Rel-19, downlink coverage enhancements have been introduced to support beam-hoping permitted by the extending SSB periodicity to 160 ms and the lean control signal design [6]. In 6G, the effort shall be pursued to allow flexible transmission in time with a reduced overhead of control signaling for the satellite system efficiency.

Proposal 2	To study for the 6G radio interface, optimizations of control signaling adapted to network energy saving and NTN constraints (e.g. beam hopping techniques, reduced overhead, etc.)

## 2.3	RRC energy efficient state

From the terminal standpoint, the connectivity to the satellite network can be energy consuming, which is inconsistent with the objective of energy efficiency for the 6G radio access network. Moving the terminal to an idle state is not desirable because of the latency and cost associated with reconnecting to the satellite when  a new transmission is available. So an “inactive” state should be supported for NTN, that was not the case in 5G specifications for regenerative payload,partly due to  security context inter-satellite propagation issue. 6G may offer the opportunity to design such energy efficient RRC with appropriate control plane procedures to support this state across different NTN architectures, i.e. transparent, regenerative and split architectures.

Proposal 3	The 6G radio access shall be designed to support energy-efficient RRC state (e.g. inactive mode) for terminals not in transmission for both TN and NTN

# 3	Conclusion

Proposal 1	The 6G radio access shall be able to support grant-free initial access protocol for NTN. This may also be beneficial for TN under some use cases.

Proposal 2	To study for the 6G radio interface, optimizations of control signaling adapted to network energy saving and NTN constraints (e.g. beam hopping techniques, reduced overhead, etc.)

Proposal 3	The 6G radio access shall be designed to support energy-efficient RRC state (e.g. inactive mode) for terminals not in transmission for both TN and NTN

# References

RP-252912, “Revised SID: Study on 6G Radio”, NTT Docomo, CMCC, AT&T, Vodafone, Beijing, CN, September 2025.

RP-250810, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT Docomo, Deutsche Telekom, Incheon, KR, March 2025.

3GPP TR 38.914, “	Study on 6G Scenarios and requirements”, v0.1.1, Beijing, CN, September 2025.

6GWS-250040, “Vision and priorities for 6G RAT to support NTN”, Thales, Incheon, KR, March 2025.

R2-2507644, “6G Radio Access Technology general aspects for NTN”, THALES, Airbus, Echostar, Novamint, Fraunhofer IIS, Prague, CZ, October 2025.

R1-2401937, Report of RAN1#116 meeting, RAN1#116bis, April 15-19 2024, Changsha, CN

De Gaudenzi, Riccardo et al. “Random Access Schemes for Satellite Networks: from VSAT to M2M” (2016) -A Survey. International Journal of Satellite Communications and Networking. 36. 10.1002/sat.1204.