# 3GPP TSG-RAN WG2 #131bis	R2-2507651

# Prague, Czech Republic, 13-17 Oct 2025

Source:	Rakuten Mobile Inc

Title:	Discussion on Requirements for UE as reader

Document for:	Discussion and Decision

Agenda Item:	9.2.2 – Topology 2

- Introduction

Rel-19 established the baseline for Ambient IoT procedures. For Topology-2, it clarified that the interface between the network and the intermediate UE (Reader) falls within the RAN2 scope, while device-side air-interface specifics remain outside. For Rel-20, RAN2 is requested to complete Uu procedures so that a UE can be enabled as a Reader in DS-2, reliably relay DT paging/trigger, forward DO-DTT indications, manage access occasions and re-access, and handle short AS-level identifiers, all under RRC-based control and serving-cell validity.

- Discussion

# 2.1 UE-Reader enablement on Uu, lifecycle (serving-cell scope), mobility and coexistence with Uu activity

A UE acting as Reader needs a compact enable/disable control on Uu, a clear validity model limited to the serving cell, deterministic behavior when planned device interactions overlap imminent Uu activity, and predictable outcomes at mobility and radio failure. Without common rules, implementations diverge on whether a staged device-facing action proceeds, is briefly delayed, or is cancelled; whether session state persists over handover; and what the UE reports when configured actions cannot be executed. Deterministic pre-emption handling, a lifecycle model covering RLF/HO, and light event visibility enable stable network decisions for DT/DO-DTT and re-access.

Observation 1: Overlap with imminent Uu UL/DL causes pre-emption oscillations without a bounded delay option and a defined cancellation point.

Observation 2: During HO, staged actions cannot be executed by source if continuation/termination is not explicit.

Proposal 1: RAN2 to Specify a minimal Uu configuration to enable/disable the Reader role, indicate serving-cell validity and optional time validity, and permitted A-IoT usage.

Proposal 2: RAN2 to define pre-emption rules for overlap with Uu (when to cancel, when a single short delay is allowed, and when to abandon), with deterministic outcomes.

Proposal 3: RAN2 to define lifecycle at RLF/HO: suspend on failure; release at HO completion unless re-enabled in target; and allow a short event indication.

# 2.2 DT in Topology-2: Uu mapping, execution window, precedence and duplicate suppression

For DT, the gNB provides compact context on Uu and the UE-Reader composes the device-addressable transmission. The issues are late delivery around HO/cancellation, overlapping contexts, and duplicates across serving-cell change. A small set of timing and ordering rules fixes this without a new layer.

Observation 3: Late delivery is likely if execution windows are implicit or loosely bounded, leading to paging after cancellation or in the wrong cell.

Observation 4: Without a stable transmission token, retries across serving-cell changes can produce duplicate paging.

Proposal 4: RAN2 to specify a DT Uu mapping that includes an execution window (start/stop or offset/duration) and a shall-not-deliver rule after expiry and at HO unless the target re-issues the context.

Proposal 5: RAN2 to include a de-duplication token valid for the intent lifetime across serving-cell change and allow a short outcome code (delivered / expired / pre-empted / mobility-terminated) reported on error or when polled.

# 2.3 DO-DTT in Topology-2: forward indication, correlation/ordering and device-facing feedback

For DO-DTT, the UE forwards concise indications of device-originated events and renders the network decision back to the device, preserving the success semantics agreed in Rel-19 (explicit failure only when no further downlink will follow; otherwise success may be implicit via subsequent downlink). Dense scenarios introduce multi-candidate and reordering risks; mobility and UL congestion can lead the network to act before the forward indication is processed. A compact correlation/ordering concept on the forward path, bounded buffering with deterministic discard, and a small loss-reason set allow the network to remain in control without heavy signaling.

Observation 5: Mis-association occurs if multiple device indications arrive closely without correlation and ordering markers.

Observation 6: Downlink may be scheduled before the related forward indication; feedback semantics must remain consistent.

Proposal 6: RAN2 to study how to avoid mis-associations in multiple device scenarios.

Proposal 7: RAN 2 to specify feedback methodology via the UE-Reader that ensures explicit/implicit success.

Proposal 8: RAN2 discuss defining buffer handling under UL load with a size limit and a clear discard rule (e.g., oldest-first), and to allow a gNB-configured reporting rate per Reader-UE.

# 2.4 Device-originated access-occasion control and re-access (DO-DTT)

Rel-19 allows initial access occasions, in-round updates, and re-access during or between rounds. In Topology-2, late updates can overlap with already prepared occasions; without a common rule, some implementations may double-schedule while others over-cancel. Subset-based re-access improves contention but can starve the same subset.

Observation 7: Double scheduling or double cancellation occurs when late updates meet staged occasions without a common rule.

Observation 8: if subset selection repeatedly favours the same ordering under persistent load some device may be starved of resources.

Proposal 9: RAN 2 to discuss ways to avoid duplicated scheduling and resource starvation.

Proposal 10: RAN2 require to implement fairness for re-access (rotation or cap on deferrals)

# 2.5 AS-level identifier policy for device-originated sessions (CBRA/CFRA, DO/DO-DTT)

Short AS-level IDs keep signalling small, but in Topology-2 they must work reliably with late collision checks and mobility.
For CBRA, reuse the device’s value after contention resolution; if a later ambiguity is detected, switch quickly to a new ID so the device does not act twice.
For CFRA, assign the ID in the first downlink and re-assert it promptly after a serving-cell change to avoid stale responses.
Make the uniqueness per Reader-UE and per session, and treat the ID as session-scoped with implicit expiry. If the state is lost, use a short re-establish step instead of a heavy re-inventory.

Observation 9: Late collision can transiently map multiple devices to one value unless a rapid fallback path is specified.

Observation 10: After mobility, delayed re-assertion can lead to stale device responses to an old value.

Proposal 11: RAN2 to investigate both CBRA and CFRA mapping and related AS-Level IDs to map multiple devices.

- Conclusions

Based on the discussion above, we conclude this contribution with the following proposals

Proposal 1: RAN2 to Specify a minimal Uu configuration to enable/disable the Reader role, indicate serving-cell validity and optional time validity, and permitted A-IoT usage.

Proposal 2: RAN2 to define pre-emption rules for overlap with Uu (when to cancel, when a single short delay is allowed, and when to abandon), with deterministic outcomes.

Proposal 3: RAN2 to define lifecycle at RLF/HO: suspend on failure; release at HO completion unless reenabled in target; and allow a short event indication.

DT-

Proposal 4: RAN2 to specify a DT Uu mapping that includes an execution window (start/stop or offset/duration) and a shall-not-deliver rule after expiry and at HO unless the target re-issues the context.

Proposal 5: RAN2 to include a de-duplication token valid for the intent lifetime across serving-cell change, and allow a short outcome code (delivered / expired / pre-empted / mobility-terminated) reported on error orwhen polled.

DO-DT-

Proposal 6: RAN2 to specify a forward indication on Uu carrying a per-event correlation token and, when needed, an ordering index, to avoid mis-association in multiple-device cases.

Proposal 7: RAN2 to specify the feedback method via the UE-Reader: explicit failure only when no subsequent downlink will follow; otherwise success may be implicit via the subsequent downlink.

Proposal 8: RAN2 to define buffer handling under UL load with a UE-side queue size limit, a clear discard rule (oldest-first), and a gNB-configured reporting rate per Reader-UE.

Proposal 9: RAN2 to define precedence and cancellation rules to avoid duplicated scheduling and to prevent resource starvation when late updates overlap staged access occasions.

Proposal 10: RAN2 to require fairness for re-access assignment (e.g., rotation or a cap on deferrals) to avoid repeated postponement of the same devices.

Proposal 11: RAN2 to specify AS-level identifier policy for CBRA and CFRA: CBRA reuse with fast fallback on ambiguity; CFRA early assignment with prompt re-assertion after mobility; AS-ID unique per Reader-UE and per session with implicit expiry.

- References