3GPP TSG RAN WG2 Meeting #131bis         	                          R2-2507654

Prague, CZ, Oct 13th – 17th, 2025

Agenda Item:	8.1.2.2

Source:	Xiaomi

Title:	Discussion on open issues of AIML LCM [E041, H003/A105/S047, E042, Z001/Z002, B206]

Document for:	Discussion and Decision

# Introduction

Based on the ASN.1 review, in this contribution, we further discuss the remaining open issues of AI/ML air LCM (e.g., applicability reporting, etc), including E041, N021/H003/A105/S047, E042, Z001/Z002, Z005/H009, H010, Z007, B206/O300.

# Discussion

## E041

RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. E041; AIML; 2; Associated ID validity; R2-25xxxx; Ericsson (Jens); V022; ToDo. [Description]: According to current specifications, once an associated ID value is signaled from the network to the UE, the meaning of that value at the UE (or UE-side entity) corresponds to the same beam property/deployment forever. There is no way for the network to reset/reassign that value to other beam properties and inform the UE that should forget the old meaning of that value. We think it is unreasonable to assume that a given meaning holds for a given associated ID forever. It may happen that e.g. some hardware is replaced and the old hardware is not used again anywhere else in the network, or there is a failure/reset in the network entity storing/assigning associated IDs, after which the legacy associated ID meanings (assigned to deployment characteristics) cannot be recovered. Although such events may be very unlikely, they may still happen at a certain point in the future. Thus, there should be a way for the network and UE to make sure that they have the same understanding of an associated ID value. This is important for the UE (or UE-side entity) to be able to understand if a model exists and/or is trained for the same deployment assumed by the network for an associated ID.

First of all, the alignment between network and UE on the meaning of associated ID is the key factor to the succeed of UE-side beam management. It is true that, we cannot preclude/ignore the case where network vendors update its hardware and a different beam property applies. However, during this change, we believe the alignment between UE and network can still be maintained in the following approaches:

Approach 1: Network updates the meaning of associated ID, by indicating validity duration of the associated ID

Ideally, it would be good for UE to know the validity duration of an associated ID at the network side. Though this approach provides a dynamic way to allow network to change the associated ID to different beam deployments, due to the unpredicted change of network hardware, it is not clear how long of the validity duration and how often network needs to update this information to the UE.

Approach 2: Network allocates different associated ID to new network hardware deployments, i.e., every time network beam property is changed, a new associated ID is allocated.

According to the current specification, 24bits is allocated, where 16, 777, 216 number of associated IDs can be used within a PLMN across multiple vendors. In our understanding, this is sufficient for different network deployment and updates.

AssociatedId-r19 ::= BIT STRING (SIZE (24))

Whenever network updates its hardware, network can assign a new associated ID to the new deployment and configure UE to re-start training data collection based on the new associated ID. In our understanding, there’s no need to release/reconfigure the meaning of the existing associated ID, as other network deployment may share the same associated ID and network may not upgrade all deployments at the same time. It is still possible that both legacy and new hardware deployment co-exist in the real deployment. Therefore, a new associated ID to the new deployment characteristic is preferred, to avoid UE mixing-up the meaning of the same associated ID to different network deployments. Additionally, since a new associated ID will be assigned, there’s no need for UE to know whether there’s a change at the network side.

Observation 1: It is possible that both old and new hardware deployment coexist in the real deployment. The meaning of associated ID to the old hardware deployment cannot be released/replaced.

Since a new associated ID is assigned, one may concern that the UE needs to retrain a UE-side model, which further increases UE complexity. From UE point of view, we don’t think it’s a big problem, as UE-side models with different associated IDs are trained and stored at the UE-side training entity located at application layer.

Compared between Approach #1 and Approach #2, since the WI is closed, we prefer to consider a solution without specification impact in this release.

Proposal 1: [E041] is PropReject. Network allocates a new associated ID when beam property has changed at the network side.

## N021/H003/A105/S047

The configurations for inference, monitoring and UE-side data collection for AI/ML beam management and AI/ML CSI prediction have been discussed in N021/H003/A105/S047. The key controversial issue is whether and how to separate different configurations for different use cases and purposes.

We understand the motivation of combining the same parameters within the same IE and only use reportQuality-r19 to differentiate which parameter needs to be used. However, by doing so, it loses the readability and increases UE implementation complexity (i.e., UE has to open and read each and every parameter for the right configuration), especially by putting UE-side data collection parameters inside a parameter name with ‘inference’. Therefore, we prefer to clearly separate the configurations for different use cases and purposes into different parameters. That is:

- configurationForChannelMonitoringBM-r19 for AI/ML BM;

- configurationForChannelMonitoringCSI-r19 for CSI prediction;

- configurationForUE-DataCollectionBM-r19 for AI/ML BM UE-side data collection;

- configurationForUE-DataCollectionCSI-r19 for CSI prediction UE-side data collection (only including reference to CSI-ReportConfigId).

Proposal 2: [N021/H003/A105/S047] Use separate parameters for different use cases and purposes, especially ‘between AI/ML BM monitoring and CSI prediction monitoring’, and ‘between UE-side data collection and inference prediction configuration’.

## E042

RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. E042; AIML; 2; Necessity of dedicated flag to configure applicability reporting via UAI; R2-25xxxx; Ericsson (Andra); V022; ToDo. [Description]: RAN2#130 agreed the following: “2 Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled. Assume this applies to Option A and B, FFS if anything different needs to be done for option B (if specified)” After adding the sets of inference related parameters for option B in otherConfig, it was not discussed how to combine this flag in ASN.1 with the configurations for option B. [Proposed Change]: We will provide a more detailed discussion and solutions in a Tdoc. Some possible solutions are: - Remove the separate flag reportApplicabilityUAI-r19 and assume that applicapility reporting via UAI is implicitly configured when applicabilityReportConfig is included in otherConfig. - Keep the flag reportApplicabilityUAI-r19 within applicabilityReportConfig, as it is. - Move the flag reportApplicabilityUAI-r19 one level up, directly under otherConfig.

In our understanding, when RAN2 made agreement for having a dedicated flag in OtherConfig, the key motivation is to indicate that UE is allowed to report applicability reporting via UAI. Since now we introduced applicabilityConfigList-r19, where network can provide inference related configurations, if the list is presented, it can be used to implicitly indicate that UE is allowed to report applicability via UAI. A separate bit is not needed.

Proposal 3: [E042] is PropAgree. If applicabilityConfigList-r19 is presented, UE is allowed to report applicability via UAI.

## Z001/Z002

RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. Z001; AIML; 1; Preference suggestion to the functionality that is reported as inapplicable in RRCReconfiguration/UAI; R2-25xxxx; Fei; V015; ToDo. [Description]: Currently, for the applicability reporting via RRCReconfigurationComplete, it has been defined as below: 4> for each entry within applicabilitySetConfigList associated with the concerned serving cell, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation): 5> include an entry in the applicabilityInfoReportList and set the content as follows: 6> set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId; 6> set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId; 6> if the applicabilityStatus is set to inapplicable: 7> if the UE prefers to release the concerned ApplicabilitySetConfig, include releaseConfigurationPreference; For the functionality that is provided in the OtherConfig whose applicability has been changed, there is no need for UE to provide the information of the releaseConfigurationPrefernece in the RRCReconfigurationComplete/UAI as there is no hurt to keep it still in the otherConfig.

First of all, the nature of inference related configuration in OtherConfig and inference configuration in RRCReconfiguration has no fundamental difference, as both configurations are used for UE to check applicability of UE-side model. If the UE itself knows certain UE-side model(s) will not be applicable in the future, it is applicable to both inference configuration and inference related configuration.

Last but not least, we think a unified UE behaviour for handling Option A and Option B is necessary to reduce UE complexity. That is, for both configurations received from Option A and Option B, UE can indicate to release the concerned applicability configuration.

Proposal 4: [Z001/Z002] is PropReject.

## B206/O300

RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. B206; AIML; 1; Transfer of applicability information in handover command preparatoin; Congchi Zhang; V011; ToDo. [Description]: Relevant to the RAN2 discussion and the note below in 38.300 - NOTE 3: UAI can be sent from the source gNB to the target gNB to exchange applicability reporting referring to the configurations from the source gNB. The current UAI in handover preparation information message only conveys what has been reported by UE in the last UAI report. In the context of applicability reporting procedure, that means the target gNB may only be informed about the recently changed applicability information (i.e., delta instead of complete appliability information). It can be easily resolved by clarifying that the UAI in handover preparation information can also contain the complete applicability information not only the last reported ones.

First of all, in our understanding, handing over the full list of UE’s applicability report is only applicable for the case where source gNB and the target gNB share the exact same network deployment and preference of enabling UE’s UE-side model. During the past discussion, considering network vendors have expressed the view to even consider a cell-specific associated ID, we don’t think it is a common case that the same applicability report at the source gNB can still be applicable after UE’s handover, especially when associated ID is cell-specific.

Observation 2: Considering some network vendors want cell-specific associated ID, it is very likely that UE’s full applicability report at source gNB will not be applicable at target gNB, due to the change of associated ID (and its representative meaning of beam property).

Therefore, after UE’s handover, the target gNB can always request UE to report its full applicability information that is applicable to its own, either via Option A or Option B.

Proposal 5: [B206] is PropReject. The target gNB can always request UE to report its full applicability information that is applicable to the target gNB after UE’s handover.

# Conclusion

Observation 1: It is possible that both old and new hardware deployment coexist in the real deployment. The meaning of associated ID to the old hardware deployment cannot be released/replaced.

Proposal 1: [E041] is PropReject. Network allocates a new associated ID when beam property has changed at the network side.

Proposal 2: [N021/H003/A105/S047] Use separate parameters for different use cases and purposes, especially ‘between AI/ML BM monitoring and CSI prediction monitoring’, and ‘between UE-side data collection and inference prediction configuration’.

Proposal 3: [E042] is PropAgree. If applicabilityConfigList-r19 is presented, UE is allowed to report applicability via UAI.

Proposal 4: [Z001/Z002] is PropReject.

Observation 2: Considering some network vendors want cell-specific associated ID, it is very likely that UE’s full applicability report at source gNB will not be applicable at target gNB, due to the change of associated ID (and its representative meaning of beam property).

Proposal 5: [B206] is PropReject. The target gNB can always request UE to report its full applicability information that is applicable to the target gNB after UE’s handover.