3GPP TSG-RAN3 Meeting #130																R3-258108

Dallas, USA, 17-21 November, 2025

Agenda Item:	12.2.1

Source:	ZTE Corporation

Title:	(TP to TR 38.745) Consideration on Multiple-hop UE trajectory

Document for:	pCR

- Introduction

In Rel-18, cell-based UE trajectory prediction was limited to the first-hop target NG-RAN node. Multi-hop UE trajectory prediction across gNBs was identified as one of the objectives of AI/ML for NG-RAN enhancements phase 3. At the early stage of the Rel-19 phase, the topic of multi-hop UE trajectory prediction across gNBs was discussed, and several solutions were proposed.

Last meeting, several understandings were discussed and following agreements are reached:

The following solutions are considered for supporting multi-hop UE trajectory:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

Focus on cell-based granularity for measured and predicted multi-hop UE trajectory.

Study multi-hop UE trajectory for L3 HO.

The delivery of the measured&predicted cell-based multi-hop UE trajectory should not impact the core network.

The request and reporting to support measured multi-hop UE trajectory between gNBs should utilize the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure, at least for the first hop.

In this contribution, we further provide our understanding and proposed solutions for multi-hop UE trajectory prediction across gNBs, along with the corresponding TP to TR 38.745.

- Discussion

2.1 Principles

The definition of multi-hop UE trajectory is agreed as below:

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order.

And the impact of multi-hop UE trajectory can be divided into two categories: predicted UE trajectory and measured UE trajectory. Before we discussed the detailed solutions to the support of multi-hop UE trajectory, the overall scenarios should be clarified, which is helpful to further discussion. Thus, following clarification was discussed in the last meeting during offline discussion.:

WA: The cell-based multiple-hop UE trajectory prediction is transferred to the target NG-RAN node via the Handover Preparation procedure.

Common understanding:

- The cell-based multiple-hop UE trajectory can be updated to remove the past visited cell(s).

- When UE moves to unpredicted target cell, the serving gNB can discard the cell-based multiple-hop UE trajectory prediction

First, it should be emphasized that multi-hop UE trajectory prediction may be used by the target node for subsequent mobility optimization. That is, multi-hop UE trajectory serves as a type of assistance information, and whether to use it depends entirely on the target node.

- It is up to the target node to decide whether to leverage the received multi-hop UE trajectory for subsequent mobility optimization.

In order to have a full picture of predicted UE trajectory, we provide the example of flow for the support of multi-hop UE trajectory prediction. Following is the flow for the support of multi-hop UE trajectory prediction, assuming the predicted UE trajectory is correct:

Figure 1. Example of flow for the support of multi-hop UE trajectory prediction

The source node (gNB-1) generates a multi-hop UE trajectory prediction, which consists of a list of cells belonging to one or more gNBs in chronological order, and provides it to the target node (gNB-2). Then, the UE may visit cells in the target gNB (gNB-2), and the target gNB (gNB-2) can evaluate whether the received predicted UE trajectory is correct. Only if the receiving gNB has evaluated that the received predicted UE trajectory is correct will it continue to use the predicted UE trajectory and forward it to the next base station. If the evaluation is accurate, when the next inter-gNB handover needs to occur, the source node (gNB-2, which has now become the source node) can forward the remaining predicted UE trajectory to the target node (gNB-3) via the Handover Request message for subsequent mobility optimization. Each forwarded multi-hop UE trajectory is updated to remove the past visited cell(s).

- Following principle for multi-hop UE trajectory prediction should be acknowledged first:

      - Only if the receiving gNB has evaluated that the received predicted UE trajectory is correct will it continue to use the predicted UE trajectory and forward it to the next gNB.

      - When UE moves to unpredicted target cell, the serving gNB can discard the mismatching cell-based multiple-hop UE trajectory prediction.

      - Each forwarded multi-hop UE trajectory is updated to remove the past visited cell(s).

2.2 Predicted multi-hop UE trajectory

Based on these principles, the specific solution for multi-hop UE trajectory could be clearly studied. With respect to the UE trajectory prediction, it was discussed during the last meeting.

Transfer the multi-hop UE trajectory prediction in UE associated or non-UE associated message?

There are two options proposed:

- Option 1: Forward predicted multi-hop UE trajectory over Handover Request Message

- Option 2: Forward predicted multi-hop UE trajectory over Data Collection Reporting Initiation Message

For single-hop predicted information, it is currently delivered to the target NG-RAN node via the Handover Request message. The predicted UE trajectory is a per-UE measurement, and Option 2 Data collection initiation procedure is a non-UE associated message, making Option 2 completely infeasible. Additionally, by adhering to the current mechanism and according to the current principles, Option 1 can be fully utilized without any specification impacts. The predicted multi-hop UE trajectory could also be delivered to the target gNB via the Handover Request message. When the target node (e.g., gNB2) receives the predicted UE trajectory from source node (e.g., gNB1), it can, according to this trajectory information, forward the remaining predicted UE trajectory to its own target node (e.g., gNB3) during the next handover.

- The delivery of predicted multi-hop UE trajectory can continue to be carried in the Handover Request message to the next target gNB.

2.3 Measured multi-hop UE trajectory

For single-hop measured UE trajectory, the measured information is requested and reported through the Data Collection Initiation procedure and Data Collection Update procedure. For the measured multi-hop UE trajectory across gNBs, the collected measurement results should be forwarded to the initial gNB. This allows the initial gNB to evaluate and analyze the end-to-end performance of the trajectory prediction and mobility optimization.

Following is the agreement for the request and report for multi-hop UE trajectory between gNBs:

The request and reporting to support measured multi-hop UE trajectory between gNBs should utilize the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure, at least for the first hop.

Regading this agreement, one scheme mentions that, for multi-hop UE trajectory collection, except for the first hop, the collection configuration is carried by the Handover Request message. This was discussed in the Rel-18 phase regarding inter-base station configuration information and whether to use non-UE associated messages or UE associated messages. The conclusion was to use non-UE associated messages for configuration. Therefore, this scheme violates the previous principle. Additionally, the configured base station needs to reply whether it can perform the measurement and, if not, the reason; however, using the Handover Request seems unable to meet this requirement.

- Using the Handover Request message to carry collection configuration violates the previous R18 principle.

- Revise the previous agreement to “The request and reporting to support measured multi-hop UE trajectory between gNBs should utilize the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure.”

From our perspective, the same mechanism can be applied to collect measurements. The initial gNB initiates the Data Collection procedure and configures the duration required for the measurements. Subsequent gNBs, based on the actual UE residence time, also initiate the data collection procedure towards their neighbor gNBs and adjust the configured measurement duration as needed. Upon receiving the Data Collection Update message, the gNB integrates the locally measured UE trajectory with the reported measured UE trajectory and forwards the combined information to the previous requesting gNB.

Following is the example of signaling procedures for multiple hop UE trajectory:

Figure. Signalling procedure for multiple-hop UE trajectory across gNBs

Step 1&2. The initial gNB (gNB1) initiates the Data Collection procedure and configures a 30-second measurement duration.

Step 3. gNB1 sends a HANDOVER REQUEST carrying the predicted UE trajectory.

Step 4. UE handover to next gNB (gNB2), and stay for 20 seconds.

Step 5&6. After the UE hands over to gNB2 and remains for 20 seconds, gNB2—based on the UE’s actual residence time—initiates its own Data Collection procedure toward the neighbor cell, configuring a 10-second duration for gNB3.

Step 7. gNB2 sends a HANDOVER REQUEST with the remaining predicted trajectory to next gNB.

Step 8. The UE then hands over to gNB3 and stays for 10 seconds, during which gNB3 performs the configured measurement.

Step 9&10. gNB3 reports the measured trajectory for 20–30 seconds to gNB2; Upon receiving a Data Collection Update message, the gNB integrates its locally measured UE trajectory with the reported results and forwards the combined information to the previous gNB.

Step 11. gNB2 integrates and relays the aggregated 20–30-second results to gNB1; and gNB1 finally consolidates the end-to-end measured multi-hop UE trajectory over the configured window.

- The collection of multi-hop UE trajectory measurements can be supported by the existing mechanisms with minimal standard impacts. The intermediate measurement gNB integrates its locally measured UE trajectory with the reported results received from the next gNB, and then forwards the combined information to the previous gNB.

- Conclusion

Following is the observations and proposals:

- It is up to the target node to decide whether to leverage the received multi-hop UE trajectory for subsequent mobility optimization.

- Following principle for multi-hop UE trajectory prediction should be acknowledged first:

      - Only if the receiving gNB has evaluated that the received predicted UE trajectory is correct will it continue to use the predicted UE trajectory and forward it to the next gNB.

      - When UE moves to unpredicted target cell, the serving gNB can discard the mismatching cell-based multiple-hop UE trajectory prediction.

      - Each forwarded multi-hop UE trajectory is updated to remove the past visited cell(s).

- The delivery of predicted multi-hop UE trajectory can continue to be carried in the Handover Request message to the next target gNB.

- Using the Handover Request message to carry collection configuration violates the previous R18 principle.

- Revise the previous agreement to “The request and reporting to support measured multi-hop UE trajectory between gNBs should utilize the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure.”

- The collection of multi-hop UE trajectory measurements can be supported by the existing mechanisms with minimal standard impacts. The intermediate measurement gNB integrates its locally measured UE trajectory with the reported results received from the next gNB, and then forwards the combined information to the previous gNB.

- Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 4.1	Multiple-hop UE trajectory

### 4.1.1	Use case description

Editor’s Note: Capture the description of use case

In Rel-18, the cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order.

Only if the receiving gNB has evaluated that the received predicted UE trajectory is correct will it continue to use the predicted UE trajectory and forward it to the next gNB. When UE moves to unpredicted target cell, the serving gNB can discard the mismatching cell-based multiple-hop UE trajectory prediction. Each forwarded multi-hop UE trajectory is updated to remove the past visited cell(s).

For the measured multi-hop UE trajectory across gNBs, the collected measurement results should be forwarded to the initial gNB. This allows the initial gNB to evaluate and analyze the end-to-end performance of the trajectory prediction and mobility optimization.

### 4.1.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

The delivery of predicted multi-hop UE trajectory can continue to be carried in the Handover Request message to the next target gNB. If the remaining predicted UE trajectory information is useful, the next gNB can rely this assistance information, and forwards to the subsequent gNB. The process continues in a similar manner.

The gNBs in this process can request the measured UE trajectory that span across multiple gNBs. The intermediate measurement gNB integrates its locally measured UE trajectory with the reported results received from the next gNB, and then forwards the combined information to the previous gNB.

Following is the signaling example to support multiple hop UE trajectory across gNBs.

Figure. Signalling procedure for multiple-hop UE trajectory across gNBs

Step 1&2. The initial gNB (gNB1) initiates the Data Collection procedure and configures a 30-second measurement duration.

Step 3. gNB1 sends a HANDOVER REQUEST carrying the predicted UE trajectory.

Step 4. UE handover to next gNB (gNB2), and stay for 20 seconds.

Step 5&6. After the UE hands over to gNB2 and remains for 20 seconds, gNB2—based on the UE’s actual residence time—initiates its own Data Collection procedure toward the neighbor cell, configuring a 10-second duration for gNB3.

Step 7. gNB2 sends a HANDOVER REQUEST with the remaining predicted trajectory to next gNB.

Step 8. The UE then hands over to gNB3 and stays for 10 seconds, during which gNB3 performs the configured measurement.

Step 9&10. gNB3 reports the measured trajectory for 20–30 seconds to gNB2; Upon receiving a Data Collection Update message, the gNB integrates its locally measured UE trajectory with the reported results and forwards the combined information to the previous gNB.

Step 11. gNB2 integrates and relays the aggregated 20–30-second results to gNB1; and gNB1 finally consolidates the end-to-end measured multi-hop UE trajectory over the configured window.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>