3GPP TSG-RAN3 Meeting #130	R3-258115

Dallas, US, 17 - 21 November, 2025

Agenda item:	13.3

Source:	Xiaomi

Title:	(TP to pCR 38.765) Sensing procedures and singalling

Document for:	Other

1	Introduction

Study the procedures, signaling between RAN and CN to support ISAC [RAN3]

Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3]

- Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts.

- No inter-gNB coordination will be studied.

- Coordination with SA2 as necessary.

In this contribution, we’d like to discuss the sensing procedures and signalling of R20 5GA ISAC according to the following SID in [1] and the progresses we mad in [2].

2	Discussion

2.1 	General Sensing functions of gNB-SF interface

In last RAN3 meeting, a sensing reporting procedure including sensing request/response/report has been captured in the baseline pCR in [2]. During the discussions, some companies proposed starting with the general functions of the gNB-SF interface and then studying the corresponding procedures for each function. We believe this is a good approach for structuring the study and discussion, therefore the functions of gNB-SF interfaces are discussed first in this contribution.

In general, we think the following functions of gNB-SF interface (i.e., Nx in pCR) can be further studied in RAN3.

Sensing Support Information Exchange: This function allows an SF to obtain sensing support information from a gNB. The specific details of the information remain under discussion. Based on SA2’s progress on gNB selection in [3], an SF may require knowledge of a gNB’s supported sensing area in order to make gNB selection decisions. By default, such information can be obtained by SAF via OAM. However, since a gNB may (de)-activate certain TRPs for sensing e.g., due to network load or energy-saving considerations, its supported sensing area may change over time. This dynamic nature highlights the need for a signaling-based solution to ensure that the information remains up to date.

Observation 1, According to SA2, the SF selects gNB(s) for sensing based on factors such as the gNB’s supported sensing area, which can be obtained via OAM by default.

Observation 2, Since the supported sensing area of a gNB may change with the activation or deactivation of its TRPs for sensing, a signaling-based mechanism is required to ensure the exchange of accurate and up-to-date information.

Sensing Initiation and Reporting: This function enables the SF to request sensing to a gNB and receive the corresponding results from the gNB. While RAN3 has agreed on this sensing reporting procedure in [2], we believe the term “sensing initiation and reporting” is more appropriate than “sensing reporting”, as it encompasses both the initiation of sensing and the reporting of measurement results.

Sensing Stop: this function allows gNB to stop sensing and release the sensing related resource, and the sensing stop may be triggered by SF or gNB.

Regarding other procedures, such as sensing modification, we believe the approach depends on the information contained in the sensing request, the specific elements that require updating, and whether the update can be achieved by issuing a new sensing request. These aspects can be further discussed once the fundamental procedures and signaling are clearly defined.

Observation 3, “sensing initiation and reporting” is more appropriate than “sensing reporting”, as it encompasses both the initiation of sensing and the reporting of measurement results.

Observation 4,  other procedures (e.g., sensing modification) can be further discussed once the fundamental procedures and signaling are clearly defined.

Proposal 1, RAN3 agrees the following gNB-SF interface functions for procedure discussion:

- Sensing Support Information Exchange

- Sensing Initiation and Reporting

- Sensing Stop

2.2 	Sensing initiation and reporting

As RAN3 already agreed the sensing reporting procedures in [2] in last meeting, we start the information discussion on this procedure in this section.

Information in sensing request

In general, we think the following information is needed in sensing request:

| Information in sensing request | Motivation |
| --- | --- |
| Sensing measurement ID | In the NRPPa protocol, an LMF Measurement ID is used within the measurement procedure and measurement report procedure to uniquely identify a specific measurement. Similarly, for sensing, we propose introducing a dedicated Sensing Measurement ID to serve the same purpose. |
| Report configuration | As agreed below in section 7.1.4 in SA2 TR in [3], different reporting mode (e.g., one-shot, periodic) are considered for a sensing service, the selected gNB also needs to know it to generate and send reports accordingly. - Sensing Entity can support one time, periodical Sensing Data to the Sensing Function. Editor's note: The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.. Further discussions may be needed to define the assistance information associated with each reporting mode, which can be addressed in upcoming RAN3 meetings. |
| Requested sensing information (target sensing area or requested TRP) | As we discussed in [4], a gNB may control multiple TRPs that collectively cover a wide area. If the target sensing area is smaller than the supported sensing area of a gNB, not all the TRPs need to perform sensing. Then, how to select TRP needs to be discussed. Typically, it depends on the target sensing area provided in the sensing service request from the consumer. The following options are proposed to be discussed in RAN3: Option 1: The gNB selects the TRP based on the sensing area information provided in the sensing request. This implies that the target sensing area information should be included in the sensing request message. Option 2: The SF selects the TRP based on the sensing area information from the sensing service request and the supported sensing capabilities of each TRP, which are configured via OAM or signaling. In this case, the requested TRP information should be included in the sensing request message. (similar to positioning) As we discussed in [4], we prefer option 1, so that gNB does not need to notify the TRP information to the SF, gNB can select the TRPs according to the target sensing area by its implementation. |

Proposal 3, RAN3 agrees the following information can be included in a sensing request:

- Sensing measurement ID

- Report configuration

- Requested sensing information (i.e., target sensing area)

Information in sensing response/report

The following information is needed in sensing response/report.

| Information in sensing response/report | Motivation |
| --- | --- |
| Sensing Measurement ID | Same as positioning, the measurement ID should be included in the sensing response/report to identify a specific sensing measurement. |
| Sensing Measurement Results | As RAN1 is still discussing the types of sensing measurements, the specific measurement formats and content are yet to be determined. Therefore, RAN3 can await the outcome of RAN1’s discussions. As we discussed in [4], the sensing measurement results may use different transport protocols from the sensing control signalling. Therefore, the sensing measurements can be included in sensing report, but FFS on whether it can also be included in sensing response, which also depends on the measurement types. |

Proposal 4, RAN3 agrees that sensing response message at least include the sensing measurement ID, and the sensing report message includes the sensing measurement ID and sensing measurement results.

Proposal 5, The detailed structure and content of the sensing measurement results will be defined based on the conclusions from RAN1.

2.3 	Sensing Support Information Exchange

As we discussed in section 2.1, we see the necessity of introducing the sensing support information exchange at least for gNB selection purpose. However, using which procedure to realized it may depend on the architecture and protocol design. The following options may need further discussion with taking SA2’s conclusion into account.

| Options | Procedures to be used for sensing support information |
| --- | --- |
| Option 1 Supported information is included in the connection setup or general configuration update procedure | gNB initiated connection setup Nx could be new protocol for direct architecture or NG if NG is re-used. |
| Option 2 Supported information is included in a Dedicated procedure for sensing | SF-initiated procedure The connection is pre-configured or already setup. (the connection between gNB and SF is established by OAM or previous procedures) |

Observation 5, the procedures for sensing support information exchange depends on the on-going architecture and protocol discussions.

Proposal 6, RAN3 study the following procedure options to support sensing support information exchange.

- option 1, gNB initiated interface setup or general configuration update procedure

- option 2, SF initiated dedicated procedure for sensing support information

2.4 	Sensing Stop

The following examples can be referred for sensing stop procedure study.

| Examples | Procedures |
| --- | --- |
| Positioning | In positioning, the following class 2 procedures for measurement stop - Measurement Abort (class 2, LMF-initiated), enable the LMF to abort an on-going measurement - Measurement Failure Indication (class 2, gNB-initiated), allows the gNB to notify the LMF that the measurements previously requested with the Measurement procedure can no longer be reported |
| Ambient IoT | In Ambient IoT, a class 1 and class 2 procedures for A-IoT session termination - A-IoT Session Release (class 1, LMF-initiated), enable the A-IoT CN node to terminate all activities related to the indicated A-IoT session - A-IoT Session Release Request (class 2, gNB-initiated), enable the NG-RAN node to request the A-IoT CN node to release the A-IoT session |

If sensing is expected to be dynamic, adaptive, and TRP-dependent, the positioning-style procedures (Measurement Abort + Measurement Failure Indication) are more appropriate. They allow both top-down and bottom-up control at a granular level.

If sensing is expected to be session-based, with clear start and end points, the Ambient IoT-style procedures (Session Release + Session Release Request) may be more suitable, ensuring clean session lifecycle management, session-based is also depends on the protocol design of sensing results transmission. If there’s GTP-U tunnel established between gNB ans SF, the Ambient IoT-style procedures are more appropriate.

At least, RAN3 can agree that both gNB and SF may initiated sensing stop, details are FFS.

Observation 6, The choice of sensing stop procedure depends on the operational model: positioning-style procedures are better suited for dynamic, adaptive, and TRP-dependent sensing requiring fine-grained control, while Ambient IoT-style procedures are more appropriate for session-based sensing with clear start and end points, particularly when the sensing results transmission relies on a GTP-U tunnel between the gNB and SF.

Proposal 7, RAN3 agrees to study both SF-initiated and gNB-initiated sensing stop procedures, FFS on class 1 or class 2.

In the Annex, we provide the corresponding TP to pCR 38.765 to capture the proposals in this paper.

Proposal 8, RAN3 agrees the TP to pCR 38.765 in the Annex.

3	Conclusion

In this contribution, we discussed sensing procedures and signalling, and have the following proposals.

General Sensing function of gNB-SF interface

Observation 1, According to SA2, the SF selects gNB(s) for sensing based on factors such as the gNB’s supported sensing area, which can be obtained via OAM by default.

Observation 2, Since the supported sensing area of a gNB may change with the activation or deactivation of its TRPs for sensing, a signaling-based mechanism is required to ensure the exchange of accurate and up-to-date information.

Observation 3, “sensing initiation and reporting” is more appropriate than “sensing reporting”, as it encompasses both the initiation of sensing and the reporting of measurement results.

Observation 4, other procedures (e.g., sensing modification) can be further discussed once the fundamental procedures and signaling are clearly defined.

Proposal 1, RAN3 agrees the following gNB-SF interface functions for procedure discussion:

- Sensing Support Information Exchange

- Sensing Initiation and Reporting

- Sensing Stop

Sensing initiation and reporting

Proposal 3, RAN3 agrees the following information can be included in a sensing request:

- Sensing measurement ID

- Report configuration

- Requested sensing information (i.e., target sensing area)

Proposal 4, RAN3 agrees that sensing response message at least include the sensing measurement ID, and the sensing report message includes the sensing measurement ID and sensing measurement results.

Proposal 5, The detailed structure and content of the sensing measurement results will be defined based on the conclusions from RAN1.

Sensing Support Information Exchange

Observation 5, the procedures for sensing support information exchange depends on the on-going architecture and protocol discussions.

Proposal 6, RAN3 study the following procedure options to support sensing support information exchange.

- option 1, gNB initiated interface setup or general configuration update procedure

- option 2, SF initiated dedicated procedure for sensing support information

Sensing Stop

Observation 6, The choice of sensing stop procedure depends on the operational model: positioning-style procedures are better suited for dynamic, adaptive, and TRP-dependent sensing requiring fine-grained control, while Ambient IoT-style procedures are more appropriate for session-based sensing with clear start and end points, particularly when the sensing results transmission relies on a GTP-U tunnel between the gNB and SF.

Proposal 7, RAN3 agrees to study both SF-initiated and gNB-initiated sensing stop procedures, FFS on class 1 or class 2.

Proposal 8, RAN3 agrees the TP to pCR 38.765 in the Annex.

4	Reference

[1] RP-252819, “Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR”, Xiaomi, China Telecom

[2] R3-257332, (pCR to TR 38.765) Study on Integrated Sensing And Communication (ISAC) for NR, China Telecom, Xiaomi

[3] 3GPP TR 23.700-14, “Study on Integrated Sensing and Communication”, Sep. 2025.

[4] R3-258114, (TP to pCR) ISAC general aspects and protocol stacks, Xiaomi

5	Annex (TP for pCR 38.765)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

8	RAN-CN procedures and signalling

Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

The Nx protocol provides the following functions:

-	Sensing Support Information Exchange: This function allows an SF to obtain sensing support information from a gNB.

-	Sensing Initiation and Reporting: This function enables the SF to request sensing and receive the corresponding results from a gNB.

-	Sensing Stop: this function allows LMF or gNB to stop sensing and release the sensing related resource.

The functional procedures is shown in Figure 8-1.

Figure 8-1: Functional Procedures for sensing

- An SF obtains the sensing support information of a gNB via sensing support information procedure, it may use it for gNB selection when when a sensing request is received.

- The SF selects the gNB(s) for sensing, initiates the sensing process in the selected gNB(s) and receives the corresponding sensing reports.

- Upon completion of sensing or in the event of sensing failure, a sensing stop procedure is performed to terminate or stop the sensing operation.

8.x1	Sensing Initiation and Reporting

Editor’s Note: the following may need further refinement.

Figure 8.x1-1: Message flow for sensing initiation and reporting

1.	The SF sends sensing request to the gNB, it includes the sensing measurement ID, report configuration, and the  requested sensing information (i.e., the target sensing area) in the sensing request, gNB shall use the included information to perform sensing and report the sensing results.

2.	The gNB sends sensing response including the sensing measurement ID to the SF.

NOTE 1, the gNB may instead send a sensing failure to the SF indicating that the sensing procedure could not be performed.

3.	If requested in Step 1, the gNB sends sensing report(s) including the sensing measurement ID and the sensing mesurement results to the SF.

8.x2	Sensing Support Information Exchange

Editor’s Note: the following may need further refinement.

There are two alternative procedures for sensing support information exchange.

One candidate procedures is shwon in Figure 8.x2-1.

Figure 8.x2-1: Candidate Message flow for sensing support information exchange

- The gNB sends xx request to the SF, it includes the supported sensing information of a gNB, SF may use it to perform the gNB selection.

Editor’s Note: the XX request/response message can be Nx setup request/response, or RAN configuration update/ack or new gNB intiated procedures.

2.	The SF sends XX response to the gNB.

One candidate procedures is shwon in Figure 8.x2-2.

Figure 8.x2-2: Candidate Message flow for sensing support information exchange

- The SF sends sensing support information request to the gNB, it may request the supported sensing information of a gNB

2.	The gNB sends sensing support information response to the SF, if requested in step 1, it includes the supported sensing information of a gNB, 1.SF may use it to perform the gNB selection.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>