3GPP TSG-RAN WG3 Meeting #130	R3-258140

# Dallas, USA, November 17-21, 2025

Agenda item:		10.2.1

Source:	T-Mobile USA, Nokia, FiberCop, Qualcomm

Title:	Service Aware RAN Requirements for 6G

Document for:		Discussion and Agreement

# Introduction

Existing 5G systems provide QoS mechanisms that are largely static and do not enable near real-time observability or adaptation of RAN behavior based on service characteristics or application performance. With the emergence of advanced 6G use cases such as Robotics, Integrated Sensing and Communication (ISAC), and AI-native applications, the current QoS model needs to be enhanced.

Service Aware RAN introduces mechanisms that make RAN adaptive, programmable, and responsive to service types and application-level conditions in real time. This contribution proposes that RAN3 agrees to Service-Aware RAN concept as a key requirement for 6G RAN architecture.

# Discussion

## Motivation

In 5G, 5QI values define service-level QoS parameters set by the Core Network. RAN receives 5QI information but has no visibility into real-time service KPIs or user experience, limiting its ability to optimize scheduling, mobility, and resource control for latency- or throughput-sensitive applications.

This limitation results in:

- Inefficient resource utilization for services such as XR, Robotics, and Agentic AI

- The need for over-dimensioned RAN deployments to guarantee QoS

- Automated policy enforcement tied to service type

Service-Aware RAN addresses these issues by enabling RAN to dynamically:

- Detect and classify service types

- Monitor real-time service performance and KPI deviations

- Adjust RAN functions, RAN operation, and resource allocation to maintain SLAs

Gaps identified in existing 5G architecture:

| Area | Limitation | Required Enhancement |
| --- | --- | --- |
| QoS Enforcement | Static 5QI definitions | Near real-time adaptive QoS mechanisms |
| Observability | No RAN-level service observability | Near real-time application QoS attributes |
| Policy Control | Limited or static policy application | Application type based policy enforcement adaptation |
| Automation | Manual configuration | Closed-loop AI-assisted automation |
| Modularity | Tight coupling of control and user planes | Support for flexible control and user plane |

Proposal 1: RAN3 to Agree on Service-Aware RAN Requirements for 6G RAN Architecture

- Service characteristics and awareness—enable RAN to identify service types, map them according to QoS profiles and assigned network slices.

- Near real-time application observability to RAN—support observability mechanisms to observe application performance, detect and report violations of measurable QoS attributes to management entities.

- Policy enforcement and adaptation—support dynamic enforcement of slice- or application type-based policies that enforce specific management such as resource management, traffic shaping, compute/memory allocation.

- Closed-loop automation—apply feedback mechanisms using inputs from UE and network measurements to continuously monitor and adjust RAN performance that meets slice SLAs.

- Flexibility and modularity—apply flexible control plane and user plane

# Proposal

Based on the discussion in the previous sections, we propose the following:

Proposal 1: RAN3 to Agree on Service-Aware RAN Requirements for 6G RAN Architecture

- Service characteristics and awareness—enable RAN to identify service types, map them according to QoS profiles and assigned network slices.

- Near real-time application observability to RAN—support observability mechanisms to observe application performance, detect and report violations of measurable QoS attributes to management entities.

- Policy enforcement and adaptation—support dynamic enforcement of slice- or application type-based policies that enforce specific management such as resource management, traffic shaping, compute/memory allocation.

- Closed-loop automation—apply feedback mechanisms using inputs from UE and network measurements to continuously monitor and adjust RAN performance that meets slice SLAs.

- Flexibility and modularity—apply flexible control plane and user plane

# References

[1] RP-251986, T-Mobile USA, AT&T, Nokia, Ericsson, Telstra, Verizon, “Requirements for Service Aware 6G RAN”

[2] R3-256716, T-Mobile USA, Qualcomm, Nokia, “6G Service Aware RAN Architecture Option”

[3] R2-2508358, T-Mobile USA, “6G Service Aware RAN - Operator View”