3GPP TSG-RAN WG3 Meeting #130	R3-258143

Dallas, USA, 17-21, November, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0208 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Positioning Data Unavailable IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Ericsson, Xiaomi, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_air-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedure for data collection for NG-RAN-sided models for UEs undergoing positioning has been captured in Section 6.23 of TS 23.273. If user consent is not granted, the LMF sends an appropriate indication to the NG-RAN as defined in TS 38.455. Therefore, SA2 clearly indicates that a lack of user consent is one reason the LMF may be unable to provide the requested data to the NG-RAN. The Positioning Data Unavailable IE within the POSITIONING DATA COLLECTION REPORT message only indicates 'not supported' or 'not available'. This does not clearly specify that the inability to provide the data is due to user consent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce the new codepoint "no user consent" in the Positioning Data Unavailable IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is missing the reason that the LMF may be unable to provide the requested data to the NG-RAN due to user consent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Start of changes >>>>>>>>>> >>>>>>>>>> >>>>>>>>>>

9.1.5	Messages for Positioning Data Collection Information Transfer Procedures

9.1.5.1	POSITIONING DATA COLLECTION REPORT

This message is sent to report positioning information for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Positioning Data Information | O |  | 9.2.107 |  | YES | ignore |
| Positioning Data Unavailable | O |  | ENUMERATED(not supported, not available, …, no user consent) |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>> >>>>>>>>>> >>>>>>>>>>

9.3.5	Information Element definitions

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Skipped >>>>>>>>>> >>>>>>>>>> >>>>>>>>>>

#### -- P

PositioningDataUnavailable ::= ENUMERATED {

not-supported,

not-available,

...,

no user consent}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of changes >>>>>>>>>> >>>>>>>>>> >>>>>>>>>>