3GPP TSG-RAN WG3 Meeting #130	R3-258156

Dallas, US, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1607 | Rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interaction between Handover Success and Cell Switch Notification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, LG Electronics, NTT DoCoMo, ZTE, Offinno, Jio Platforms, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | After LTM failure, the UE may perform fast LTM recovery towards a candidate cell different from the target cell in the target gNB. In this case, the target gNB will still send the HANDOVER SUCCESS message to the source gNB, but with a Requested Target Cell ID different from the one indicated in the CELL SWITCH NOTIFICATION message. The source gNB should treat this case normally to avoid any error handling. Hence, further clarification in stage 3 is needed as the interaction between Handover Success and Cell Switch Notification in the case of fast LTM recovery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made to the XnAP for inter-CU LTM. - Add stage 3 text for interaction between Handover Success and Cell Switch Notification in case of fast LTM recovery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is ambiguous in case of fast LTM recovery leading to wrong handling at source. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

### 8.2.8	Handover Success

#### 8.2.8.1	General

The Handover Success procedure is used during a conditional handover or a DAPS handover or LTM to enable a target NG-RAN node to inform the source NG-RAN node that the UE has successfully accessed the target NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.2.8.2	Successful Operation

Figure 8.2.8.2-1: Handover Success, successful operation

The target NG-RAN node initiates the procedure by sending the HANDOVER SUCCESS message to the source NG-RAN node.

If late data forwarding was configured for this UE, the source NG-RAN node shall start data forwarding using the tunnel information related to the global target cell ID provided in the HANDOVER SUCCESS message.

When the source NG-RAN node receives the HANDOVER SUCCESS message, it shall consider all other CHO preparations accepted for this UE under the same UE-associated signalling connection in the target NG-RAN node as cancelled.

Interactions with other procedures

If a CONDITIONAL HANDOVER CANCEL message was received for this UE prior the reception of the HANDOVER SUCCESS message, the source NG-RAN node shall consider that the UE successfully executed the handover.

The source NG-RAN node may initiate Handover Cancel procedure towards the other signalling connections or other candidate target NG-RAN nodes for this UE, if any.

If a HANDOVER SUCCESS message is received from the target NG-RAN node to which a CELL SWITCH NOTIFICATION message was sent, and the Requested Target Cell ID IE contained in the HANDOVER SUCCESS message is not the target cell indicated in the corresponding CELL SWITCH NOTIFICATION message, the source NG-RAN node shall consider that the UE has performed fast LTM recovery in the cell indicated by the Requested Target Cell ID IE successfully.

#### 8.2.8.3	Unsuccessful Operation

Not applicable.

<< End of Changes >>