3GPP TSG-RAN WG3 Meeting #130	R3-258185

Dallas, US, 17th ~21th November 2025

Title:	Consideration on A-IoT Device Identification

Agenda Item:	9.2.7

Source:	Huawei, CMCC, China Unicom, China Telecom, Lenovo

Document for:	Discussion

# Introduction

In this contribution, we further discuss the A-IoT Device Identification received in Inventory Request, based on other WGs progresses.

# Discussion

Currently, in NGAP spec, the A-IoT Device Identification Requested IE is encoded as follows:

#### 9.3.3.73	A-IoT Device Identification Requested

This IE represents the Requested Identity of the A-IoT device(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Device Identification for Paging | M |  |  |  |
| >Single Device |  |  |  |  |
| >>Single Device Identity | M |  | BIT STRING (SIZE(1..1024, ...)) | Includes the AIoT device Identifier defined in TS 23.369 [60]. |
| >Group Devices |  |  |  |  |
| >>Group Device Identity | M |  | BIT STRING (SIZE(1..1024, ...)) | Includes the Filtering Information defined in TS 23.369 [60]. |
| >All Devices |  |  | NULL |  |

This was implemented based on SA2 previous description:

The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.8, or a single AIoT Device Identifier.

It is noted that in the LS from SA2 S2-2509655, SA2 further agreed a CR S2-2509652, in which this part is updated to:

From RAN3 point of view, in the the Semantics decstiption part, we should update both of the single device branch and group device branch to refer to “AIoT Device Identification Information defined in TS 23.369 [60]”.

Proposal: For the A-IoT Device Identification Requested IE, update both the Semantics descriptions of the Single Device branch and Group Devices branch to “Includes the AIoT Device Identification Information defined in TS 23.369 [60]”.

# Conclusion

In this contribution, we further discussed the A-IoT Device Identification received in Inventory Request, get the following proposals:

Proposal 2: For the A-IoT Device Identification Requested IE, update both the Semantics descriptions of the Single Device branch and Group Devices branch to “Includes the AIoT Device Identification Information defined in TS 23.369 [60]”.

The corresponding NGAP CR is provided in [1].

# Reference

[1]	R3-258186 Correction on A-IoT Device Identification, R19 NGAP CR