3GPP TSG-RAN WG3 Meeting #130	R3-258196

Dallas, US, 17th–21st  November, 2025

Agenda item:	13.2

Source: 	Tejas Networks

Title: 	Network Architecture and Protocol stack for NR ISAC

Document Type:	Discussion

# Introduction

In RAN3 #129Bis, logical architecture for ISAC as shown in Figure 1 was agreed to be captured in TR.

Figure 1 Logical architecture for ISAC

In SA2 #171, following interim agreements were captured for KI#1 System Architecture to Support Sensing.

7 Interim Agreements 7.1 Agreed Principles 7.1. Y Agreed Principles for KI#1 System Architecture to Support Sensing To supports KI#1 System Architecture to Support Sensing, the following principles are concluded: Principle 1: one new Network Function (i.e., Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF). NOTE 1: There is no standardized interface between the SCF and SPF in Rel-20 5G-A. NOTE 2: How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g., co-located or separated, can be discussed during the normative phase. NOTE 3: The final name and Acronym for Sensing Function may need further update if necessary. Principle 2, NO dedicated storage NF is needed to store the Sensing data and Sensing result. Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE). NOTE 4: Privacy protection and other security aspects will be coordinated with SA WG3, and the related impact to architecture enhancement will be based on SA WG3 during the normative phase. NOTE 5: Other KI’s interim conclusions will be aligned with KI#1 conclusions.

In this contribution, we discuss the sensing protocol for supporting Integrated Sensing and Communication (ISAC) for NR.

# Discussion

## 2.1 Architecture 1: Indirect interface

The Sensing Function (SF) receives a request for sensing from a third party. The SF then sends a sensing service request to an AMF. The AMF forwards the sensing service request towards gNB(s) via NGAP. The gNB process the sensing request, performs sensing and reports sensing result to the SF in control plane (via AMF).

Architecture 1 supports sensing in RAN with changes to control plane alone. This allows SF to transparently connect to RAN nodes via AMF using a new NR Sensing protocol (NRSP). The sensing configuration and sensing result are exchanged over NRSP.

Figure 2 Sensing overall reference architecture -1 for NG-RAN

Proposal 1: In case of Architecture 1, NG interface is reused, RAN3 need to enhance NGAP specification to support new sensing functionalities, procedures and messages.

In this option, the gNB shall connect with the Sensing Function like the way it was designed for the Positioning feature. In this case, the existing NG interface can be reused, and a new application protocol NRSP, similar as the NRPPa, should be defined. Furthermore, the communication between the gNB and the Sensing Function can be achieved by the new application protocol on the top of NGAP. The ISAC specific information should be transferred transparently through the Sensing-PDU over NG interface. In this option, the protocol stack can be designed like the Positioning as shown in Figure 3, with the new application protocol on top over the NGAP.

Figure 3 Protocol Layering for SF to NG-RAN Signaling for Architecture-I

## 2.2 Architecture 2: Direct Interface

Figure 4 Sensing overall reference architecture -2 for NG-RAN

In Architecture 2, gNB connects to Sensing Function with a new direct interface dedicated for sensing and without involving AMF and UPF/SMF. Along the initial UAV use case, many more use cases will be considered for ISAC. As number of use cases increases, the sensing traffic between RAN and the SF is expected to be significant. A new interface is proposed to reduce signaling latency and enable more efficient sensing message interactions between RAN and the SF.

Proposal 2: The new interface contains control protocol stack or both control plane and data plane between NG-RAN node and the Sensing function depending on the sensing use cases.

In case of Alternative 3, RAN connecting to the new SF directly, RAN3 may need to define a new interface between the RAN node and the SF. If a new interface is introduced, considering that the new protocol is used for context management and signalling transmission over new interface for sensing, the protocol stack of NxAP can use the same protocol stack as NG-C, i.e., including NxAP/SCTP/IP/Date link layer/Physical layer. Need for a direct data plane between RAN and the SF shall be part of study.

Proposal 3: In case of Architecture-2, RAN3 needs to introduce a new interface (Nx), and the new interface should use protocol stack NxAP/SCTP/IP as other RAN3 control plane interfaces and GTP/UPDP/IP for data plane as other RAN3 user plane interfaces

Figure 5 Protocol Layering for SF to NG-RAN Signaling for Architecture-2

The Nx data plane interface (Nx-U) can be defined between the NG-RAN node and the SF. The data plane protocol stack of the Nx interface is shown on Figure 6. The transport network layer is built on IP transport and GTP-U is used on top of UDP/IP to carry the Sensing data between the NG-RAN node and the SF following design similar to user plane protocol stack for other network interfaces. In case of Architecture-1, control plane signaling should be done using indirect interface via AMF and the Nx-U data plane interface should be used for sensing data transfer between gNB and SF.

Figure 6 Nx-U Protocol Stack

Figure 7 Nx User plane Protocol Stack

In case of split gNB architecture, a gNB-DU will include TRP and sensing functionality. gNB-CU will pass the sensing configuration/requests received from SF to gNB-DU via F1AP. gNB-DU will perform the sensing and generates sensing result and sends it to gNB-CU via F1AP. As discussed above, a new data plane may also be needed between gNB-CU and gNB-DU depending on sensing result size, frequency etc. While, the discussion of the aggregated gNB should be prioritized in this release, and the details of split architecture on ISAC could be discussed later, depending on the progress of the aggregated gNB.

Proposal 4: CU/DU split gNB for sensing should be discussed based on the solution for aggregated gNB in R20.

# Conclusion

Proposal 1: In case of Architecture 1, NG interface is reused, RAN3 need to enhance NGAP specification to support new sensing functionalities, procedures and messages.

Proposal 2: The new interface contains control protocol stack or both control plane and data plane between NG-RAN node and the Sensing function depending on the sensing use cases.

Proposal 3: In case of Architecture 2, RAN3 needs to introduce a new interface (Nx), and the new interface should use protocol stack NxAP/SCTP/IP as other RAN3 control plane interfaces and GTP/UPDP/IP for data plane as other RAN3 user plane interfaces

Proposal 4: CU/DU split gNB for sensing should be discussed based on the solution for aggregated gNB in R20.

# Reference

- RP-251861, New SID: Study on Integrated Sensing And Communication (ISAC) for NR

- RP-252819, Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR

- TR 22.837: "Study on Integrated Sensing and Communication".

- TS 22.137: "Integrated Sensing and Communication"

- R3-257332, (pCR to TR 38.765) Study on Integrated Sensing And Communication (ISAC) for NR, Prague, Czech Republic, 13 – 17 October 2025

- RAN3 #129bis Chair Notes

# Annex: