3GPP TSG-RAN WG3 Meeting #130	R3-258240

Dallas, USA, 17 – 21 Nov. 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on A-IoT Inventory and Command procedure figures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In Clause 16.23.6 Inventory Procedure, the Step 7 in Figure 16.23.6-1 states “A-IoT Session Release as defined in clause 16.x.8”. And it should be replaced by the “16.23.8” to align the procedure description. - Similar issue for Clause 16.23.7 Command Procedure. Update the Step 1 description from “Inventory procedure as specified in 16.x.6 step 1~6” to “Inventory procedure as specified in 16.23.6 step 1~6”. And Update the Step 5 description from “A-IoT Session Release as defined in clause 16.x.8” to “A-IoT Session Release as defined in clause 16.23.8”. - In RAN3#127, RAN3 has an agreement that “AIoTF obtain the AIoT RAN information via OAM configuration. The AIoT RAN information includes supported AIoT area, served reader ID list.” However, the AIoTF obtain the served reader ID list is not reflected in current stage 2 description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the “A-IoT Session Release as defined in clause 16.x.8” to “A-IoT Session Release as defined in clause 16.23.8” in Figure 16.23.6-1. - Update the Step 1 description from “Inventory procedure as specified in 16.x.6 step 1~6” to “Inventory procedure as specified in 16.23.6 step 1~6” in Figure 16.23.7-1. - Update the Step 5 description from “A-IoT Session Release as defined in clause 16.x.8” to “A-IoT Session Release as defined in clause 16.23.8” in 16.23.6 step 1~6” in Figure 16.23.7-1. - Refine the wording to relftect the the AIoTF obtain the served reader ID list via OAM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some steps description is not clear for the inventory and command precedurec figures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.23.2, 16.23.6, 16.23.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

### 16.23.6	Inventory Procedure

Figure 16.23.6-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.23.6-1: Inventory procedure

<<<<<<<<<<<<<<Omit unrelated part>>>>>>>>>>>>>>>>

7.	If there is no follow-on command, the A-IOT CN triggers the A-IoT CN node initiated A-IoT Session Release procedure as defined in clause 16.23.8.

### 16.23.7	Command Procedure

Figure 16.23.7-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure.

Figure 16.23.7-1: Command procedure

The Command procedure addresses only one A-IoT device.

1.	Prior to the Command procedure, the Inventory procedure as defined in clause 16.23.6 is performed involving the concerned device and potentially other A-IoT devices.

<<<<<<<<<<<<<<Omit unrelated part>>>>>>>>>>>>>>>>

5.	Upon completion of all the command transmission for all related devices within the session, the A-IOT CN triggers the A-IoT CN node initiated A-IoT Session Release procedure as defined in clause 16.23.8.

Next Change

### 16.23.2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

<<<<<<<<<<<<<<Omit unrelated part>>>>>>>>>>>>>>>>

The AIOTF may request the gNB to perform an A-IoT operation within a specific area (requested service area). The requested service area may be indicated by means of a list of A-IoT Areas and/or a list of readers. The A-IoT Area is represented by an A-IoT Area ID.

The AIOTF is aware of the area within which the gNB supports provision of A-IoT services by means of OAM configuration. This area may be represented as A-IoT Areas and/or the reader ID list supported by the gNB.

The AIOTF may also be aware of the reader’s location by means of OAM configuration.

The AIOTF is aware of the mapping relationship among gNBs, readers and A-IoT areas by means of OAM configuration, as needed.

End Change