3GPP TSG-RAN WG3 Meeting #130	R3-258287

Dallas, USA, 17th – 21st November, 2025

Agenda Item:	10.3.1

Source: 	OPPO

Title:  	Discussion on 6G RAN-CN interfaces

Document for:	Discussion and Decision

- Introduction

In Rel-20 6G SID [1], the objectives led by RAN3 are captured as follows.

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

Besides, an interim milestone is set for RAN3 as shown below.

#### 4.1.1 Interim Milestone Interim results shall be delivered as per the milestones below, in coordination with the Rel-20 RAN Plenary 6G Study (FS_6G_RAN_Scen_Req). TSG#112 (June/2026): RAN1 to provide interim assessment on the following areas: - Waveform, modulation, channel coding: scope of enhancements beyond NR baseline ((2) a, c) - Channel bandwidth (min and max), frame structure, numerology ((2) b, d) - Basic sync signal structure and associated periodicity(ies) ((2) h) For objectives where RAN4 may be impacted, RAN1 shall coordinate with RAN4 early to enable the above assessment by June 2026. RAN3 to provide interim study results to allow TSGs to make a decision on: - RAN-CN interface: P2P vs SBI - RAN internal interfaces: CU-DU split, CP-UP split. NOTE: It is planned to decide on Release-21 timeline in June/2026.

This contribution would like to provide our considerations on RAN-CN interface.

- Discussion

During the migration of core network from 4G to 5G, one of the important evolutions of core network is to transit to service-based architecture, which paves the way for ongoing evolution and enhancements in network capability. Now, in 6G, whether to extend SBA to RAN-CN interface or not needs to be further studied.

In 5G, the interface between RAN node and AMF is P2P, and the interfaces between AMF and other NFs are service-based.

NAS signalling from/to UE is carried by NGAP signalling via the NG control plane interface, i.e., besides handling the NAS signalling for mobility management, i.e., NAS-MM, AMF also needs to forward the NAS signalling between UE and other NFs, e.g., NAS-SM, SMS, UE policy or LCS. The protocol stack is shown as below.

Similarly, from RAN node side, besides handling control plane signalling between RAN node and AMF, AMF also has to forward the control plane signalling of other NFs, e.g., NRPPa. The protocol stack is shown as below.

With new services such as AI/ML and sensing in 6G, heavy transport overhead is expected due to data collected from UE side and RAN node side, including both UE-associated data and non-UE-associated data. Especially for non-UE-associated large-volume data from RAN node side, e.g., sensing measurement data from TRP, AI/ML data of RAN node for training/model transfer, it is not applicable to transfer over user plane due to non-UE-associated nature, however relying on legacy control plane to transfer this kind of large-volume data also is questionable, considering that in case of heavy overhead for control plane signalling transfer, a centralized AMF and the P2P interface between RAN node and AMF would be the bottleneck.

- In case that heavy transport overhead is expected in 6G, especially for new services such as AI/ML and sensing, a centralized AMF and the P2P-based RAN-CN interface between RAN node and AMF are the bottleneck for control plane signalling transfer.

One potential approach in 6G is to convert P2P-based RAN-CN interface for control plane connection between RAN node and NFs (including AMF) to be service-based, which is definitely to be discussed and studied in RAN3. In our understanding, we are open to discuss SBI for RAN-CN interface, but it seems RAN3 has not enough knowledge or confidence to look into the detailed SBI protocol design, e.g., HTTP/2 or QUIC(HTTP/3)-based SBI solutions, which is supposed to be CT4’s work if correct. Therefore, a straightforward way forward is to discuss whether to support service-based RAN-CN interface or not based on use cases/requirements/pain points/benefits in RAN3, and leave the detailed protocol design of SBI solution to other WGs such as CT4.

- The detailed protocol design of SBI solutions, e.g., HTTP/2 or HTTP/3-based SBI, should be within the scope of CT4.

Furthermore, for RAN-CN interface for user plane connection, i.e., NG user plane between RAN node and UPF, it seems that no such use case is foreseen currently. Therefore, we understand that it is good to clarify this.

- No use case to support service-based RAN-CN interface for user plane connection between RAN node and UPF in 6G.

- RAN3 study whether to support service-based RAN-CN interface for control plane connection or not, and leave the detailed protocol design of SBI solution to other WGs, e.g., CT4, if supported.

As mentioned above, we understand the motivation to support service-based RAN-CN interface is more for new services in 6G, such as AI/ML and sensing. For legacy connectivity services, P2P interface should be kept anyway, considering the backward compatibility of core network evolution and the support of interworking between 5G and 6G, so further study is needed on the support of service-based interface.

- For service-based RAN-CN interface, RAN3 study the support for new services (e.g., Sensing, AI/ML) provided by 6G as starting point, FFS for legacy connectivity services.

3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- In case that heavy transport overhead is expected in 6G, especially for new services such as AI/ML and sensing, a centralized AMF and the P2P-based RAN-CN interface between RAN node and AMF are the bottleneck for control plane signalling transfer.

- The detailed protocol design of SBI solutions, e.g., HTTP/2 or HTTP/3-based SBI, should be within the scope of CT4.

- No use case to support service-based RAN-CN interface for user plane connection between RAN node and UPF in 6G.

- RAN3 study whether to support service-based RAN-CN interface for control plane connection or not, and leave the detailed protocol design of SBI solution to other WGs, e.g., CT4, if supported.

- For service-based RAN-CN interface, RAN3 study the support for new services (e.g., Sensing, AI/ML) provided by 6G as starting point, FFS for legacy connectivity services.

4. Reference

[1] RP-252912, New SID: Study on 6G Radio, NTT DOCOMO