3GPP TSG-RAN WG3 Meeting #130                                                                      R3-258309

Dallas, US, 17 – 21, November 2025

Source:	CATT

Title:	(TP to BL pCR for TR 38.765) Discussion on network architecture for ISAC

Agenda Item:	9.2.2

Document for:	Discussion

- Introduction

In RAN#108 meeting held in June 2025, the new study item on Integrated Sensing and Communication (ISAC) for 5G NR was approved. This topic only supports the gNB-based mono-static sensing for UAV target usecases. The objectives of this topic are described in the SID [1] as follows:

Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. o For other waveform and reference signals, companies are to share relevant information o No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. o FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

Per RAN SID [1], RAN3 will study the procedure, signalling between CN and RAN as well as the network architecture for sensing. In the last RAN3 meeting, the following conclusion was made [2]. The BL pCR for TR 38.765 [3] captures the logical architecture of ISAC in section 7.

Capture logical architecture for ISAC RAN3 focus on sensing protocol in coordination with SA2 To continue the discussion on protocol stacks, particularly the protocol stack for sensing reporting, to be aligned with SA2 conclusions.

In this paper, we discuss the network architecture for gNB-based mono-static sensing for UAV target use cases. Section 2 gives the details discussion and followed by Section 3 that summarizes the conclusion of this paper.

- Discussion

  - Sensing data reported to SF

According to the descriptions for the study item of 5G ISAC, which were approved in RAN and SA, the functionality split between RAN and SF is clear. However, the type of sensing measurement data generated by the gNB and reported to the SF is not determined, which is a big issue. Currently in RAN1, the categories of measurement data for NR ISAC are under discussion. The following levels of sensing measurement data for the UAV use case are proposed by companies during the RAN1#122bis meeting:

- Level 1: raw data, which is the time-domain samples or subcarrier-domain samples of the channel response from each antenna port, or anything equivalent - Level 2: (partial raw data) - Amplitude and phase profile of delay, and/or Doppler, and/or angle - E.g., Amplitude and phase profile of delay-angle - Level 3 per path (subset of Level 2): - Option 1: Delay, Doppler, angle, and power per detected path for a given time stamp - Definition: one path is associated with one delay, and one or multiple Doppler, and one or multiple angles - A path can correspond to one or multiple detected points - A path can correspond to one or multiple detected objects - Level 4 per point: - Option 1: Range, velocity, angle, and power per detected point for a given time stamp - Definition: A point is defined by one range/delay, one Doppler and one angle - Option 2: Position, velocity, and power per detected point for a given time stamp - Definition: A point is defined by one location and one velocity - Level 5: e.g., {position, velocity, no target ID} - Definition: Position and velocity for a detected object/target for a given time stamp - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is not reported - Level 6: e.g., {position, velocity, target ID} - Definition: Position and velocity for a detected object/target for a given time stamp - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is reported - SINR, RSRP from perspective of sensing - Time stamp

According to RAN1 discussion, the sensing data for sensing by gNB can be categorized into three types:

- Raw data (Level 1 and Level 2)

- Data for detected path/point (Level 3 and Level 4)

- Data for detected object/target (Level 5 and Level 6)

Observation 1：According to RAN1 study, the sensing data generated by gNB can be categorized into three types:

- Raw data (Level 1 and Level 2)

- Data for detected path/point (Level 3 and Level 4)

- Data for detected object/target detectionn (Level 5 and Level 6)

It is assumed that RAN1 may not preclude any kind of sensing data from above categories. In our opinion, since the measurement data of gNB for sensing needs to be transmitted to SF over the RAN-CN interface and RAN3 specify the signalling and procedures over RAN-CN interface, RAN3 should be responsible for deciding the type of sensing data reported from RAN to SF. This decision may be based on RAN1’s input if necessary.

Proposal 1: RAN3 to determine the type of sensing data which is generated by gNB and reported to SF.

There are series of physical processing from raw data to object/target detection. If the object/target is not detected by gNB, the gNB reports the raw data or data for detected path/point to SF and the rest processing have to be carried out by SF. On the contrary, if the object/target is detected by gNB and the data for detected object/target is reported to SF, the SF does not need to perform the object/target detection. In our view, there are multiple disadvantages in case the gNB does not detect the object/target:

- The algorithms (e.g., detection and false alarm algorithm) to generate the data for the detected path/point may be different in different gNBs, which leads to inconsistent path/point detection results depending on gNB implementation. It is difficult for SF to use these data to derive object/target detection appropriately without knowing the detailed handling carried out by gNB.

- There is no metric for raw data and data for detected path/point. Therefore, the performance of these results cannot be evaluated, which means that the performance of object/target detection based on these data cannot be guaranteed.

- Requires the enormous data to be reported from RAN to SF, which imposes to network heavily.

On the contrast, if the gNB detects the object/target and sends the sensing measurement data for detected objects/targets to SF, all above disadvantages can be avoided and thus the optimized performance can be achieved.

- The gNB has full information about the data processings implemented and thus can utilize the local information to derive correct object/target detection.

- All the gNBs have consistent metrics of performance based on the data for detected objects/targets.

- Require much smaller data to be reported to CN.

Thus, we think the gNB should at least be capable of detecting object/target and sending the data for detected objects/targets to SF.

Proposal 2: RAN3 approve that gNB can detect the object/target and send the data for detected objects/targets to SF.

  - How to send the sensing signalling/data

The general procedure was approved in the last meeting, which including Sensing Request, Sensing Response and Sensing Report exchanging between gNB and SF. Other messages and procedures, such as the Sensing Failure and Sensing Release (initiated by the gNB or CN), may also be introduced. But it’s not clear how to send the sensing signal and data.

Sensing signalling

The Sensing Request includes the configuration parameters used for gNB to decide how to perform sensing. The Sensing Response indicates that the gNB accepts the sensing request and the Sensing Failure indicates that the gNB rejects the sensing request. The Sensing signalling may include:

- Sensing Request/Response

- Sensing Failure

- Sensing Release (initiated by gNB or CN)

The sensing signalling should be delivered via C-plane, considering that the security and reliability need to be guaranteed.

Proposal 3: The sensing signalling should be delivered via C-plane protocol.

Sensing data

Sensing data refers to the measurement data for the detected object/target generated by the gNB. To fulfill the requirement of sensing results exposure, SF may configure different methods of sensing report to gNB, for example, one-time report, periodic report or event-triggered report. For one-time report, the sensing data can be included in the Sensing Response message and delivered via C-plane.

Proposal 4: The one-time report for sensing data can be included in the Sensing Response message.

Due to the real-time feature of the sensing service for UAV object detection or tracking, the gNB is probably configured with periodic or event-triggered sensing report. The continuous detection or tracking for the moving UAV objects may lead to successive data reporting from the gNB to SF over time. If the periodic sensing report is delivered via C-plane, the transmission would not be efficient due to the integrity protection applied by the C-plane protocols, but in the meantime, the continuous data will lead to high processing load to C-plane.

Thus, it would not be appropriate to deliver the sensing data via C-plane in that case. A data plane connection between gNB and SF is needed. The D-plane connections can be managed by the C-plane signalling, for example, it can be established, modified or released as needed.

Proposal 5: RAN3 discuss delivering the periodic or event-triggered report of sensing data via the D-plane, which is managed by the C-plane signalling.

  - Protocol stacks for C-plane

The last RAN3 meeting has agreed on the logical architecture for ISAC [3]. However, the arthitecture aspects are up to SA2 decision. In last SA2 meeting [6], some principles were agreed for KI#1: System Architecture to Support Sensing, as below:

Principle 1: one new Network Function (i.e., Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF). NOTE 1: There is no standardized interface between the SCF and SPF in Rel-20 5G-A. NOTE 2: How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g., co-located or separated, can be discussed during the normative phase. NOTE 3: The final name and Acronym for Sensing Function may need further update if necessary.

It can be observed that SA2 has agreed to introduce one new network function, but no conclusion is reached regarding the connection method between RAN and SF (direct connection or AMF involved indirect connection).

RAN3 agreed to focus on sensing protocol in coordination with SA2, namely, RAN3 should concentrate on the RAN own aspects, including the applicable interface and the protocol stack between the RAN and the CN.

The C-plane protocol stack should be generic and independent to the network architecture. Figure 1 shows the Control-plane protocol stack between gNB and CN:

Figure1.	Control-plane protocol stack for ISAC

The protocol stack supports both the indirect architecture where gNB connects to SF via AMF and the direct architecture where gNB connects to SF directly. In case of indirect architecture, the CN includes AMF and SF. While for direct architecture, the CN only includes SF. NxAP may be NGAP including the sensing signalling or carrying a new protocol. Alternatively, it can even be a new protocol. Accordingly, Nx-C interface may be NG-C interface or a new interface.

Thus, for this protocol stack, whether CN includes AMF and SF depends on what architecture is selected by SA2, and the details of NxAP need further discussion in RAN3. RAN3 can capture above protocol stack for C-plane in TR 38.765 with the notes that CN may include AMF and SF which is up to SA2 and NxAP needs further discussion.

Proposal 6: RAN3 capture above C-plane protocol stack for ISAC in TR 38.765, with the notes that NxAP needs further discussion and the CN may include AMF and SF which is up to SA2 decision.

  - Protocol stacks for D-plane

As we propose in section 2.2, the periodic or event-triggered report for sensing data would need to be delivered via D-plane. In D-plane, the gNB can connect to SF via a gateway function, or it can directly connect to SF. The D-plane network architecture should also be decided by SA2. Similar as the C-plane protocol stack, the D-plane protocol stack should be independent with the network architecture.

Figure 2 shows the D-plane protocol stack between gNB and CN:

Figure 2 D-plane protocol stack for ISAC

For this protocol stack, GTP-U and UDP protocols are used. If gNB connects to SF directly, the CN includes only SF. If the gNB connects to SF via other core-network function, the CN includes SF and other core-network function. Whether the CN includes SF and other core-network function or only includes SF depends on SA2 decision.

RAN3 can capture above D-plane protocol stack for ISAC in TR 38.765 with the note that the CN may include SF and other corenetwork function which is up to SA2 decision. The TP is provided in the Annex.

Proposal 7: RAN3 to capture above D-plane protocol stack for ISAC in TR 38.765 with the note that the CN may include SF and other core-network function which is up to SA2 decision.

- Conclusion

According to the analysis in section 2, we propose:

Proposal 1: RAN3 to determine the type of sensing data which is generated by gNB and reported to SF.

Proposal 2: RAN3 approve that gNB can detect the object/target and send the data for detected objects/targets to SF.

Proposal 3: The sensing signalling should be delivered via C-plane protocol.

Proposal 4: The one-time report for sensing data can be included in the Sensing Response message.

Proposal 5: RAN3 discuss delivering the periodic or event-triggered report of sensing data via the D-plane, which is managed by the C-plane signalling.

Proposal 6: RAN3 capture above C-plane protocol stack for ISAC in TR 38.765, with the notes that NxAP needs further discussion and the CN may include AMF and SF which is up to SA2 decision.

Proposal 7: RAN3 to capture above D-plane protocol stack for ISAC in TR 38.765 with the note that the CN may include SF and other core-network function which is up to SA2 decision.

- Reference

- RP-251861, New SID: Study on Integrated Sensing And Communication (ISAC) for NR, RAN#108, AT&T

- RAN3_129bis_agenda_20251017_EOM2

- 3GPP TR 23.700-14 V1.0.0 (2025-09)

- R3-257332, RAN3 BL pCR for study on ISAC for NR v1

- 3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

- S2-2509659, Interim agreement/Conclusion for KI#1 System Architecture to Support Sensing

- Annex (TP to BL pCR for TR 38.765)

<<<<<<<<<<<<<<<<<<<< Start of TP>>>>>>>>>>>>>>>>>>>>

7	Network architecture

Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied.

This clause identifies and describes the logical architecture to support the sensing in the overall 5G system architecture.

Figure 7.1 depicts a logical architecture for ISAC, where the Nx interface is between the gNB and the SF.

Figure 7.1	Logical architecture for ISAC

This logical architecture is independent of the transport, e.g. direct or via the AMF, between the gNB and the SF.

Editor’s Note:  The details of "Nx" interface need further discussion and decision.

Figure 7.2 shows the control-plane protocol stack between gNB and CN:

Figure 7.2	Control-plane protocol stack for ISAC

NOTE 1:	Whether "NxAP" is realised by including SF information in NGAP or by carrying a new protocol layer over NGAP, needs further discussion and decision.

NOTE 2: The CN may include AMF and SF which is up to SA2 decision.

Figure 7.3 shows the data-plane protocol stack between gNB and CN:

Figure 7.3	Data-plane Protocol Stack for ISAC

NOTE 3: The CN may include SF and other core-network function which is up to SA2 decition.

<<<<<<<<<<<<<<<<<<<< End of TP >>>>>>>>>>>>>>>>>>>>