3GPP TSG-RAN WG3 #130                                                    R3-258456

Dallas, US, 17th – 21st Nov, 2025

Agenda Item:	12.2.2

Source:	CATT

Title:	Discussion on support of AI/ML enabled intra-CU LTM

Document for:	Discussion

Introduction

In last RAN3 meeting, the following agreements and FFS have been achieved:

Prioritize normal intra-CU LTM scenarios.

Cover both L1 and L3 Measurement Report based Intra-CU LTM?

For CU-DU split architecture, the following two scenarios are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

RAN3 can further discuss whether the model inference can be located at gNB-DU to support AI/ML-based intra-CU LTM.

RAN3 to study AI/ML assisted candidate cell selection for LTM Handover Preparation

RAN3 to study TA value and validity predication for Intra-CU LTM in spatial and/or temporal domain

Beam prediction should also be considered?

For AI/ML study we consider both L1 and L3 measurement based Intra-CU LTM.

Study candidate beams along with candidate cells for AI/ML based Intra-CU LTM HO preparation.

Further discuss whether to study addition of beam prediction in UE Trajectory prediction.

Further discuss predicting TA value in the next meeting.

Further discuss prediction of validity time of a measured TA value.

Here we provide some analysis on AI/ML based intra-CU LTM.

Discussion

  - Description and objectives

In last RAN3 meeting, following FFS is discussed:

Cover both L1 and L3 Measurement Report based Intra-CU LTM?

In Release 19, L1 and L3 based LTM is supported in mobility enhancements WI. As for the new introduced L3 based LTM, its mechanism is similar like legacy L3 handover because it is also triggered by L3 report, and R18 AI/ML-based Mobility Optimization has support collecting L3 report to optimize L3 handover, so, there may be no specification impact to support it.

Proposal 1: It is proposed to support both L1 and L3 Measurement Report based Intra-CU LTM.

Following FFSs are related with optimization on TA value.

RAN3 to study TA value and validity predication for Intra-CU LTM in spatial and/or temporal domain.

Further discuss predicting TA value in the next meeting.

Further discuss prediction of validity time of a measured TA value.

LTM support early TA acquisition procedure, aiming to enable the UE performs a RACH-less LTM, and decrease handover interruption. Currently early TA acquisition procedure is triggered by source DU by implementation. However, each time UE execute early TA, a short interruption will happen at source node. To avoid unnecessary interruption, when to trigger the TA acquisition and the candidate cells to be triggered early TA shall be considered carefully. Besides, when source DU received the TA, it responsible for maintaining TA validity, how to effective maintain it is not discussed in LTM part. AI/ML model could be introduced to optimize TA acquisition procedure and guarantee provides valid TA value for LTM.

Proposal 2: It is proposed to introduce AI/ML assisted optimization on TA acquisition procedure and TA validation.

Optimization on candidate cells and candidate beams have been agreed in last RAN3 meeting captured below:

Study candidate beams along with candidate cells for AI/ML based Intra-CU LTM HO preparation.

Optimization on candidate cells selection can be supported by R18 mobility optimization case because LTM candidate cell is also selected based on L3 report which is the same as CHO, while how to optimize candidate beam selection needs further discussion. In our opinion, optimization on candidate beam should be performed based on beam perdition. The FFS on beam prediction is captured below in last RAN3 meeting:

Beam prediction should also be considered?

UE Location Prediction has been supported in R18 mobility optimization. While for LTM, TCI state is included in cell switch command which impacts LTM performance. AI/ML model can make prediction on beam level to facilitate better beam selection.

Proposal 3: It is proposed to support beam prediction to optimize beam selection.

  - Input/output/feedback of AI/ML Model Inference

To complete the above objectives, besides legacy input in R18 AI/ML-based Mobility Optimization, gNB may need following new information as input.

From the local node:

-	beam level UE trajectory prediction;

-	Configuration on triggering TA acquisition procedure (e.g., target cell ID, triggering time or period, RACH information);

Beam level UE trajectory prediction can assist gNB to select suitable TCI state included in cell switch command.

Information on TA acquisition procedure is used to optimize TA configuration to provide valid TA for LTM.

From the UE:

-	UE beam level location information

-	UE Mobility History Information with beam level

-	UE L1 and/or L3 measurement report

From neighbouring NG-RAN Nodes:

-	beam level UE’s history information from neighbour

-	TA information from neighbour after triggering TA acquisition procedure.

-	UE LTM result in past that were successful and unsuccessful

Beam level UE’s history information assists UE trajectory (beam) prediction.

TA information is used to optimize TA acquisition procedure.

AI/ML-based LTM model can generate following information as output:

-	beam level UE trajectory prediction

-	suggested candidate cells list

-	Predicted L1 execution condition, i.e. handover trigger points.

-	TA acquisition configuration (e.g.,target cell ID, triggering time or period, RACH information)

-	TA validity results

Following feedback can be considered to be collected from NG-RAN nodes:

-	beam level UE trajectory information;

-	RACH-less or RACH based access in target cell.

-	TA value used during RACH to target cell.

Feedback information from target NG-RAN is used by AI/ML Model to retrain or revise the Inference for mobility UE.

Proposal 4: It is proposed for RAN3 to discuss the above new Input/output/feedback of AI/ML Model Inference on the bases of R18 AI/ML-based Mobility Optimization.

  - Location of AI/ML Model Inference

In last RAN3 meeting, model inference located in the gNB-CU has been agreed as below:

For CU-DU split architecture, the following two scenarios are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

Further, whether model inference can be located at gNB-DU was discussed as below:

RAN3 can further discuss whether the model inference can be located at gNB-DU to support AI/ML-based intra-CU LTM.

For L1 based LTM, the benefit for model inference located in the gNB-DU is listed as following:

- gNB-DU receives L1 report, triggers LTM execution and TA acquisition procedure accordingly. AI/ML Model Inference deployed in gNB-DU should also be responsible for predicting LTM trigger point, TCI state, TA acquisition configuration.

- L1/L2 measure report is transferred to gNB-DU directly. Since the date volume of L1/L2 measure report may be very large, it is not proposed to transfer L1/L2 measure report from DU to CU.

- Beam level prediction is under discussion now. gNB-CU is not aware of UE beam level information. Since UE beam information may vary rapidly, gNB-DU could not provide beam level information to gNB-CU in time. Therefore, if beam level prediction is supported, model inference should be located in the gNB-DU.

Proposal 5: Considering the feature of LTM, model inference located in the gNB-DU should be discussed.

Conclusions

Proposal 1: It is proposed to support both L1 and L3 Measurement Report based Intra-CU LTM.

Proposal 2: It is proposed to introduce AI/ML assisted optimization on TA acquisition procedure and TA validation.

Proposal 3: It is proposed to support beam prediction to optimize beam selection.

Proposal 4: It is proposed for RAN3 to discuss the above new Input/output/feedback of AI/ML Model Inference on the bases of R18 AI/ML-based Mobility Optimization.

Proposal 5: Considering the feature of LTM, model inference located in the gNB-DU should be discussed.

Reference

[1] RAN3_129bis_agenda

[2] TR38.745

TP for TR38.745

4.2	AI/ML assisted Intra-CU LTM

- 4.2.1	Use case description

Editor’s Note: Capture the description of use case

L1/L2 Triggered Mobility (LTM) is specified in TS 38.300 [2].

Intra-CU LTM is specified in TS38.401[3].

AI/ML can be used to optimise Intra-CU LTM procedures, e.g., to enhance Network and UE performance, optimize resource allocation and reduce mobility failures.

- 4.2.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

- 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

For CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

-  AI/ML Model Inference is located in the gNB-DU

- 4.1.2.2		Input data of AI/ML based Intra-CU LTM

Editor’s Note: To be updated

From the local node:

-	beam level UE trajectory prediction;

-	Configuration on triggering TA acquisition procedure (e.g., target cell ID, triggering time or period, RACH information);

From the UE:

-	UE beam level location information

-	UE Mobility History Information with beam level

-	UE L1 and/or L3 measurement report

From neighbouring NG-RAN Nodes:

-	beam level UE’s history information from neighbour

-	TA information from neighbour after triggering TA acquisition procedure.

-	UE LTM result in past that were successful and unsuccessful

- 4.1.2.3		Output data of AI/ML based Intra-CU LTM

For AI/ML optimization of Intra-CU LTM the following information can be considered as output data:

- Candidate celland beam for LTM HO Preparation

-	beam level UE trajectory prediction

-	suggested candidate cells list

-	Predicted L1 execution condition, i.e. handover trigger points.

-	TA acquisition configuration (e.g.,target cell ID, triggering time or period, RACH information)

-	TA validity results

- 4.1.2.4		Feedback of AI/ML based Intra-CU LTM

The following data is required as feedback data for AI/ML optimization of Intra-CU LTM.

-	beam level UE trajectory information;

-	RACH-less or RACH based access in target cell.

-	TA value used during RACH to target cell.