3GPP TSG RAN WG3 Meeting #130	R3-258506

Dallas, USA, 17-21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0451 | rev | 2 | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the UE Initial Access procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The description used to reference the message sent by the UE in step 5) of the signalling flow for UE Initial access is wrong and does not match the message that is depicted in the associated figure 8.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace “RRC CONNECTION SETUP COMPLETE” with “RRCsetupComplete”. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has no impact on the protocol. This CR has no functionality impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent reference to signalling message that can lead to implementation confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS38.401CRxx TS38.401CR0454 TS38.401CR0453 TS38.401CR0452 TS38.401CR0451 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.1: Resubmission to R3#127-bis Rev.2: Resubmission to R3#130 |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Start of set of Changes >>>>>>>>>>>>>>>>>>>>

## 8.1	UE Initial Access

The signalling flow for UE Initial access is shown in Figure 8.1-1.

Figure 8.1-1: UE Initial Access procedure

1.	The UE sends an RRCSetupRequest message to the gNB-DU.

2.	The gNB-DU includes the RRC message and, if the UE is admitted, the corresponding low layer configuration for the UE in the INITIAL UL RRC MESSAGE TRANSFER message and transfers to the gNB-CU. The INITIAL UL RRC MESSAGE TRANSFER message includes the C-RNTI allocated by the gNB-DU. If the gNB-DU identifies the UE as a Reduced Capability UE during the random access procedure, a NR RedCap UE Indication is provided in the INITIAL UL RRC MESSAGE TRANSFER message.

3.	The gNB-CU allocates a gNB-CU UE F1AP ID for the UE and generates a RRCSetup message towards UE. The RRC message is encapsulated in -the DL RRC MESSAGE TRANSFER message.

4.	The gNB-DU sends the RRCSetup message to the UE.

5.	The UE sends the RRCSetupCompletemessage to the gNB-DU.

6.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

7.	The gNB-CU sends the INITIAL UE MESSAGE message to the AMF.

8.	The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU.

9.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message to establish the UE context in the gNB-DU. In this message, it may also encapsulate the SecurityModeCommand message. In case of NG-RAN sharing, the gNB-CU includes the serving PLMN ID (for SNPNs the serving SNPN ID).

10.	The gNB-DU sends the SecurityModeCommand message to the UE.

11.	The gNB-DU sends the UE CONTEXT SETUP RESPONSE message to the gNB-CU.

12.	The UE responds with the SecurityModeComplete message.

13.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

14.	The gNB-CU generates the RRCReconfiguration message and encapsulates it in the DL RRC MESSAGE TRANSFER message.

15.	The gNB-DU sends RRCReconfiguration message to the UE.

16.	The UE sends RRCReconfigurationComplete message to the gNB-DU.

17.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU.

18.	The gNB-CU sends the INITIAL CONTEXT SETUP RESPONSE message to the AMF.

<<<<<<<<<<<<<<<<<<<< End of set of Changes >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>