3GPP TSG-RAN WG3 Meeting #130	R3-258509

Dallas, USA, 17 – 21 November 2025

Agenda item:	9.2.5

Source:	Ericsson, CATT, Jio Platforms, Ofinno

Title:	Summary of Essential Corrections and Open Issues for Rel-19 LTM

Document for:	Discussion and Decision

# 1	Introduction

This contribution summarizes the essential corrections and remaining issues for Rel-19 LTM, based on the discussions and papers submitted at RAN3#129-bis. It consolidates relevant CRs and related topics to facilitate further discussion and agreement.

The objective is to clarify open points and reach consensus on the way forward for CR finalization at the RAN3#130 meeting.

# 2	For the Chairman’s Notes

Propose the following:

Stage-2 CRs to be agreed:

- R3-258263 Correction on SN initiated inter-SN SCG LTM procedure (Ofinno, Ericsson, CATT, Qualcomm, Huawei, Google, LG Electronics, Nokia, Samsung, Lenovo, China Telecom)

- R3-258510 Corrections for Rel-19 LTM (Ericsson, Jio Platforms, CATT, China Telecom, Lenovo, LG Electronics, Nokia, ZTE, Samsung, Ofinno)

- R3-258157 Correction on inter-CU LTM and conditional intra-CU LTM (Huawei, Jio Platforms, CATT, China Telecom, Lenovo, Ericsson, LG Electronics, Qualcomm Incorporated, Google, Ofinno, Nokia, Samsung)

Stage-3 CRs to be agreed:

- R3-258293 Correction on LTM Cell Switch Notification (LG Electronics, CATT, China Telecom, Ericsson, Google, Huawei, Jio Platforms, NEC, Nokia, NTT Docomo, Ofinno, Qualcomm, Samsung, Verizon, ZTE)

- R3-258294 Correction related to LTM Security Information (LG Electronics, CATT, China Telecom,  Ericsson, Google, Huawei, Jio Platforms, NEC, Nokia, NTT Docomo, Ofinno, Qualcomm, Samsung, Verizon, ZTE)

- R3-258508 Corrections on Inter-CU LTM (Ericsson, Lenovo, LG Electronics, Nokia, Samsung, ZTE, Ofinno)

- R3-258392 Essential correction for inter-CU LTM on F1AP (CATT, Huawei, China Telecom, LG Electronics, Nokia, Samsung, ZTE, Ofinno, Ericsson)

- R3-258390 Essential correction on inter-CU LTM in DC (CATT, Huawei, China Telecom, Nokia, Samsung, Ericsson)

Acknowledged issues to be discussed:

- Issue 1: Modification of LTM configurations, potentially including the release of PRACH resources

- Issue 2: Early RACH Resource Requester ID to be transferred over Xn for LTM preparation

- Issue 3: Request for CSI-RS resource configurations for early CSI acquisition

- Issue 4: UE-based TA measurement

- Issue 5: L2 reset

# 3	Discussion

## 3.1 Essential Corrections

This section lists CRs and contributions addressing essential corrections and clarifications related to Rel-19 LTM.

### Stage-3 Changes (Xn/F1)

This section lists CRs and contributions addressing essential corrections and clarifications related to Rel-19 LTM.

| [4] | R3-256705 | LTM CFRA Resource Information transfer (ZTE Corporation) | CR1531r, TS 38.423 v19.0.0, Rel-19, Cat. F |
| --- | --- | --- | --- |
| [5] | R3-256706 | LTM UE Association Information Transfer (ZTE Corporation) | CR1532r, TS 38.423 v19.0.0, Rel-19, Cat. F |
| [10] | R3-256871 | Essential correction for inter-CU LTM on F1AP (CATT,Huawei, China Telecom) | CR1605r, TS 38.473 v19.0.0, Rel-19, Cat. F |
| [11] | R3-256873 | Essential correction on inter-CU LTM in DC (CATT,Huawei, China Telecom) | CR1549r, TS 38.423 v19.0.0, Rel-19, Cat. F |
| [24] | R3-257125 | Correction on LTM Cell Switch Notification (LG Electronics Inc.) | CR1582r, TS 38.423 v19.0.0, Rel-19, Cat. F |
| [25] | R3-257126 | Correction related to LTM Security Information (LG Electronics Inc.) | CR1632r, TS 38.473 v19.0.0, Rel-19, Cat. F |
| [45] | R3-257175 | Corrections on Inter-CU LTM (Ericsson, Lenovo, LG Electronics) | CR1597r, TS 38.423 v19.0.0, Rel-19, Cat. F |

One proposal overlaps between the above CRs, as shown below.

### LTM Candidate cells to be cancelled list

[13] proposed introducing for NR-DC a new list for LTM Candidate PSCell to be cancelled.

[45] proposed multiple clean-up changes, including to introduce a new LTM Candidate Cells To Be Cancelled list covering both standalone and NR-DC cases.

In summary, there is some common understanding among several companies on introducing a new list, with a suggested approach provided below.

#### 9.2.3.xxx	LTM Cells To Be Cancelled List

This IE indicates a list of LTM cells to be cancelled.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| LTM Cells To Be Cancelled Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | reject |
| >LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

Q1a: Is it agreeable to introduce a new LTM Cells To Be Cancelled List to be used for both standalone and NR-DC cases?

| Company name | Comment |
| --- | --- |
| E/// | Agree. From specification point of view, it would be a clean way to use a separate list for to be cancelled LTM cells. |
| CATT | No strong view, prefer to minimize the specific impact. |
| LGE | Support to introduce a dedicated list for the cancelled cell list, instead of re-using the famous 9.2.3.242 LTM Candidate PSCell Request List. |
| Qualcomm | Yes. |
| Huawei | Functional is ok. For stage 3, similar view as CATT. |
| Samsung | Yes. It is simple and clear to introduce a new LTM Cells To Be Cancelled List IE. |
| ZTE | Agree |
| Google | OK |
| Ofinno | Yes, agree. For standalone, it can be included in e.g., LTM Configuration Update message. |

Summary:

All companies agree to introduce a new dedicated IE for the LTM Cells To Be Cancelled List. This will cover both standalone and NR-DC cases.

Proposal:

Reflect the above in the stage-3 CR as listed below.

Q1: Can all the changes in the above CRs to be agreed?

| Company name | Comment |
| --- | --- |
| E/// | In principle fine with the proposed changes. Two comments are given. - In [10], prefer not to add description for Request for CSI-RS Resource Configuration for CSI Acquisition IE, since there is one open point to discuss whether to remove this request IE from both Xn and F1. - In [11], if the optionality of the IEs would be changed, the order should be updated, i.e., mandatory IEs on the top. |
| CATT | [4] CFRA in LTM configuration update request is use for collected information synchronization, so it should keep. Whether need add CFRA resource in the LTM configuration update ack message is depending on LTM configuration modification is supported or not. [5] No strong view. [10][11][24][25]: ok. |
| LGE | [4] R3-256705, I am sorry but there seems a misunderstanding as the LTM CFRA Resource Information IE in LTM Config Update message is to distribute the CFRA resources prepared by candidate cells during the first round of HO preparations. Also, no need to add the LTM CFRA Resource Information IE in LTM Config Update Ack message, as we don’t support the modification of CFRA resources. [5] R3-256706, sorry but we have different views. First, we think there is no need to fix by moving the UE CTXT identifiers into the node-level IE. After an Inter-CU LTM, the new serving CU has to trigger LTM Config Update to all the other candidate CU(s) per cell basis to distribute newly derived keys. The UE CTXT identifier can be included only once for a specific candidate CU. Also, delivering the UE CTXT identifier pre cell basis to the target CU via CELL SWITCH NOTIFICATION works fine. Second, we think there is no need to add “Last Target NG-RAN Node ID” for the UE CTXT identifier. From a candidate CU point of view who received LTM Config Update message(s) from the new serving CU (after Inter-CU LTM), from our understanding, identifying the UE context can always be based on the UE association with the pervious serving CU. [11] R3-256873, we have comments for the proposed changes (2), (3), (6): - (2): From our understanding, there could be some candidate PSCells lying in S-SN, for which SN CHG REQD > 9.2.3.223 can provide their CSI resource config. If removed, we are not sure how MN can provide CSI resource config to T-SN via SN ADD REQ > LTM Candidate PSCell Addition Information Request > 9.2.3.223 - (3): RAN3 had the agreement that the MN sends the R19 set ID for each candidate PSCell to the source SN and other candidate SNs via the SN Modification Request message. - (6): The cover page should be for SN MOD CNFM (not SN CHG REQD), but RAN3 had agreed to support partial cancellations for DC case: The MN may inform the source SN about the cancellation of the prepared candidate PSCells at a candidate SN using SN modification request message. We think the IE is still needed for this case. |
| Huawei | [4], same view as CATT and LGE. [5] support to remove the Last Target NG-RAN node UE XnAP ID per candidate cell, but with different reason as justified in Proposal 3 in [29] [30]. For the first change, the reason should be as justified in Proposal 4 in [29]. |
|  | [10], [11], [24], [25] are ok. [45] should be discussed one by one. It is not as easy as jut removing the magic words from the spec… |
| ZTE | In [10], the 1st change (Add id-LTMgNB-ID in ASN.1 part of UE CONTEXT SETUP REQUEST) is not correct and not needed. According to the IE structure, the LTMgNB-ID should not be under “EarlySyncInformation-Request-ExtIEs”, it is already included in “LTMgNB-DU-IDs-Item-ExtIEs”. LTMgNB-DU-IDs-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= { { ID id-LTMgNB-ID CRITICALITY reject EXTENSION GlobalGNB-ID PRESENCE optional }, ... } In [11], not understand the 6th change (LTM modification in DC is not support, LTM Configuration ID Mapping List IE is not need contain in S-NODE CHANGE REQUIRED CONFIRM message. - should be SN modification confirm message). The MN node may provide the updated information for SCG LTM in the LTM Candidate PSCell Information Update Confirm IE of the S-NODE MODIFICATION CONFIRM message, so the ID mapping list is needed? |
| Google | R3-256705: Agree with LGE R3-256706: OK with the intention but need further check R3-256871: Generally OK R3-256873: Agree with the points on proposals 2, 3, 6 made by LGE R3-257125: Generally OK R3-257126: Generally OK R3-257175: Generally OK |
| ZTE2 | [5] R3-256706, for the first change, to remove the UE XnAP ID in per cell while add UE XnAP ID in node level (the node ID is not needed, the detailed change can be further check), agree with HW for the reasons of P3 and P4 in [29]. In addition, in Cell Switch Notification, to inform the LTM UE association, UE XnAP ID in node level is more clear and better, to avoid any repetition. |

Summary:

[4] – There are still different views on whether CFRA resources should be included in the LTM Configuration Update Acknowledge message. This issue is related to the discussion on modification of LTM configurations. It is proposed to revise the CR and join the discussion under option issue (ii).

[5] – Some companies support the intention, though concerns remain regarding the change of UE context identifiers to the node-level IE. It is proposed to update the CR accordingly and circulate it to the group for further review.

[10][11] – Additional clarifications are needed based on the received comments. The majority of companies indicated that the changes are generally acceptable, but some of the changes need to be taken out. It is proposed to revise the CRs and provide answers to the questions.

[24][25] – No comments were received. It is proposed to agree with the CRs and proceed with co-signing if companies are supportive.

[45] – This CR already covers the LTM Cells To Be Cancelled List. It is proposed to update and circulate the revised version to check whether the removal of all “to-be-refined” is acceptable to the group.

Proposal:

Authors to revise and upload the drafts for review and co-signing.

### Stage-2 Updates

| [7] | R3-256831 | [Draft TP for TS 38.473, TS 38.401] Remaining issues in Intra-CU Conditional LTM (Qualcomm Incorporated) | discussion |
| --- | --- | --- | --- |
| [14] | R3-257033 | Correction on SN initiated inter-SN SCG LTM procedure (Ofinno, LLC) | draftCR |
| [22] | R3-257050 | CSI-RS Coordination for inter-gNB LTM (Ericsson) | draftCR |
| [23] | R3-257058 | Corrections for LTM in NR-DC (Ericsson) | draftCR |
| [35] | R3-257152 | Correction on inter-CU LTM and conditional intra-CU LTM (Huawei, Jio Platforms, CATT, China Telecom, Lenovo) | CR0502r, TS 38.401 v19.0.0, Rel-19, Cat. F |
| [36] | R3-257153 | Correction on inter-CU LTM (Huawei, Jio Platforms, CATT, China Telecom, Lenovo) | draftCR |

The papers propose adding further clarifications and completing missing parts in the descriptions. Companies are invited to review and provide comments if any.

Q2: Are these stage-2 CRs acceptable for agreement?

| Company name | Comment |
| --- | --- |
| E/// | - In [7], Note 3 can be merged to normative text. - In [14], the last sentence in the Note seems not necessary; for the step 23 – 27, the MN should not mandated to trigger release procedure to release the SN. Suggest changing to “may”. - In [22], ok to add optional steps for CSI-RS coordination procedure - In [23], ok to add descriptions to transfer TCI states and R19 Set ID. - In [35], the new Note 3 and description on request for CSI-RS resource configuration for early CSI acquisition are related to the open point vi, thus such changes can be discussed until there is conclusion for the open issue. - In [36], editorial changes. |
| CATT | [22]:Current NOTE 3 is for the same purpose NOTE 3: The source gNB may initiate CSI-RS Coordination procedure to activate or deactivate CSI-RS resource(s) of some candidate cells(s). [7][23][14][35][36]: In principle fine with the proposed changes. |
| LGE | [7] R3-256831, for P1/P2, the RAN2 RRC EarlyUL-SyncConfig contains TAT value (two TATs in case of MIMO with 2TAs). Thus, removed from RAN3 F1AP BL CR with the following agreement: To remove the TAT value IE in the UE CONTEXT SETUP RESPONSE message in the BLCR. For P3/P4, stage-2 changes seem OK though. |
| Qualcomm | 256831: As proponents, agree with the clarifications. The first one incorporates the fact that the gNB-CU generates the L3 execution conditions for CLTM; this fact is not mentioned anywhere in the signalling flow description and seems important to capture. The second one states that the gNB-CU can trigger towards the source gNB-DU an LTM cell switch based on an L3 measurement report, when the UE is configured with Intra-CU CLTM. 257033: Agree with the clarifications. 257050: Agree with the changes. 257058: Agree with the changes. 257152: Agree with the changes, as they are all valid. 257153: Agree with the changes except for Step 6 of the signalling flow, in which it is not clear why the LTM configuration ID mapping list has been removed. |
| Huawei | [7] stage 3 is not needed as LGE pointed out. Stage 2 is ok. [22] similar comment as CATT. The principles followed in the past is to simply the stage 2 message flow, that’s why the note is there. In general, we are fine with either way. Just to say we should stick to original agreements, principles, rules as much as we can to avoid making changes back and forth. [23] editorials. [35],[36] are ok. |
| Google | No strong view on clarifying stage 2 texts with the above contributions. |
| Ofinno | [14] 257033, as proponent, agree with the changes. I may reword some texts as you suggested. [22] R3-257050 agree with the changes [35][36] agree with changes |

Summary:

Since the stage-2 changes are relatively straightforward to manage, it is suggested to align and consolidate the agreements within each specification.

- TS 37.340: merge the agreed changes from [14] and [23] (Ofinno)

- TS 38.401: merge the agreed changes from [7] and [35] (Huawei)

- TS 38.300: merge the agreed changes from [22] and [36] (Ericsson)

Proposal:

Companies to update and upload revised drafts for review.

## Open issues for discussion

This section lists topics where additional discussion or consensus is required.

### Modification of LTM configurations

The following open points are identified for further discussion on support for modification of LTM configurations.

Whether the modification of LTM configurations is allowed over network signalings?

If yes, which configuration needs to be modified?

When the LTM modification can be triggered? For example, during or after LTM preparation? Is it possible to modify during LTM execution or in subsequent LTM?

If there is any configuration that can be modified, which way is preferred?

a. Existing LTM Cancel procedure, and the source re-initiate the configuration procedure

b. New procedure

Q3: Companies are invited to provide comment/preference if any.

| Company name | Comment |
| --- | --- |
| E/// | To allow quick modification of LTM configurations and reconfigure the UE, a new procedure would be more efficient. |
| CATT | a. Existing LTM Cancel procedure, and the source re-initiate the configuration procedure |
| LGE | We are fine to have a new procedure for modification, but we would like to limit the modification use case for LTM, only for the updates of CSI-RS resource configurations. |
| Qualcomm | We think that signalling for the modification of an LTM configuration should be supported since it seems to be needed. LTM modification can be source gNB/gNB-DU-initiated or candidate gNB/gNB-DU-initiated, since a candidate cell can belong to a source gNB-DU or a candidate gNB-DU. There are two scenarios: - Modification during or after LTM preparation, and before the first LTM cell switch - Modification during subsequent LTM operation For both the scenarios, we think that the following candidate cell configurations may need modification: - CSI-RS resource configurations for L1 measurements and CSI acquisition, e.g., changes in dedicated CSI-RS resources, and consequently, CSI report configuration - Early UL Sync Configuration, LTM CFRA Resource Configuration, since the CFRA resource availability and associated RACH configurations may change Regarding the question “If there is any configuration that can be modified, which way is preferred?”, our preference is “b”, new procedure. Option “a” can involve significant signalling overhead as it requires cancellation of the preparation first, and then the configuration. LTM modification should not result in any ASN.1 changes to the existing RRC reconfiguration signalling, as specified in TS 38.331 for LTM, i.e., the existing RRC reconfiguration should already be supporting LTM modification. |
| Huawei | For sure, this is a kind of functional optimisation. Open to discuss, but would like to see a overall analysis first before making the decision. Like scenarios, spec impacts, alternative solutions, pros and cons, etc. |
| Samsung | We are basically okay to have modification procedure, checked internally it is assumed to be supported from other WGs. For the procedure, no strong view. Both E///’s and CATT’s suggestions are fine. |
| ZTE | Considering the complexity/significant spec impacts and R19 is to be freezing, prefer to discuss the support of LTM modification in Rel-20. Option (a) by reusing existing procedures is preferred to minimize spec impacts. |
| Google | Yes. Although follow-up modification should be generally avoided, it should be still allowed. |
| Ofinno | Allowing the modification of LTM configurations is beneficial since lack of resources always happen. Prefer to cover both: - CSI-RS resource configurations - Early UL Sync Configuration and LTM CFRA Resource Configuration |

Summary:

Majority companies agree to support the modification of LTM configurations in Rel-19. Some company mentioned that no RRC signaling change would be needed. Several companies commented that the modification should cover CSI-RS resource configurations, Early UL Sync Configuration, and LTM CFRA Resource Configuration. Regarding the solution, a new procedure is preferred by some components.

Proposal:

This topic needs to be addressed. Companies to bring contributions and continue discussing the support for modification of LTM configurations and the items to be included.

### TA timer handling

[1] The source gNB(-DU) signals to target gNB(-DU) the RSRP measured during TA Acquisition along with the TA value within the F1AP DU-CU CELL SWITCH NOTIFICATION and XnAP CELL SWITCH NOTIFICATION. The target gNB-CU forwards the RSRP within the F1AP CU-DU CELL SWITCH NOTIFICATION message.

[6] The source gNB-DU provides the remaining TA validity time along with the TA values, to a candidate gNB-DU using the Cell Switch Notification messages over F1-AP and Xn-AP.

Q4: Is additional TA validity information needed from the source to the target node?

| Company name | Comment |
| --- | --- |
| E/// | Fine to provide validity info |
| CATT | No need, follow Rel-18, new source can guarantee TA validity by implementation. |
| LGE | We are OK for [6] R3-256830 P7, rather than providing the RSRP measurements as proposed in [1] R3-256530 P5 (but no strong view). |
| Qualcomm | Yes. For Inter-CU LTM, we think providing information as in our proposal in [6] involving providing remaining TA validity times, enables the candidate gNB-DU to track the validity of the TA values when the UE is in the target cell. Otherwise, the candidate gNB-DU has no clue – just providing TA values does not help the candidate gNB-DU to determine how long the TA value is valid. This aspect was agreed for Intra-CU CLTM, and it is important for Inter-CU LTM as well. We do not support the solution in [1] as it is not clear how it works and may require other working groups, e.g., RAN2, RAN1, to check whether it is a valid proposal. |
| Huawei | No strong view. Maybe the TAT remaining time is more straightforward. |
| Samsung | Not strong need for proposal in [1]; Slightly prefer the proposal in [6] due to having some benefits since target DU can determine when to trigger early TA acquisition. |
| ZTE | No need, follow Rel-18, new source can guarantee TA validity by implementation. |
| Google | Prefer following the Rel-18 principle. |

Q5: If the answer to Q4 is YES, what information is preferred?

| Company name | Comment |
| --- | --- |
| E/// | Transferring the remaining TA validity time would be a straight forward way. |
| LGE | See the above comment. |
| Qualcomm | The additional information preferred is the remaining TA validity times associated with the TA values as in [6]. |
| Huawei | See above. |
| Samsung | Can define a new IE, like “TA validity time” associated with each TA value |

Summary:

3 companies supported introducing the remaining TA validity time. 2 companies had no strong view but indicated they were also fine with the proposal. 3 companies preferred to follow the Rel-18 approach, i.e., up to implementation.

Proposal:

No consensus is reached. Companies to gather support and bring contributions to the meeting for further discussion if needed.

[7] For CLTM, A candidate gNB-DU signals the TA timer value of its candidate cell to the gNB-CU. Two options are given.

- Option 1: The UE Context Setup Response message is used.

- Option 2: The TA Information Transfer procedure is used so that a candidate gNB-DU provides the TA timer value to the source gNB-DU, which then uses the UE Context Modification Response message to signal the TA timer value to the gNB-CU.

Q6: Can it be agreed to transfer the TA timer value from the candidate DU to the candidate CU for CLTM?

| Company name | Comment |
| --- | --- |
| E/// | Option 1 is preferred. |
| CATT | TAT is already include in EarlyUL-SyncConfig, not need to transfer it explicitly. |
| LGE | Agree with CATT |
| Qualcomm | Yes, it seems to be needed, since the gNB-CU provides the TA timer value to the UE through RRC signalling. We prefer Option 1 among the two options listed – the TA timer value can be provided during the LTM preparation procedure. |
| Huawei | Same view as CATT and LGE. |
| Samsung | Agree to CATT |
| ZTE | Agree with CATT |
| Google | Agree with CATT |
| Ofinno | Both are fine, slightly prefers option 1. |

Summary:

The majority view is that no change is needed, as the TAT is already included in the Early UL Sync Configuration.

Proposal:

No consensus is reached on the need.

[8] RAN3 to discuss the issue on how to ensure that TA Information is correctly transferred from the candidate SN-DU to source SN-DU in the scenario of inter-CU SCG LTM. It is proposed to transfer new IEs as below for the issue on how to ensure that TA Information is correctly transferred from the candidate SN-DU to the source SN-DU in inter-CU SCG LTM.

- Introduce MN ID in Early Sync Information Request IE in F1AP UE Context Setup/Modification Request message

- Introduce MN ID in the F1AP DU-CU TA Information Transfer message from candidate SN-DU to candidate SN-CU to inform candidate SN-CU to which MN it should forward this TA information.

- Introduce Source gNB ID (indicating source SN) in the XnAP TA Information Transfer message from candidate SN-CU to MN to inform MN to which SN (source SN-CU) TA information should be forwarded.

- Add LTM gNB ID IE in the Early UL Sync Configuration IE.

Q7: Is the above proposal to transfer TA related information from the candidate SN-DU to the source SN-DU agreeable?

| Company name | Comment |
| --- | --- |
| E/// | Thinking of another way to solve this issue, i.e., introduce UE F1AP/XnAP IDs to the TA Info Transfer procedures. |
| CATT | We acknowledge related information for early RACH request is missing in DC case, open to find a solution with less spec. impact. |
| LGE | We acknowledge the issue but we think this can be solved by the smart MN implementation. The MN can override its own gNB ID (instead of S-SN ID) when relaying early RACH requests. For Inter-SN LTM, MN can do some internal mapping to let C-SN deliver the calculated TA value to itself (i.e. MN) as a final destination (not S-SN), then MN can forward to S-SN who triggered early synchronization accordingly. |
| Qualcomm | Yes. |
| Huawei | Issue acknowledged. Can further discuss the details. |
| Samsung | In my understanding, the solution from E/// seems not work since candidate DU cannot know which UE based on RACH resource sent by UE and does not know how to decides the UE F1AP IDs. More clarification may be needed for more well understanding regarding to the solution from LGE. How to work based on MN implementation. The last two modifications are for the issue which are existed in the current specification even if do not introduce MN ID in Early Sync Information Request IE in F1AP UE Context Setup/Modification Request message and F1AP DU-CU TA Information Transfer message. - Introduce Source gNB ID (indicating source SN) in the XnAP TA Information Transfer message from candidate SN-CU to MN to inform MN to which SN (source SN-CU) TA information should be forwarded. (IE missed because Early Sync Information Request IE already includes Global gNB ID IE) - Add LTM gNB ID IE in the Early UL Sync Configuration IE.(IE missed because Early Sync Information Request IE already includes this IE) |
| ZTE | We think it could be gNB implementation. E.g., as shown in the figure in [8], in step 11 the candidate SN-CU is able to correctly forward the TA info associated with <SN1 ID, requester ID1/gNB-DU1> to the right MN1 by recording the received <SN1 ID, requester ID1/gNB-DU1> from MN1 in step 2. Similarly, in step 12 the MN1 is able to correctly forward the TA info associated with <SN1 ID, requester ID1/gNB-DU1> to the right SN by recording the <SN1 ID, requester ID1/gNB-DU1> is received from which SN in step 1. For the LTM gNB ID in Early UL sync Configuration IE, we are wondering Whether the same gNB-DU ID can be used in different gNB ID? If yes, LTM gNB ID is needed, otherwise, it is not needed. |
| Google | OK with the intention but need further check |

Summary:

The issue is widely acknowledged. Companies would need more discussion on the possible solutions.

Proposal:

No conclusion was reached on the solution. The issue remains to be resolved, and companies are invited to bring contributions for further discussion at the next meeting.

### Early RACH Resource Requester ID

[6] For obtaining the Early TA RACH configuration for subsequent LTM, each candidate gNB-CU provides the pair (candidate gNB ID, List of RACH resource request IDs) to the source gNB-CU in Handover Request Ack.

[8] discuss how source gNB-CU acquires the gNB-DU ID (Early RACH Resources Requester ID) information under the candidate gNB-CU before handover request is initiated. It is proposed to introduce the Early RACH Resources Requester ID IE in Served Cell Information NR IE for serving cells in case of split gNB in XnAP Xn Setup/NG-RAN node Configuration Update procedure, so as to let the source gNB-CU acquires the Early RACH Resources Requester ID information belongs to the candidate gNB-CU.

[31] Exchange the early RACH Resources Requester IDs in the Served Cell Information IE between neighbouring gNBs during Xn Setup and NG-RAN node configuration update for support of inter-CU LTM.

Q8: Is it necessary to exchange the Early RACH Resource Requester IDs over the Xn interface?

| Company name | Comment |
| --- | --- |
| E/// | Yes |
| CATT | Yes |
| LGE | We acknowledge that, in order for the early RACH Resources Requester ID mechanism to work as intended (shared preamble allocation on a per-DU basis), S-gNB indeed needs to know in advance whether each (chosen) candidate cell belongs to the same or different DUs. But it seems still feasible for S-gNB to simply assign distinct early IDs for each candidate cell (i.e. treating every cell as if belonging to different DUs). While this means that each requested candidate cell may split preamble resources for all the other candidate cells, the cells that actually share the same DU will still receive the assigned resources for them. Though delivered individually, but under the same DU so still can be aggregated for using those assigned resources (effectively per DU basis). |
| Qualcomm | Yes, it is necessary for Early TA RACH configuration for subsequent LTM. |
| Huawei | Yes |
| Samsung | Yes Regarding the solution from LGE, we want to say RAN3 has agreed that the early RACH Resources Requester ID is an implicit gNB-DU ID, and the format of this ID is same with the gNB-DU ID. Therefore, we should follow this agreement and mechanism. And furthermore, XnAP TA Information Transfer procedure also use this ID to indicate source gNB-CU to forward the TA value to corresponding gNB-DU indicated by this ID. Therefore, the solution from LGE may be introduced other potential issue, e.g, TA information transfer procedure is failure to arrive the correct source gNB-DU. |
| ZTE | Yes |
| Google | Yes |

Q9: If the answer to Q7 is YES, which IE and message should be used?

| Company name | Comment |
| --- | --- |
| E/// | The IDs can be transferred in the Served Cell Information before handover preparation. |
| CATT | Exchange the early RACH Resources Requester IDs in the Served Cell Information IE between neighbouring gNBs during Xn Setup and NG-RAN node configuration update for support of inter-CU LTM. |
| Qualcomm | We do understand that our proposed solution [6] involves an extra step after the LTM configuration update procedure. The extra step could be quite simple and again invoke the LTM configuration update procedure. We think that this is better than the solutions proposed in [8] and [31]. The drawback with the solutions in [8] and [31] is that new LTM procedure specific information is introduced in the Xn interface setup messages which seems a bit strange, e.g., if the source gNB-CU decides not to use LTM for handover purposes, then this information is not used at all. |
| Huawei | Same view as E/// and CATT. |
| Samsung | Agree to E///, CATT and Huawei. R19 LTM procedure is an important feature on UE mobility, it is also acceptable to introduce the Xn interface enhancement to support this feature. Moreover, consider the same issue is existed in the inter-CU SCG LTM scenario, Xn enhancement solution is also applicable. |
| ZTE | We think that transferring the requester IDs in LTM preparation phase is better, i.e. the solution in [6]. In the HO preparation phase, the impact of this delay is negligible. Even if the source obtains the early sync resource in the first phase, it will not send it to the UE immediately; instead, it needs to wait until all candidate cells have completed negotiations before generating the final configuration and sending it to the UE. There are drawbacks for for solution in Xn setup. Firstly, the requester ID is unnecessary for gNBs that do not perform LTM and are unwilling to implement early sync. More importantly, we want to remind that the purpose of introducing this requester ID is to avoid exposing the internal structure of CU-DU to other gNBs. If the requester ID is informed to neighboring gNBs during the Xn setup, it violates this intention. |
| Google | Use the Xn Setup or NG-RAN node configuration update seems feasible. But open for other solutions. |

Summary:

It is well acknowledged that the Early RACH Resource Requester ID needs to be transferred over Xn before handover preparation.

Two options are available:

- Option 1: The IDs are transferred during Xn Setup and NG-RAN Node Configuration Update procedures.

- Option 2: The IDs are transferred during LTM preparation phase.

Proposal:

Companies continue discussing the solution between these two options.

### PRACH Resources Release

[16][17][18][19] resubmit proposals concerning the possibility for the candidate node to release PRACH resources and inform the source node accordingly. Given the limited availability of preamble resources, it is necessary to define a mechanism that allows dynamic release of resources under the candidate node’s control.

Q10: Does company agree that a mechanism for releasing PRACH resources is needed?

| Company name | Comment |
| --- | --- |
| E/// | Yes, still see the necessity due to the limited preamble indexes per cell in the candidate gNB. |
| CATT | Open to discuss, considering whether release and re-initiate also workable. |
| LGE | No strong view but doesn’t seem essential.. But if operators strongly request, then open to discuss. |
| Qualcomm | Yes, the mechanism for releasing PRACH resources, as given in proposals 1 and 2 in [15], seems useful. But, if LTM modification is agreed to be supported, this change can be included in that discussion. |
| Huawei | Similar view as LGE. |
| Samsung | Agree with E///. |
| ZTE | We think this is an optimization. The candidate can cancel the LTM if necessary, and after that the source can further initiate LTM towards the candidate if needed. |
| Google | Open for discussion if LTM modification is to be supported. |
| Ofinno | Yes, it’s beneficial to support releasing PRACH resource. |

Q11: If the answer to Q9 is YES, should a new procedure be introduced, or can an existing procedure be reused?

| Company name | Comment |
| --- | --- |
| E/// | A new procedure being initiated by the candidate is preferred since it is the candidate node who controls the resources. |
| Qualcomm | We prefer that the LTM modification procedure, as discussed in Section 2.2.(ii), is used. |
| Samsung | Prefer new procedure |
| Ofinno | Either is fine |

Summary:

4 companies support introducing a mechanism for PRACH resource release. 4 companies are open to continue the discussion. This topic is related to the modification of LTM configurations.

Proposal:

Companies bring contributions to continue the discussion on this topic and to propose whether a new procedure should be introduced or combined with the potential LTM configuration modification procedure.

### CSI-RS Related Topics

[42] proposed to remove the Request for CSI-RS Resource Configuration for Early CSI Acquisition IE for both Xn and F1.

Q12: Whether to remove the Request for CSI-RS Resource Configuration for Early CSI Acquisition IE?

| Company name | Comment |
| --- | --- |
| E/// | Yes, the source node has no knowledge when it should initiate the request for early CSI acquisition. |
| CATT | Prefer to keep it, otherwise it hard to limited when will the candidate cell provide CSI-RS Resource Configuration for Early CSI Acquisition. Candidate cell provide CSI-RS at unsuitable time may lead to 1) unnecessary RRCreconfigurtaion 2) addition procedure for providing CSI-RS report configuration. |
| LGE | Agree with Ericsson. Whether to configure CSI-RS resource config for CSI acquisition should be determined based on UE capability by a candidate cell. |
| Qualcomm | We agree that the Request for CSI-RS Resource Configuration for Early CSI Acquisition IE can be removed. It is up to the candidate gNB-DU to provide the CSI-RS resources for Early CSI acquisition when it receives a request for CSI-RS resource configuration for a candidate cell. |
| Huawei | This was discussed in the WI phase. Better to stick to the previous decision unless any problem is identified. The source may also decide based on UE capability. And the source even has more real time UE Qos information for ongoing services. |
| Samsung | Prefer to keep the current design unless some critical issues are identified. |
| ZTE | Agree to remove. It is up to candidate whether to provide the CSI-RS resource config for early CSI acquisition. The explicit request indicator is not needed. |
| Google | Open for discussion. If the source is indeed lack of the knowledge of such UE and candidate side capability, it can be removed. |

Summary:

5 companies agree to remove the Request for CSI-RS Resource Configuration for Early CSI Acquisition IE. 3 companies prefer to keep as it is.

Proposal:

The issue needs to be further discussed.

[26] While for CSI acquisition, there are two options for the source gNB to request the candidate gNB to provide the CSI report configuration for early CSI Acquisition.

- Option 1: relying on the current CSI resource configuration IE in the LTM CONFIGURATION UPDATE message

- Option 2: introducing a new indicator

It is proposed that in the LTM CONFIGURATION UPDATE message, the source gNB provides the per candidate cell LTM-CSI-ResourceConfigId to the candidate gNB as the request indicator for CSI report configuration for CSI acquisition, also for CSI-IM measurements.

Q13: Is it necessary to introduce a new indicator from the source gNB to the candidate gNB for CSI report configuration?

| Company name | Comment |
| --- | --- |
| E/// | Option 1 |
| CATT | Option 1 is workable, candidate cell can understand based on previous request. |
| LGE | Option 1. Though we understand the intention that an explicit request from S-gNB (or C-CU) can separate it from preparing subsequent LTM, we think that it should be the C-gNB (and C-DU) who makes decision whether to configure CSI-RS “report” config for CSI acquisition (based on the UE capability and common CSI resource configuration provided), given that this is measured before cell switch, and the UE reports to the target cell (not the source cell) after cell switch commanded. |
| Qualcomm | We agree with [26] that a new indicator is not needed and that the existing IEs can be used. |
| Huawei | Disagree above. Please double check the problem identified in [26] for option 1. |
| Samsung | Slightly prefer Option 2 since it provides enough flexibility without the restriction. |
| ZTE | Option 1 |
| Google | Open for discussion. |

Summary:

The majority of companies preferred that no change is required.

Proposal:

Noted.

### R19 Set ID

[33] clarify the ambiguities on the Rel-19 set ID list implementation in either stage 2 or stage 3 specification to avoid any IOT issue.

Q14: Do we need to clarify the implementation of R19 Set ID in both stage-2 and stage-3?

| Company name | Comment |
| --- | --- |
| E/// | Stage-2 would be good enough. |
| CATT | Better to have such clarification. |
| LGE | P1 of stage-2 clarification seems enough (we can work out detailed stage-2 wording during the meeting). |
| Qualcomm | No. We don’t think that the Stage-2 clarifications described in [33], are needed. The solution agreed in the last RAN3 meeting is enough, i.e., the source gNB assigns a Rel-19 Set ID list for each candidate gNB and sends it in HANDOVER REQUEST, and then the candidate gNB assigns the Rel-19 Set IDs for its own candidate cells and provides feedback to the source gNB. |
| Huawei | Follow majority. |
| Samsung | Stage-2 seems to be enough. |
| ZTE | Disagree to change the IE to mandatory. Can further check whether some clarification is needed and where to capture it. |
| Google | Clarifications seems needed. Open for both stage 3 and stage 2. |

Summary:

No consensus is made. Clarification in stage-2 is slightly preferred.

Proposal:

Contribution-driven if needed

## New topics

The topics in this section are raised for the first time and intended for initial discussion. Companies are invited to go through the proposals and share any comments or observations for further work and potential follow-up contributions.

### Halt/resume subsequent intra-CU LTM

[1] Introduce over F1AP signalling from new Source gNB-CU to new Source gNB-DU to halt and resume further subsequent Intra-CU LTM mobility commands toward the UE.

### Introduce cell switch type

[1] Add a “Cell Switch Type (ENUMERATED(RACH-less, RACH-based))” in F1AP and XnAP CELL SWITCH NOTIFICATION messages.

### Fast LTM recovery

[20][21] Since the source gNB needs to know whether all the candidate gNBs agreed to configure Fast LTM Recovery before configuring the UE. Therefore, coordination over Xn is required.

### CLTM

[2] gNB-CU signals to gNB-DU a list of L3 C-LTM Candidate Cells in UE CONTEXT MODIFICATION REQUEST and UE CONTEXT SETUP REQUEST messages.

### Shared CFRA resources

[3] Preamble index and RA-RNTI should be transmitted from source DU to the target DU after source DU sends an LTM cell switch command MAC CE to UE with CFRA resource included. RNA3 to discuss whether to reuse existing Xn and F1 interface messages or introduce new interface messages to support this.

### UE based TA measurement ID

[8] For UE Based TA Measurement ID assignment for source cell and each candidate cell(s), reuse the same solution on Rel-19 set IDs assignment among CUs.

### L2 Reset

[40] RAN3 should discuss and decide on the specific configuration IEs to be included within XnAP IEs or the inter-node CG-Config message. In inter-CU MCG LTM, the candidate gNB shall provide a LTM No Reset ID associated with the candidate cell if being requested by the source gNB. In inter-CU SCG LTM, the candidate SN shall provide a LTM No Reset ID associated with each prepared candidate PSCell if being requested by the MN.

Q15: Any comment to the above topics?

| Company name | Comment |
| --- | --- |
| E/// | Fast LTM recovery and L2 reset can be discussed. |
| CATT | [1] In a hybrid scenario, the gNB shall know it need trigger Rel-19 MAC CE even for intra-DU LTM. Same issue with LG CR [25] R3-257126. [20][21]: source gNB can decided whether UE can execute fast recovery at target side, which is similar way for CHO. Is there any different behaviour form target presepctive for support or not support fast recovery?. [2] In C-LTM discussion, for one configuration, whether RAN3 support some of the candidate cell only support network based LTM? Open to discuss. [3] CFRA resource is not shared based on our understanding. Only the RACH resource for early sync is shared. [8] Whether support UE based TA is depend on the time difference between 2 cells. Source is able to configured UE based TA as it know the timing information of the candidate cells, which is exchanged in served UE information. [40] Support with comments. 1) No need to add the LTM Reset Information IE in CELL SWITCH NOTIFICATION, other parts are aggregable for us. 2) Should this information be provided based on the request or directly from the candidate cell? |
| LGE | [1] R3-256530 P4: We think indicating “RACH-less” to the target cell is indeed necessary. In LTM, RACH vs RACH-less is determined based on TAC included in CSC MAC CE. If RACH-less, two options: CG vs DG, depending on how it was pre-configured. If C-DU knows “RACH-less”, (CG) it can start activating CG; (DG) it can start sending PDCCH using the indicated TCI state. [20][21] R3-257047-48: We think that fast LTM Recovery is a UE capability that doesn’t require preferences from CUs.. [2] R3-256531: We acknowledge that there is indeed an issue because S-DU cannot distinguish whether the received TA value for a candidate cell needs to be immediately delivered to the UE (because of C-LTM) or can be delivered together with CSC MAC CE (non C-LTM). [8] R3-256857: We agree with the spirit of re-using the Rel-19 Set ID solutions across CUs, but would like to discuss first whether the UE-based TA measurement can be applicable for cells belonging to different CUs. [40] R3-257157: For P1/P2, we are OK with the spirit to check across XnAP IEs vs inter-node RRC message details. For P3-P5, we are open to discuss L2 No Reset handling across SNs, but prefer to follow the Rel-19 Set ID solution. |
| Samsung | - Halt/resume subsequent intra-CU LTM Seem no issue since UE sent RRC Reconfiguration Complete message to target gNB-CU with the new key corresponding target cell while inter-CU LTM execution. After that, if intra-CU LTM happens, both target gNB-CU and UE can know this is an intra-CU LTM execution and decide the key change or not. - Introduce cell switch type No need, even if keeping current mechanism, seem no big issue. - Fast LTM recovery Assume fast LTM recovery is always supported inter-CU LTM and it is no critical issue. - CLTM make sense, can support this proposal. - Shared CFRA resources it is an optimization but it is too late to specify out an acceptable solution. - UE based TA measurement ID Support this proposal since this is essential issue. If not addressed, UE based TA measurement cannot work inter-CU (SCG) LTM. Furthermore, this feature should be supported inter-CU LTM due to below agreement from RAN3. But this discussion is not enough due to other issues during the WI phase. - Source gNB generate the UE Based TA Measurement Configuration, and transfer it to all candidate gNB(s) via LTM Configuration Update message. - L2 Reset Support RAN3 to discuss this issue on how to configure L2 reset ID for each candidate cell. Whether to consider the similar solution on Rel-19 set IDs assignment among CUs? |
| ZTE | For UE based TA measurement ID and L2 No Reset, prefer to follow the Rel-19 Set ID solution. While other topics are somehow optimization or not essential. |
| Google | [40] As the proponent we think the L2 Reset issue is an essential correction and should be discussed. Although following a similar solution on Rel-19 Security Change ID could work, we see the benefit of the proposed solution given that the L2 Reset is always performed for the inter-CU scenario and the issue can be resolved without restricting the source to assign different and exact number of values in advance and parallel preparation procedures won’t be blocked. |
| Ofinno | [1] R3-256530 P4: support to discuss the issue on cell switch type. Introducing a “cell switch type” in F1AP and XnAP is necessary for the completion of Rel-19 work. [3] CFRA resource can be shared based on our understanding. Prefer to discuss this issue. |

Summary:

Based on the feedback, UE-based TA measurement and L2 reset received the majority of support.

Proposal:

Companies to prioritize discussion on solutions for these two issues as the first priority among the new topics.

## 3.4 Miscellaneous

# 4 Conclusion

It is proposed to agree on the stage-2 and stage-3 CRs and to continue the discussion on the identified issues.

# 5 References

- R3-256530 Discussion on remaining LTM Issues (Nokia)

- R3-256531 Remaining Conditional LTM Issues (Nokia)

- R3-256699 Discussion on shared CFRA resource for inter-CU LTM (vivo)

- R3-256705 LTM CFRA Resource Information transfer (ZTE Corporation)	CR1531r, TS 38.423 v19.0.0, Rel-19, Cat. F

- R3-256706 LTM UE Association Information Transfer (ZTE Corporation)	CR1532r, TS 38.423 v19.0.0, Rel-19, Cat. F

- R3-256830 [Draft TP for TS 38.473, TS 38.423] Remaining issues in Inter-CU LTM (Qualcomm Incorporated)

- R3-256831 [Draft TP for TS 38.473, TS 38.401] Remaining issues in Intra-CU Conditional LTM (Qualcomm Incorporated)

- R3-256857 Essential Corrections for Inter-CU (SCG) LTM (Samsung)

- R3-256858 (CR to 38.473) Essential correction for Inter-CU SCG LTM (Samsung)	CR1602r, TS 38.473 v19.0.0, Rel-19, Cat. F

- R3-256871 Essential correction for inter-CU LTM on F1AP (CATT,Huawei, China Telecom)	CR1605r, TS 38.473 v19.0.0, Rel-19, Cat. F

- R3-256873 Essential correction on inter-CU LTM in DC (CATT,Huawei, China Telecom)	CR1549r, TS 38.423 v19.0.0, Rel-19, Cat. F

- R3-256874 Discussion on LTM candidate PSCell cancel procedure in DC (CATT,Huawei)

- R3-256875 Correction on LTM candidate PSCell cancel procedure in DC (CATT)	CR1550r, TS 38.423 v19.0.0, Rel-19, Cat. F

- R3-257033 Correction on SN initiated inter-SN SCG LTM procedure (Ofinno, LLC)	draftCR

- R3-257042 PRACH resources for RACH-less LTM (Ericsson, Jio Platforms, Lenovo, Verizon Wireless, NTT DOCOMO)

- R3-257043 PRACH resources for Inter-CU LTM (Ericsson, Jio Platforms, Lenovo, Verizon Wireless, NTT DOCOMO)	CR1574r, TS 38.423 v19.0.0, Rel-19, Cat. F

- R3-257044 PRACH resources for RACH-less LTM (Ericsson, Jio Platforms, Lenovo, Verizon Wireless, NTT DOCOMO)	CR1625r, TS 38.473 v19.0.0, Rel-19, Cat. F

- R3-257045 PRACH resource for RACH-less LTM (Ericsson, Jio Platforms, Lenovo, Verizon Wireless, NTT DOCOMO)	CR0499r, TS 38.401 v19.0.0, Rel-19, Cat. F

- R3-257046 PRACH resources for RACH-less LTM (Ericsson, Jio Platforms, Lenovo, Verizon Wireless, NTT DOCOMO)	draftCR

- R3-257047 Fast LTM recovery for Inter-CU LTM (Ericsson, Jio Platforms, Lenovo)

- R3-257048 Fast LTM Recovery for Inter-CU LTM (Ericsson, Jio Platforms, Lenovo)	CR1575r, TS 38.423 v19.0.0, Rel-19, Cat. F

- R3-257050 CSI-RS Coordination for inter-gNB LTM (Ericsson)	draftCR

- R3-257058 Corrections for LTM in NR-DC (Ericsson)	draftCR

- R3-257125 Correction on LTM Cell Switch Notification (LG Electronics Inc.)	CR1582r, TS 38.423 v19.0.0, Rel-19, Cat. F

- R3-257126 Correction related to LTM Security Information (LG Electronics Inc.)	CR1632r, TS 38.473 v19.0.0, Rel-19, Cat. F

- R3-257130 Correction on CSI Report Configuration for CSI Acquisition for inter-CU LTM (Huawei, Jio Platforms, CMCC, China Telecom, NEC, Lenovo)

- R3-257131 Correction on CSI report configuration for CSI acquisition for inter-CU LTM (Huawei, Jio Platforms, CMCC, China Telecom, Lenovo)	CR1634r, TS 38.473 v19.0.0, Rel-19, Cat. F

- R3-257132 Correction on CSI report configuration for CSI acquisition for inter-CU LTM (Huawei, Jio Platforms, CMCC, China Telecom, Lenovo)	CR1584r, TS 38.423 v19.0.0, Rel-19, Cat. F

- R3-257146 Essential corrections to XnAP for inter-CU LTM (Huawei, Jio Platforms, CMCC, Lenovo)

- R3-257147 Essential corrections to XnAP for inter-CU LTM (Huawei, Jio Platforms, CMCC, Lenovo)	CR1590r, TS 38.423 v19.0.0, Rel-19, Cat. F

- R3-257148 Exchange of early RACH Resources Requester ID for inter-CU LTM (Huawei, Jio Platforms, CATT, CMCC, China Telecom, Lenovo)

- R3-257149 Exchange of early RACH Resources Requester ID for inter-CU LTM (Huawei, Jio Platforms, CATT, CMCC, China Telecom, Lenovo)	CR1591r, TS 38.423 v19.0.0, Rel-19, Cat. F

- R3-257150 Correction on the Rel-19 set ID assignment in inter-CU LTM (Huawei, Jio Platforms, CATT, China Telecom)

- R3-257151 Correction on the Rel-19 set ID assignment in inter-CU LTM (Huawei, Jio Platforms, CATT, China Telecom)	CR1592r, TS 38.423 v19.0.0, Rel-19, Cat. F

- R3-257152 Correction on inter-CU LTM and conditional intra-CU LTM (Huawei, Jio Platforms, CATT, China Telecom, Lenovo)	CR0502r, TS 38.401 v19.0.0, Rel-19, Cat. F

- R3-257153 Correction on inter-CU LTM (Huawei, Jio Platforms, CATT, China Telecom, Lenovo)	draftCR

- R3-257154 Corrections on inter-CU MCG LTM without SN change (Huawei, Jio Platforms, Lenovo, Google)

- R3-257155 Correction on security key update for inter-CU MCG LTM (Huawei, Jio Platforms, China Telecom, Lenovo, Google)	CR1593r, TS 38.423 v19.0.0, Rel-19, Cat. F

- R3-257156 Stage 2 corrections to inter-CU MCG LTM (Huawei, Jio Platforms, Lenovo, Google)	draftCR

- R3-257157 Discussion on supporting inter-CU SCG LTM and the L2 reset function in inter-CU LTM (Google)

- R3-257158 Correction to L2 reset in inter-CU LTM (Google)	CR1594r, TS 38.423 v19.0.0, Rel-19, Cat. F

- R3-257172 Remaining issues for CSI-RS coordination (Ericsson, Jio Platforms, Verizon Wireless, LG Electronics)

- R3-257173 Corrections on CSI-RS coordination for LTM (Ericsson, Jio Platforms, Verizon Wireless, LG Electronics)	CR1596r, TS 38.423 v19.0.0, Rel-19, Cat. F

- R3-257174 Corrections on CSI-RS coordination for LTM (Ericsson, Jio Platforms, Verizon Wireless, LG Electronics)	CR1638r, TS 38.473 v19.0.0, Rel-19, Cat. F

- R3-257175 Corrections on Inter-CU LTM (Ericsson, Lenovo, LG Electronics)	CR1597r, TS 38.423 v19.0.0, Rel-19, Cat. F