3GPP TSG-RAN WG3 Meeting #130	R3-258518

# Dallas, USA, 17 – 21 November 2025

Agenda Item:	11.2.2

Source:	Ericsson

Title:	MRO for Conditional LTM

Document for:	Discussion and decision

# 1	Introduction

One of the objectives of the SON/MDT Rel-20 WID [RP-252560] is to specify MRO enhancements to the intra-CU conditional LTM cell switch procedure.

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] If needed, co-operate with RAN1, SA5.

This contribution discusses considerations regarding MRO enhancements to this feature. The main differences between LTM and conditional LTM (CLTM) are discussed, along with the various use cases of MRO enhancements for CLTM.

# 2	Discussion

## 2.1. Differences between LTM and CLTM from a network point-of-view

As opposed to the network triggered LTM, conditional LTM is UE triggered. The network configures the UE with the triggering conditions that the UE monitors and triggers CLTM by itself when the conditions are met. This is expected to make the CLTM procedure more robust while still retaining the main attraction of LTM, i.e. short interruption time. However, this also means that the network is not aware of certain details of the procedure. The detailed signalling steps of CLTM are shown in figure 1.

- In case of CLTM cell switch, the responsibility for triggering its execution is with the UE rather than the network.

In CLTM, the network configures the UE with a set of candidate cells, beams and TCI states and is sometimes unaware of which target cell, beam and TCI state was actually selected from these candidates by the UE for the CLTM cell switch procedure. The network also does not know the time point when the CLTM cell switch was triggered by the UE. As a result, some differences may be expected in MRO for CLTM, which are discussed ahead. For other aspects that are similar to rel-19 intra-CU LTM, the already agreed RAN3 solutions from rel-19 are proposed for CLTM.

- The network is sometimes unaware of the target cell, beam and TCI state selected by the UE from among the candidates in a CLTM cell switch procedure, as well as when the cell switch was triggered by the UE.

When a UE performs LTM recovery after a failed CLTM in a particular cell, the recovery DU (i.e. the DU hosting the recovery cell) is unable to immediately identify if this was a result of a failed CLTM, because no CU-DU CELL SWITCH NOTIFICATION message was received. Moreover, the recovery DU cannot know the type of failure, i.e. whether it was a too late CLTM, CLTM failure due to the wrong beam, or invalid TA, as all of these cases appear to be the same at the network level until the RLF report is retrieved. The recovery DU is unaware if the UE attempted CLTM to the same cell that the DU is responsible for, or to a different cell. Due to this, it cannot conclude that it was a too late CLTM failure case. Moreover, as it is unaware of the target beam and TA used by the UE, it cannot conclude that the failure was due to the wrong beam selection or invalid TA. As a result, the network-based solutions agreed for all such failure scenarios in rel-19 for intra-CU LTM cannot be used for intra-CU CLTM.

- RAN3 to agree that rel-19 network-based solutions for intra-CU LTM do not work for the use cases invalid TA and wrong beam selection in case of intra-CU CLTM.

Figure 1: Signalling procedure for intra-CU conditional LTM (inter-DU)

## 2.2 Discussions on use-cases for rel-20

At RAN3 #129-bis, the following was agreed:

- Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO, if applicable.

### 2.2.1 BFR shortly after successful CLTM cell switch

For rel-19 MRO for intra-CU LTM, it was agreed that the target DU shares the SSB index of the recovery beam with the source DU via the CU in an F1AP message, along with the UE F1AP ID. We believe a similar approach can be followed for CLTM as well.

However, as discussed earlier, the source DU is also unaware of the target cell selected by the UE from among the candidates. Whether an explicit indication of the target cell ID is required or if this is implicitly known needs to be discussed.

- Upon BFR shortly after successful CLTM, the target DU shares the SSB index of the recovery beam and the UE F1AP ID with the source DU, via the CU.

- RAN3 to discuss whether indication of the target cell ID is required upon BFR shortly after successful CLTM.

Some scenarios can be considered however where the above solution may not work or may be misleading for the network. As the UE triggers the CLTM by itself, the network is unaware of whether a cell admitted the UE as a result of CLTM, or LTM recovery e.g. after a failed CLTM cell switch, which might have either been towards the same target cell or a different candidate cell. This lack of observability might lead to misleading optimizations by the network in this scenario.

- RAN3 to find a solution to the issue that upon LTM recovery, the network is unaware if there was a previous CLTM cell switch failure, attempted either towards the same cell or to a different cell.

### 2.2.2 Failures due to wrong beam selection and invalid TA

At RAN3 #129-bis, the following issue was identified:

- Additional information required from UE. To be continued...

As mentioned previously, the network based solutions agreed for rel-19 intra-CU LTM do not work in many scenarios for intra-CU CLTM, so UE based solutions are needed for such scenarios. In rel-19, for LTM cell switch failure due to the wrong beam selection, the agreed network based solution involved signaling the SSB index of the recovery beam. A suitable UE-based solution for intra-CU CLTM can also involve the UE logging the SSB index of the recovery beam in the RLF report.

- Upon CLTM cell switch failure due to the wrong beam selection by the UE, the UE logs the SSB index of the beam used in CLTM re-establishment or CLTM recovery in the RLF report.

At RAN3 #129-bis, the following issue was identified:

- FFS whether both L1 and L3 apply to beam related selection optimization.

We note that beam related selection optimization at the network level is only possible for L1-triggered CLTM. In case of L3-triggered CLTM, the beam selection is up to the UE and hence the network has no role in its selection and therefore in its optimization. This needs to be captured as a common understanding.

- RAN3 to come to a common understanding that only L1 based beam selection optimization is applicable for CLTM and the FFS is to be removed.

Following the discussion above, a UE-based solution is also necessary for the case of invalid TA. A straightforward solution can involve the UE logging the invalid TA used in a failed CLTM, however this may not be very useful for optimization of the TA by the network. The RLF report may be retrieved by the network up to 48 hours after the event, at which point, the source DU may no longer have the TA related information for the previously failed CLTM execution. We would hence like to propose differential TA logging, i.e. logging the difference between the PDCCH ordered TA used in the failed CLTM, and the TA received in a RAR (or msgB) during the successful CLTM recovery or re-establishment. This gives the network an indication about the size of the error associated with the TA and is more useful for optimizing it.

- For failure due to invalid TA, the UE logs the difference between TA value used in the failed CLTM and the TA value received in RAR (or msgB) at CLTM re-establishment in the RLF report.

In case of CLTM, the TA provided to the UE by the network is accompanied by a TA timer (TAT) which defines how long this TA is assumed to be valid. We note that it is vital to optimize this TAT in order to avoid issues of TA becoming obsolete too quickly. In case of CLTM cell switch failures, the network is unaware of the value of the remaining time left in the TAT. Hence, we propose that the UE logs this in the RLF report.

Moreover, as the TAT can be defined for up to two TA groups (TAGs) at once, the TAG ID should also be logged for the network to carry out the correct optimizations.

- The UE logs the remaining time of the TAT along with the TAG ID in the RLF report.

### 2.2.4 CLTM cell-switch ping-pong

As with LTM, CLTM cell switches can also experience ping-pong, hence possible solutions should be considered. As a starting point, a solution similar to the one agreed for intra-CU LTM can be considered.

For rel-19 intra-CU LTM, it was agreed that the CU shares the Last Visited LTM Cells list with the DU via the CU-DU CELL SWITCH NOTIFICATION F1AP message for resolving intra-CU LTM ping-pong. However, from figure 1, we see that since the source DU does not take the decision to trigger the CLTM cell switch execution, it cannot notify the CU about it, i.e., the DU-CU CELL SWITCH NOTIFICATION F1AP message from the source DU to the CU is not used. As the CU is unaware of the CLTM execution, therefore it does not notify the target DU about it, i.e., the CU-DU CELL SWITCH NOTIFICATION F1AP message from the CU to the target DU is not used.

- The CELL SWITCH NOTIFICATION F1AP messages between DU and CU are not used in CLTM.

However, the cells involved in the ping-pong in intra-CU CLTM are under the responsibility of the same CU, so it is still possible for a CU to detect and solve a ping-pong CLTM. From figure 1, we note that the CU sends the UE CONTEXT MODIFICATION REQUEST message to both the source and target DUs during CLTM. Hence, we propose that in case of ping-pong, the Last Visited LTM Cells list can be shared via the UE CONTEXT MODIFICATION REQUEST message from the CU to the DU.

- When CU detects CLTM ping-pong, it sends the Last Visited LTM Cells list to the DU via the UE CONTEXT MODIFICATION REQUEST message.

# 3	Conclusion

According to the above discussion, we have the following observations and proposals:

Observation 1: In case of CLTM cell switch, the responsibility for triggering its execution is with the UE rather than the network.

Observation 2: The network is sometimes unaware of the target cell, beam and TCI state selected by the UE from among the candidates in a CLTM cell switch procedure, as well as when the cell switch was triggered by the UE.

Proposal 1: RAN3 to agree that rel-19 network-based solutions for intra-CU LTM do not work for the use cases invalid TA and wrong beam selection in case of intra-CU CLTM.

Proposal 2: Upon BFR shortly after successful CLTM, the target DU shares the SSB index of the recovery beam and the UE F1AP ID with the source DU, via the CU.

Proposal 3: RAN3 to discuss whether indication of the target cell ID is required upon BFR shortly after successful CLTM.

Proposal 4: RAN3 to find a solution to the issue that upon LTM recovery, the network is unaware if there was a previous CLTM cell switch failure, attempted either towards the same cell or to a different cell.

Proposal 5: Upon CLTM cell switch failure due to the wrong beam selection by the UE, the UE logs the SSB index of the beam used in CLTM re-establishment or CLTM recovery in the RLF report.

Proposal 6: RAN3 to come to a common understanding that only L1 based beam selection optimization is applicable for CLTM and the FFS is to be removed.

Proposal 7: For failure due to invalid TA, the UE logs the difference between TA value used in the failed CLTM and the TA value received in RAR (or msgB) at CLTM re-establishment in the RLF report.

Proposal 8: The UE logs the remaining time of the TAT along with the TAG ID in the RLF report.

Observation 3: The CELL SWITCH NOTIFICATION F1AP messages between DU and CU are not used in CLTM.

Proposal 9: When CU detects CLTM ping-pong, it sends the Last Visited LTM Cells list to the DU via the UE CONTEXT MODIFICATION REQUEST message.