3GPP TSG-RAN WG3 Meeting #130	R3-258598

Dallas, US, 17-21 November, 2025

Agenda item:	11.2.2

Source: 	Samsung

Title: 	(TP for SON BLCR to TS 38.401 and TS 38.300) Discussion on MRO enhancement on intra-CU conditional LTM

Document for:	Discussion and decision

Introduction

Last meeting primarily discussed the MRO enhancement on inter-CU LTM, and the following agreements are captured,

Use cases:

At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for Intra-CU conditional LTM, i.e.:

BFR shortly after successful C-LTM cell switch execution, failure due to wrong beam and outdated TA;

FFS whether both L1 and L3 apply to beam related selection optimization.

LTM connection failure: too late and too early C-LTM cell switch execution, C-LTM cell switch execution to wrong cell;

Near failure case: SHR successful HO

Successful case: ping/pong

L3 based intra-CU conditional LTM is within the scope of the WID

Solution options:

Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO for intra-CU C-LTM, if applicable.

For intra-CU C-LTM, the definition of connection failure cases for CHO defined in 38.300 are taken as baseline.

For intra-CU C-LTM, the detection mechanism of connection failure cases in 38.300 are taken as baseline.

For the connection failure cases, the last serving gNB-CU performs initial analysis.

Additional information required from UE

In this contribution, we continue to discuss the support of MRO on intra-CU conditional LTM.

Discussion

Last meeting mainly discussed the supported use cases for intra-CU CLTM, and primarily discussed the detection mechanism.

Based on the latest TS 38.300, the following general description is captured for CLTM,

CLTM cell switch is executed by the UE when L1-based or L3-based LTM cell switch execution conditions are met.

Based on the latest TS 38.401, both intra-DU CLTM and intra-CU inter-DU CLTM cases are supported. In addition, TS 38.401 captures the following description,

The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell, containing conditional LTM indication, one candidate cell ID and the CSI resource configuration for subsequent conditional LTM. The gNB-CU may provide the LTM configuration ID mapping list to the candidate gNB-DU(s). The gNB-CU may request PRACH resources from the candidate gNB-DU(s). The gNB-CU may request the candidate gNB-DU(s) to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer part of the reference configuration to the candidate gNB-DU(s). If the gNB-CU decides to initiate the L1 event-triggered conditional LTM, it also provides a list of candidate cells to which the L1 event-triggered conditional LTM is applied and requests the gNB-DU to generate the corresponding L1-based execution condition(s).

Also TS 38.473 captures the following text,

If the LTM Indicator IE is set to "C-LTM" and the Request for L1 Execution Condition IE is present in the LTM Information Setup IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall generate the conditional LTM L1 execution condition(s) for the candidate cell(s) included in the Request for L1 Execution Condition IE and provide the L1 Execution Condition List IE within the LTM Configuration IE in the UE CONTEXT SETUP RESPONSE message.

According to the description above, we can observe that,

- For CLTM, the UE can be configured with L1 execution condition and/or L3 execution condition.

- For CLTM, the L1 execution condition is determined by DU, and the L3 execution condition is determined by CU.

- The gNB-CU provides the candidate cell list for intra-CU CLTM, and gNB-DU may accept only partial of the candidate cells in the candidate cell list. CU determines L3 execution condition for those accepted candidate cells.

- The gNB-CU also provides a list of candidate cells contain in the Request for L1 Execution Condition IE subject to a specific candidate cell, and the L1 execution conditions configured by gNB-DU is from the specific candidate cell to the associated list of candidate cells. In addition, the gNB-DU configures L1 execution condition for all candidate cells which are requested by gNB-CU.

Based on the above observations, the reasons to cause failures can be due to,

- Inappropriate L1 CLTM execution condition generated by DU.

- Inappropriate L3 CLTM execution condition generated by CU.

- Inappropriate candidate cell list provided by CU (so that the CU is unable to generate L3 CLTM execution condition or unable to obtain L1 CLTM execution condition for those appropriate candidate cells).

- Inappropriate candidate cell list accepted by DU (so that the CU is unable to generate L3 CLTM execution condition for those appropriate candidate cells).

Proposal 1: The possible reasons for CLTM failure cases are as follows,

- Inappropriate L1 CLTM execution condition generated by DU.

- Inappropriate L3 CLTM execution condition generated by CU.

- Inappropriate candidate cell list provided by CU.

- Inappropriate candidate cell list accepted by DU.

After the failure occurs and UE re-establishes/recovers to another cell within the same CU, when the CU receives the RLF report from the UE, the CU performs initial analysis. If the CU identifies the failure is caused by the CU, the CU will do optimization by itself; while if the CU identifies the failure may be caused by the DU, the CU needs to forward the RLF report to the last serving/source DU.

Proposal 2: In case of failures due to inappropriate configuration on L1 CLTM execution condition or inappropriate candidate cell list accepted by DU, the gNB-CU forwards the RLF report to the last serving gNB-DU in case of too late CLTM execution, or to the source gNB-DU in case of too early CLTM execution or CLTM execution to wrong cell.

Then the open issue can be discussed based on the above analysis. The first open issue is,

BFR shortly after successful C-LTM cell switch execution, failure due to wrong beam and outdated TA;

FFS whether both L1 and L3 apply to beam related selection optimization.

In our understanding, we do acknowledge such use cases may happen, but whether additional spec impact is required should be further discussed.

For use cases including BFR shortly after successful CLTM cell switch execution and failure due to wrong beam, different from normal LTM cell switch, the beam is selected by UE for CLTM cell switch execution. As a result, since the UE performs beam selection for CLTM, the network is unable to optimize beam selection for CLTM; instead, since the wrong beam selection performed by the UE can be due to inappropriate L1/L3 CLTM execution condition or inappropriate candidate cell list configured to the UE, the solution to solve such use case can reuse the solution to solve inappropriate L1/L3 CLTM execution condition or inappropriate candidate cell list.

Observation 1: Because the UE performs beam selection for CLTM, the network is unable to optimize beam selection for CLTM.

Observation 2: The wrong beam selection by UE is due to inappropriate L1/L3 CLTM execution condition or inappropriate candidate cell list configured to the UE.

Proposal 3: For BFR shortly after successful CLTM cell switch execution and failure due to wrong beam use cases, the gNB-CU may forward the RLF report to the source gNB-DU.

The next open issue is as follows,

Additional information required from UE

In order for the gNB-CU/gNB-DU to identify the failure reason as mentioned in P1, the UE may need to include the following information in the RLF report:

- CLTM type: It indicates the last executed handover type is CLTM before the last detected connection failure. It is similar to what we have already introduced for CHO and LTM, but RAN2 may further discuss whether to reuse the current codepoint or define a new one.

- L1 execution failure: It indicates the last executed CLTM is an L1 execution before the last detected connection failure. Such information can be used for identifying Too early CLTM execution or CLTM execution to wrong cell.

- L3 execution failure: It indicates the last executed CLTM is an L3 execution before the last detected connection failure. Such information can be used for identifying Too early CLTM execution or CLTM execution to wrong cell.

- Cell ID for which the execution is performed.

- Cell list for which the execution conditions are fulfilled. Such information can be included when the execution conditions for multiple candidate cells are fulfilled (and UE selects one of the candidate cells for CLTM execution by implementation).

- Time from the execution condition met to failure. Such information can be used for distinguishing Too late CLTM execution from Too early CLTM execution/CLTM execution to wrong cell (i.e. the absence of such information will implicitly indicate that a Too late CLTM execution happens).

- Time from adding execution condition (upon receiving RRCReconfiguration message) to failure. Such information can be used for optimising CLTM configuration.

- CLTM Time alignment timer remaining time: It indicates the remaining time till the timer expiration of the CLTM Time alignment timer. Such information can be used for identifying TA outdated failure case and can be used e.g. by the gNB-CU/gNB-DU to determine a better timing on the triggering of early sync acquisition.

- CLTM RACH-less access failure indication: It indicates whether a RACH-less access failure happens before the successful RACH-based access to the same target cell. Such information can be used for identifying TA outdated failure case and can be used e.g. by the gNB-DU to provide a more proper CLTM RACH-less configuration.

Proposal 4: RAN3 is kindly asked to discuss whether to introduce the following information in the RLF report from UE for CLTM,

- CLTM type: It indicates the last executed handover type is CLTM before the last detected connection failure.

- No execution failure: It indicates neither L1 nor L3 execution conditions are fulfilled before the last detected connection failure.

- L1 execution failure: It indicates the last executed CLTM is an L1 execution before the last detected connection failure.

- L3 execution failure: It indicates the last executed CLTM is an L3 execution before the last detected connection failure.

- Cell ID for which the execution is performed.

- Cell list for which the execution conditions are fulfilled.

- Time from the execution condition met to failure.

- Time from adding execution condition (upon receiving RRCReconfiguration message) to failure.

- CLTM Time alignment timer remaining time: It indicates the remaining time till the timer expiration of the CLTM Time alignment timer.

- CLTM RACH-less access failure indication: It indicates whether a RACH-less access failure happens before the successful RACH-based access to the same target cell.

Proposal 5: Agree the TPs and the LS in the Annex.

Conclusion

In this paper, we provide discussion on MRO enhancement on intra-CU conditional LTM. The following proposals are provided,

Proposal 1: The possible reasons for CLTM failure cases are as follows,

- Inappropriate L1 CLTM execution condition generated by DU.

- Inappropriate L3 CLTM execution condition generated by CU.

- Inappropriate candidate cell list provided by CU.

- Inappropriate candidate cell list accepted by DU.

Proposal 2: In case of failures due to inappropriate configuration on L1 CLTM execution condition or inappropriate candidate cell list accepted by DU, the gNB-CU forwards the RLF report to the last serving gNB-DU in case of too late CLTM execution, or to the source gNB-DU in case of too early CLTM execution or CLTM execution to wrong cell.

Observation 1: Because the UE performs beam selection for CLTM, the network is unable to optimize beam selection for CLTM.

Observation 2: The wrong beam selection by UE is due to inappropriate L1/L3 CLTM execution condition or inappropriate candidate cell list configured to the UE.

Proposal 3: For BFR shortly after successful CLTM cell switch execution and failure due to wrong beam use cases, the gNB-CU may forward the RLF report to the source gNB-DU.

Proposal 4: RAN3 is kindly asked to discuss whether to introduce the following information in the RLF report from UE for CLTM,

- CLTM type: It indicates the last executed handover type is CLTM before the last detected connection failure.

- No execution failure: It indicates neither L1 nor L3 execution conditions are fulfilled before the last detected connection failure.

- L1 execution failure: It indicates the last executed CLTM is an L1 execution before the last detected connection failure.

- L3 execution failure: It indicates the last executed CLTM is an L3 execution before the last detected connection failure.

- Cell ID for which the execution is performed.

- Cell list for which the execution conditions are fulfilled.

- Time from the execution condition met to failure.

- Time from adding execution condition (upon receiving RRCReconfiguration message) to failure.

- CLTM Time alignment timer remaining time: It indicates the remaining time till the timer expiration of the CLTM Time alignment timer.

- CLTM RACH-less access failure indication: It indicates whether a RACH-less access failure happens before the successful RACH-based access to the same target cell.

Proposal 5: Agree the TPs and the LS in the Annex.

Reference

Annex A: TP to TS 38.401

<< Start of Change >>

7.X	MRO support for LTM

The gNB-CU receives an RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis. In case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, the gNB-CU forwards the RLF report to the last serving gNB-DU in case of too late LTM cell switch, or to the source gNB-DU in case of too early LTM cell switch or LTM cell switch to wrong cell. In case of failures due to inappropriate configuration on L1 CLTM execution condition or inappropriate candidate cell list accepted by DU, the gNB-CU forwards the RLF report to the last serving gNB-DU in case of too late CLTM execution, or to the source gNB-DU in case of too early CLTM execution or CLTM execution to wrong cell.

The target gNB-DU identifies that a Beam Failure Recovery (BFR) has happened in the UE shortly after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt). The target gNB-DU performs initial analysis and sends the recovery beam information to the source gNB-DU via the gNB-CU.

The target gNB-DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell. The target gNB-DU sends the re-established or recovery beam information, the TA value used at successful RACH-based access, or both to the source gNB-DU via the gNB-CU.

<< End of Change >>

Annex B: TP to TS 38.300

<< Start of Change >>

15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM/CLTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM/CLTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM/CLTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM/CLTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM/CLTM cell switch, Too Early LTM/CLTM cell switch and LTM/CLTM cell switch to Wrong Cell.

In case of CHO with candidate SCG(s), the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO with candidate SCG(s), Too Early CHO with candidate SCG and CHO Execution to Wrong Cell.

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM/CLTM or CHO with candidate SCG(s) is configured but the CHO/LTM/CLTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover/LTM/CLTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/the cell UE attempts LTM recovery is the cell that served the UE at the last handover/LTM/CLTM initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover/LTM/CLTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM recovery is neither the cell that served the UE at the last handover/LTM/CLTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM/CLTM was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO/LTM/CLTM execution until connection failure.

In case of Too Early Handover or Handover to Wrong Cell, the NG-RAN node receiving the failure indication may inform the NG-RAN node controlling the cell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report.

In case of Too Late Handover when UE is configured with CHO with candidate SCG(s), the NG-RAN node receiving the failure indication may send a Handover Report over Xn to the candidate NG-RAN node(s) controlling the candidate PSCell(s) whose mobility configuration caused the failure.

For MRO analysis, in case of RLFs or handover failures, the source gNB may take into account the information regarding the LBT failures occurred in UL and DL for a specific UE. For UL, the information may be collected and provided by the UE to the network in the RLF report. For DL, the information may be collected at the source and target gNBs. The target gNB may provide the information to the source gNB via the Access and Mobility Indication procedure over Xn, if requested by the source gNB during the handover preparation.

<< End of Change >>

Annex C: draft LS to RAN2

Title:	[DRAFT] LS on MRO enhancement on CLTM

Response to:

Release:	Release 20

Work Item:	NR_SON_MDT_Ph5-Core

Source:	Samsung (to be RAN3)

To:	RAN2

Cc:

Contact Person:

Name:	Xingyu HAN

Tel. Number:

E-mail Address:	xingyu.han@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN3 is discussing the MRO enhancement on conditional LTM, and identifies that the following information is beneficial to be provided by UE in RLF report,

- CLTM type: It indicates the last executed handover type is CLTM before the last detected connection failure.

- No execution failure: It indicates neither L1 nor L3 execution conditions are fulfilled before the last detected connection failure.

- L1 execution failure: It indicates the last executed CLTM is an L1 execution before the last detected connection failure.

- L3 execution failure: It indicates the last executed CLTM is an L3 execution before the last detected connection failure.

- Cell ID for which the execution is performed.

- Cell list for which the execution conditions are fulfilled.

- Time from the execution condition met to failure.

- Time from adding execution condition (upon receiving RRCReconfiguration message) to failure.

- CLTM Time alignment timer remaining time.

- CLTM RACH-less access failure indication: It indicates whether a RACH-less access failure happens before the successful RACH-based access to the same target cell.

2. Actions:

To RAN2:

ACTION: 	RAN3 kindly asks RAN2 to take the above information into account, and provide feedback if any.

3. Date of Next TSG RAN3 Meetings:

TSG-RAN3 Meeting #131           09-13 February 2026                           Goteborg, SE

TSG-RAN3 Meeting #131-bis       13-17 April 2026                                Malta, MT