3GPP TSG-RAN WG3 Meeting #130	R3-258684

Dallas, USA, 17-21, Nov 2025

Agenda Item:	10.5.1

Source:	CEWiT

Title:	Discussion on AI/ML use cases for 6G Network

Document for:	Discussions & Approval

# 1	Introduction

AI/ML is assumed to be one of the key pillars for the 6G technology. It is a powerful tool for optimizing complex problems across different layers of a network. In 3GPP, RP-251881[1] introduced a AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework and following objectives were made.

AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In RAN3#129bis [2], following agreement was made for AI/ML for 6G Radio Access Network:In RAN3#129bis [2], following agreement was made for AI/ML for 6G Radio Access Network:

the design of AI/ML algorithms and models for RAN3 led use cases are implementation-specific and out of RAN3 scope. The following use cases will be studied in RAN3: - AI/ML based Network energy saving - AI/ML based mobility optimization Other use cases can be discussed in future meetings based on contributions.

This discussion paper has been prepared to explore the high-level scenarios for the agreed potential AI/ML use cases in 6G and the RAN along with additional use case to be considered.

# 2	Discussion

## 2.1	High-level principles

From Release 17 to the current Release 19, research on AI/ML for NG-RAN has been ongoing. RAN3 has outlined AI/ML-assisted application cases and established the functional foundation for RAN intelligence within this scope. AI/ML-related subjects have also been introduced by various working groups within their own fields of study. Network energy saving, load balancing, mobility optimization, network slicing, and coverage and capacity optimization (CCO) are some of the specified AI/ML use cases for NG-RAN. It is anticipated that RAN3 would take into account integrating the established non-AI/ML features specified under 5GA into the 6G AI/ML study framework as it moves closer to 6G. To enable novel AI/ML-driven use cases in this environment, cooperation between various network entities will be crucial. It should be mentioned that AI/ML-based solutions could not always perform better than conventional implementations, depending on the communication environment. Large and representative datasets are necessary for AI/ML models to function satisfactorily in both training and inference phases. The control plane controls the transfer of AI-related data between RAN nodes in the present 5GA phase. An effective data transmission mechanism must be studied for 6G since AI/ML data interchange may need to span several protocol layers and network domains.

Further, AI/ML-related data usually consists of input, output, and feedback data during the 5GA phase. Information created by RAN AI/ML functions is referred to as output data; network monitoring information is referred to as feedback data; and input data may come from other network elements or be gathered within the RAN domain. Model-specific information and AI/ML traffic data will be added to AI/ML-related data in the 6G environment, increasing the number of data categories and the corresponding transmission requirements.

- Following high-level principles should be applied to AI/ML for NG-RAN in 6G:

        - Incorporation of the established non-AI/ML functionalities defined under 5GA into the 6G AI/ML

          - Non-AI/ML-based solutions should always be supported in real network deployments.

        - Multiple network entities should be able to operate together to support 6G AI/ML use cases.

        - Efficient data collection and signalling is beneficial to support 6G AI/ML use cases.

        - AI/ML-related data should atleast include output data, feedback data; and input data, and AI/ML traffic data.

## 2.2 	Use Cases

Network Load Balancing, Network Energy Saving, Mobility Optimization, Network Slicing, Coverage and Capacity Optimization, and other AI/ML-assisted use cases were recognized and described by RAN3 during the 5G-A discussion phase. In the meantime, RAN1 identified AI/ML use cases like positioning, beam control, and CSI feedback. LTM with AI/ML support in RAN3 during the Rel-20 stage.  Following AI/ML-assisted use cases agreed in RAN3#129bis needs to be explored in the 6G phase for different scenarios and may even undergo further study:

### 2.2.1	AI/ML based Network Energy Saving

Within the IMT-2030 framework, sustainability and energy efficiency are central design principles for 6G systems. As networks evolve toward 6G, optimizing energy consumption especially at base stations (BS or gNB) has become vital to achieving these goals. Consequently, advanced energy-saving mechanisms, applicable to both connected and idle or inactive user equipment (UE), are needed. Leveraging AI/ML-based strategies can further enhance the energy efficiency and operational flexibility of base stations.  Machine learning algorithms can predict energy efficiency and network load for upcoming periods, enabling intelligent decisions on cell activation and deactivation to achieve energy savings. Based on predicted load conditions, the system can dynamically adapt energy-saving strategies such as switch-off timing, and offloading actions to balance system performance with efficiency.

## Enhancements to Spatial and Power Domain Adaptations

- A 6G energy-saving framework should enable joint, dynamic adaptation of spatial elements and transmit power for both UE-specific and cell-specific downlink signals, optimizing real-time energy use and link quality. While 5G’s early spatial and power adaptation methods are restricted with limited applicability & flexibility, and increased CSI overhead, 6G must emphasize AI/ML-driven, context-aware spatial elements and power adaptations to reduce complexity and signalling. AI/ML enables predictive and smarter optimization of antenna and power configurations, optimized CSI reporting for various subsets of antenna elements and power, and greater flexibility, further enhanced by UE-assisted information for network decisions. Efficient CSI framework and broader adaptation including cell-specific signals are vital for AI/ML based adaptations, especially in dense networks. Both network-driven and UE sided models along with standardized data collection, signalling and model management can be considered.

- AIML based NES in 6GR should support enhancement to spatial and power domain adaptations for energy savings, considering:

  - Data collection for adapting number of ports/antenna elements/power offsets

  - Optimal CSI framework for various adaptations

  - UE assistance information as feedback data across nodes

## Enhanced DTX/DRX

- Cell DTX/DRX reduces power at both BS and UE by enabling planned “off” periods for transmission and reception. 5G’s initial DTX/DRX support is mainly limited to UE-specific channels, with always-on cell signals (like SSB and SIB) still requiring active network operation limiting energy savings. Further, there’s alignment issues with UE-DRX cycles for a number of UEs which impacts on QoS requirements and performance at UE. For 6G, DTX/DRX should extend to both UE-specific and cell-wide signals to achieve greater energy savings. AI/ML methods can enable intelligent optimization of Cell-DTX/DRX through predictive traffic modelling, fast reconfiguration, and context-aware control. These models can support adaptive management that outperforms static configurations by dynamically adjusting DTX/DRX patterns including on-duration, cycle, and start time for improved efficiency. Coordination between cell-level DTX and UE-DRX, enhanced by data collection across nodes, optimal framework, and user assistance information, further maximizes energy savings by aligning network decisions with UE requirements.

- 6G should support AI/ML based cell DTX/DRX, the enhancement should consider

  - Data collection for dynamic adaptation of cell DTX/DRX

  - Optimal framework and alignment for various adaptations across nodes

  - UE assistance information as feedback data across nodes

### 2.2.2	AI/ML based mobility optimization

Mobility optimization remains a fundamental challenge for ensuring seamless connectivity and service continuity in wireless networks. In both 5G and 6G, the ability to maintain robust mobility performance is essential to minimize call drops, radio link failures (RLFs), unnecessary and ping-pong handovers, while maintaining high Quality of Experience (QoE). With the emergence of AI/ML capabilities in the RAN (starting from Rel-18) and the continued evolution of LTM and CHO procedures, the 6G era provides an opportunity to design a unified and intelligent mobility framework.  This unified framework is expected to integrate L3 handover, L1/L2 triggered mobility (LTM/C-LTM), and conditional handovers, enhanced by AI/ML-based predictive mechanisms. Building on ongoing RAN2 and RAN3 discussions on AI/ML-assisted RRM and measurement prediction, 6G should focus on defining potential enhancements and mechanisms for AI/ML-based mobility optimization.

# Motivation :

# Gaps from 5G and 5G-A :

Existing 	AI/ML for 	NG_RAN mechanisms in Rel-18 	and Rel-19 primarily focus on L3 handover optimizations and 	cell-based UE trajectory prediction. The current 	solutions also have limited support to multi-hop, leading to dependency chains and reduced robustness in multi-node or high-speed scenarios. Also the current specification on AI/ML for mobility optimization is not supporting L1/L2-triggered nor across intra and inter-node decision domains. And further, no consistent use of contextual sensing, native AI decision loops, or multi-hop prediction 	mechanisms to support predictive mobility in heterogeneous 6G 	deployments.

# Requirements for 6GR :

- High-speed 	mobility (e.g., high-speed trains, UAVs, and NTN integration) 	demands minimal handover latency and robust predictive mobility.

- Dense network and use of FR3/THz bands will lead to frequent cell transitions where LTM and CHO need proactive optimization.

- Sensing data and data-plane contextual information can enrich AI/ML model inputs for more accurate trajectory and mobility decisions.

# Potential Enhancements :

Unified AI/ML Framework for L1–L3 Mobility:

Introduce a 	unified mobility optimization framework that jointly handles the requirements of:

- L3 handover (legacy & 	conditional)

- L1/L2 triggered mobility 	(LTM/C-LTM)

- Unified AI/ML Mobility Framework.

- RAN3 should 	define a unified AI/ML-based mobility optimization architecture that 	integrates L3 handover (legacy and CHO) and L1/L2 triggered mobility 	(LTM/C-LTM).

Sensing-Aided and Context-Enriched Model Inputs for mobility optimizations:

Incorporate sensing data, 	environmental context, and service/data-plane feedback into AI/ML 	model training and inference. Examples of inputs:

- RAN sided 	sensing reports (object/environment reflections, blockage 	indicators) and UE-side motion data 	(velocity, orientation).

- QoS and latency requirements from service plane.

- Sensing-Enhanced Input for AI/ML Models.

- RAN3 should evaluate mechanisms 	for integrating sensing-based data and service-plane data into AI/ML 	model inputs for mobility optimization.

- Such 	data can enhance model accuracy in predicting trajectory, beam, or 	cell transitions in complex radio environments.

### 2.2.3	AI/ML based positioning for network optimization

In the 6G system, UE(s) positioning service is expected to evolve from being a “supporting capability” for external applications to becoming a “strategic enabler” for network quality enhancement, intelligence and automation.The availability of precise and timely UE(s) location information can allow the network to better understand spatial and temporal dynamics in user behavior, service demand, and mobility patterns.Such insight is increasingly relevant as 6G networks are expected to operate in a data-driven, context-aware, and AI-assisted manner.

Further, the UE(s) positioning service is not envisioned as a network information source for general optimization or automation purposes in 5G NR.

The 6G-RAN should support a capability that enables the use of UE(s) positioning information by the network to enhance its automation and self-organizing functionalities. NR already supported the AIML based positioning and data collection with model being in NG-RAN. This feature should be enhanced from network optimization perpective. AIML model can derive the statistical UE distribution map for taking multiple network related decisions. This can support the following capabilities:

- RAN automation and self-organization:

UE positioning information can enhance self-configuration, self-optimization, and self-healing processes by introducing spatial context in term of location information. Examples include automatic neighbor relation creation, geography-specific mobility configuration, and location-aware interference mitigation. Position-enabled automation allows UE-specific or group-specific optimization, improving overall system adaptability.

- Contextual and data-driven operation:

UE(s) Positioning data adds a new dimension to RAN data analytics, enabling AI/ML functions to better predict and optimize future states of the networks. Spatial context can improve accuracy of mobility prediction, beam selection, and resource scheduling by allowing models to correlate user motion and density with performance trends.

- Intelligent infrastructure optimization:

UE location information complements radio and environmental measurements to improve the operation of intelligent infrastructures such as relays, network-controlled repeaters (NCRs), and reconfigurable intelligent surfaces (RIS). This enables dynamic path optimization and effective mitigation of coverage holes based on real-time user distribution.

By combining these capabilities, the Positioning-Assisted Network Optimization service allows the 6G system to transition from reactive network to predictive, context-aware, and self-optimizing network using AI/ML models. This integration of positioning information within the system’s automation and AI frameworks enhances performance, efficiency, and sustainability while maintaining compliance with privacy and security requirements.

- : 6G should support AI/ML based positioning for network automation and self-organizing. NR AIML positioning should be considered as baseline..

# 3	Conclusions

Following is our proposals and observations:

- Following high-level principles should be applied for AI/ML RAN in 6G:

        - Incorporation of the established non-AI/ML functionalities defined under 5GA into the 6G AI/ML

          - Non-AI/ML-based solutions should always be supported in real network deployments.

        - Multiple network entities should be able to operate together to support 6G AI/ML use cases.

        - Efficient data collection and signalling is beneficial to support 6G AI/ML use cases.

        - AI/ML-related data should at least include output data, feedback data; and input data, and AI/ML traffic data.

- AIML based NES in 6GR should support enhancement to spatial and power domain adaptations for energy savings, considering:

  - Data collection for adapting number of ports/antenna elements/power offsets

  - Optimal CSI framework for various adaptations

  - UE assistance information as feedback data across nodes

- 6G should support AI/ML based cell DTX/DRX, the enhancement should consider

  - Data collection for dynamic adaptation of cell DTX/DRX

  - Optimal framework and alignment for various adaptations across nodes

  - UE assistance information as feedback data across nodes

- Unified AI/ML Mobility Framework.

  - RAN3 should 	define a unified AI/ML-based mobility optimization architecture that 	integrates L3 handover (legacy and CHO) and L1/L2 triggered mobility 	(LTM/C-LTM).

- Sensing-Enhanced Input for AI/ML Models.

  - RAN3 should evaluate mechanisms 	for integrating sensing-based and service-plane data into AI/ML 	model inputs for mobility optimization.

  - Such 	data can enhance model accuracy in predicting trajectory, beam, or 	cell transitions in complex radio environments.

- 6G should support AI/ML based positioning for network automation and self-organizing. NR AIML positioning should be considered as baseline.

# 4	Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

# 7	AI/ML for RAN

## 7.1	AI/ML use cases

Editor’s note: Aspects related to

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

Following high-level principles should be applied for AI/ML RAN in 6G:

- Incorporation of the established non-AI/ML functionalities defined under 5GA into the 6G AI/ML

  - Non-AI/ML-based solutions should always be supported in real network deployments.

- Multiple network entities should be able to operate together to support 6G AI/ML use cases.

- Efficient data collection and signalling is beneficial to support 6G AI/ML use cases.

- AI/ML-related data should include output data, feedback data; and input data, Mand AI/ML traffic data.

### 7.1.1	Exisiting AI/ML assisted use cases

RAN3 identified and specified AI/ML-assisted use cases, including Network Energy Saving, and Mobility Optimization. Considering the interest and complexity, following use cases are essential to continue standardizing and studying in 6G:

-	Network Energy Saving

- AI/ML Enhancement to spatial and power domain adaptations for energy savings, considering:

  - Data collection for adapting number of ports/antenna elements/power offsets

  - Optimal CSI framework for various adaptations

  - UE assistance information as feedback data across nodes

- AI/ML based cell DTX/DRX, the enhancement should consider

  - Data collection for dynamic adaptation of cell DTX/DRX

  - Optimal framework and alignment for various adaptations across nodes

  - UE assistance information as feedback data across nodes

## 7.2 	AI/ML framework

Editor’s note: Aspects related to

- LCM procedures [RAN2, RAN1, RAN3, RAN4]

- Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>