3GPP TSG-RAN WG3 Meeting #130	R3-258759

Dallas, US, 17-21 November, 2025

Title:	LS on the encoding of AIOTF Identifier

Response to:

Release:	Release 19

Work Item:	Ambient_IoT_solutions-Core

Source:	RAN3

To:	CT4

Cc:	SA2, CT3

Contact Person:

Name:	Xingyu HAN

Tel. Number:

E-mail Address:	xingyu.han@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN3 is discussing the encoding of AIOTF Identifier over NGAP, and the following description has been specified in TS 38.413 v19.0.0, considering that in TS 29.518 NfInstanceId is used as the Data type of aiotfId:

9.3.3.70	AIOTF Identifier

This IE uniquely identifies the AIOTF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| AIOTF Identifier | M |  | OCTET STRING | Coded as the NfInstanceId IE defined in TS 29.571 [35]. |

RAN3 would like to specify the size of the OCTET STRING. On determining the size of the OCTET STRING, RAN3 finds the following description in TS 29.571 for NfInstanceId,

| NfInstanceId | string | String uniquely representing a NF instance. The format of the NF Instance ID shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 9562 [15]. The hexadecimal letters should be formatted as lower-case characters by the sender, and they shall be handled as case-insensitive by the receiver. Example: "4ace9d34-2c69-4f99-92d5-a73a3fe8e23b" (NOTE 3) |
| --- | --- | --- |

RAN3 observes that TS 29.571 captures ‘hex-and-dash’ string UUID format as an example in the description of NfInstanceId, but does not provide a clear description on how the NfInstanceId is encoded.

For now, RAN3 has two different understandings on how the NfInstanceId is encoded as defined in TS 29.571. More specifically,

Understanding 1: Each hexadecimal letter is encoded to 4 bits, and the hyphens are not encoded. Therefore, the size of the encoded string is 128 bits (i.e. 16 octets) for 32 hexadecimal letters.

Understanding 2: Each hexadecimal letter and each hyphen is encoded to 8 bits. Therefore, the size of the encoded string is 36 octets for 32 hexadecimal letters plus 4 hyphens.

2. Actions:

To CT4:

ACTION: 	RAN3 kindly asks CT4 to confirm which understanding is correct.

3. Date of Next TSG RAN3 Meetings:

TSG-RAN3 Meeting #131           09-13 February 2026                           Goteborg, SE

TSG-RAN3 Meeting #131-bis       13-17 April 2026                                Malta, MT