3GPP TSG-RAN WG3 Meeting #130	R3-258759

Dallas, US, 17-21 November, 2025

Title:	LS on the encoding of NfInstanceId

Response to:

Release:	Release 19

Work Item:	Ambient_IoT_solutions-Core

Source:	RAN3

To:	CT4

Cc:	SA2

Contact Person:

Name:	Xingyu HAN

Tel. Number:

E-mail Address:	xingyu.han@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN3 is discussing the encoding of AIOTF Identifier over NGAP, and the following description has been specified in TS 38.413 v19.0.0:

9.3.3.70	AIOTF Identifier

This IE uniquely identifies the AIOTF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| AIOTF Identifier | M |  | OCTET STRING | Coded as the NfInstanceId IE defined in TS 29.571 [35]. |

RAN3 would like to specify the size of the OCTET STRING. On determining the size of the OCTET STRING, RAN3 finds the following description in TS 29.571 for NfInstanceId,

| NfInstanceId | string | String uniquely representing a NF instance. The format of the NF Instance ID shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 9562 [15]. The hexadecimal letters should be formatted as lower-case characters by the sender, and they shall be handled as case-insensitive by the receiver. Example: "4ace9d34-2c69-4f99-92d5-a73a3fe8e23b" (NOTE 3) |
| --- | --- | --- |

RAN3 observes that TS 29.571 captures ‘hex-and-dash’ string UUID format as an example in the description of NfInstanceId, but does not provide a clear description on how the NfInstanceId is encoded.

For now, RAN3 has two different understandings on how the NfInstanceId is encoded as defined in TS 29.571. More specifically,

Understanding 1: Each hexadecimal letter is encoded to 4 bits, and the hyphens are not encoded. So the size of the encoded string is 128 bits (i.e. 16 octets) for 32 hexadecimal letters.

Understanding 2: Each hexadecimal letter and each hyphen is encoded to 8 bits. So the size of the encoded string is 36 octets for 32 hexadecimal letters plus 4 hyphens.

2. Actions:

To CT4:

ACTION: 	RAN3 kindly asks CT4 to confirm which understanding is correct.

3. Date of Next TSG RAN3 Meetings:

TSG-RAN3 Meeting #131           09-13 February 2026                           Goteborg, SE

TSG-RAN3 Meeting #131-bis       13-17 April 2026                                Malta, MT