3GPP TSG-RAN WG3 Meeting #130	R3-25xxxx

Dallas, US, 17 - 21 November, 2025

Agenda Item:	13.3

Source:	Xiaomi (moderator)

Title:	Summary of Offline Discussion for CB#17 ISAC

Document for:	Discussion

1	Introduction

This SoD is based on the following online discussion：

Capture definition of Sensing Function, referring to SA2 TR

The sensing function selects gNB(s) based at least on information about the gNB(s).

For Control Plane protocol stack, capture it is SCTP-based with NxAP (i.e. NGAP or new AP)

Discuss what gNB information is useful for Sensing Function to know to select gNB(s)

There is a Sensing Initiation function that includes a Class 1 procedure (Sensing Request/Response)

FFS whether Sensing Report is signaling procedure

There is a Sensing Stop function that can be SF-initiated and can be gNB-initiated (class 2).

FFS whether SF-initiated Sensing Stop procedure is class 1 or class 2.

CB: # 17_ISAC

-  Capture above agreements and open issues in the TR

(Xiaomi - moderator)

2	For the Chair Notes

Editor’s Note: For Rel-20 study/work items, please consider that when agreements/FFSes are captured in a TP, additional inclusion in the Chair Notes may be unnecessary (particularly for stage 3 details).

Propose the following:

- TP to TR38.765 for general aspects and protocols

- TP to TR38.765 for basic procedures

Propose to capture the following in Chair Notes:

To be continued:

3	TP to pCR on network architecture (13.2)

3.1 Capture online agreements

A draft TP is provided based on the following agreements made online.

Capture definition of Sensing Function, referring to SA2 TR The sensing function selects gNB(s) based at least on information about the gNB(s). For Control Plane protocol stack, capture it is SCTP-based with NxAP (i.e. NGAP or new AP)

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

--2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-252819: "Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR".

[x1]	3GPP 3GPP TR 23.700-14: “Study on Integrated Sensing and Communication; Stage 2”.

<<<<<<<<<<<<<<<<<<<< Next change>>>>>>>>>>>>>>>>>>>>

--3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Sensing Function 	SF

<<<<<<<<<<<<<<<<<<<< Next change>>>>>>>>>>>>>>>>>>>>

--7	Network architecture

Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied.

This clause identifies and describes the logical architecture to support the sensing in the overall 5G system architecture.

Figure 7.1 depicts a logical architecture for ISAC, where the Nx interface is between the gNB and the SF.

Figure 7.1	Logical architecture for ISAC

This logical architecture is independent of the transport, e.g. direct or via the AMF, between the gNB and the SF.

Editor’s Note:  The details of "Nx" interface need further discussion and decision.

SF: represents the Sensing Function defined in TR 23.700-14 [x1].

The sensing function selects gNB(s) based at least on the information about the gNB(s).

--7.x1	Protocol stack for sensing control signalling

Figure 7.x1-1 shows the protocol stack for sensing control signalling transmission between gNB and SF:

Figure 7.x1-1. Protocol Stack for sensing control signalling

Editor’s Note: Nx-AP could be NGAP or new protocol.

Editor’s Note: FFS on the protocol stack for sensing data report.

<<<<<<<<<<<<<<<<<<<< Change end >>>>>>>>>>>>>>>>>>>>

3.2 Discussion on online open issues

gNB Information to be known by SF

Although this open issue is captured in AI 13.3, the discussion is more related to AI 13.2, so the moderator put the discussion here.

Discuss what gNB information is useful for Sensing Function to know to select gNB(s)

7.1.3 Agreed Principles for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection: - The Sensing Function selection is based on NEF's local configuration or NRF query. - The Sensing Function registers itself to the NRF, with its NF profile. - Sensing Function is selected based on at least its supported sensing area. - Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request. - Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area.

According to the SA2 conclusion, at least the supported sensing area of a gNB should be considered as gNB information for sensing function.

Proposal 1: capture the following in the TP:

The gNB information includes the supported sensing area of a gNB.

Editor’s note x1: FFS on the other information needed.

Editor’s note x2: FFS on whether siganlling approach is needed for SF to obtain the gNB information.

Moderator summary:

4	TP to pCR on RAN-CN procedures (13.3)

4.1	Capture online agreements

A draft TP is provided based on the following agreements and open issues made online.

There is a Sensing Initiation function that includes a Class 1 procedure (Sensing Request/Response) FFS whether Sensing Report is signaling procedure There is a Sensing Stop function that can be SF-initiated and can be gNB-initiated (class 2). FFS whether SF-initiated Sensing Stop procedure is class 1 or class 2.

Some background information for companies to consider whether and how we use function or procedures for what we agreed/discussed. In both NRPPa and NGAP, the function to support the new feature/service is very general and includes multiple procedures.

| NRPPa | Function; Elementary Procedure(s). Measurement Information Transfer; a) Measurement b) Measurement Update c) Measurement Report d) Measurement Abort e) Measurement Failure Indication. |
| --- | --- |
| NGAP | 5.32 AIoT Service Management function The AIoT Service Management function is responsible for handling AIoT services. 6.28 AIoT procedures The following procedures are used for AIoT services: - Inventory Request; - Inventory Report; - Command Request; - A-IoT Session Release (AIoT CN Initiated); - A-IoT Session Release Request (gNB initiated). |

Therefore, we may need a general function (e.g., sensing management function) includes some elementary procedures.

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

--8	RAN-CN procedures and signalling

Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

Editor’s Note: the following may need further refinement.

The Nx-AP protocol supports the sensing management function.

The following procedures are used for sensing management function:

-	Sensing Initiation: includes a Class 1 procedure (Sensing Request/Response);

-	SF-initiated Sensing Stop;

- 	gNB-initiated Sensing Stop: includes a Class 2 procedure.

Editor’s Note: FFS on other functions and procedures.

Editor’s Note: FFS whether SF-initiated Sensing Stop procedure is class 1 or class 2.

--8.x	Sensing Initiation and Reporting

Editor’s Note: the following may need further refinement.

Figure 8.x-1: Message flow for sensing initiation and reporting

1.	The SF sends sensing request to the gNB.

2.	The gNB sends sensing response to the SF.

3.	If requested in Step 1, the gNB sends sensing report to the SF.

Editor’s Note: FFS whether Sensing Report is signaling procedure

--8.y	Sensing Stop

Editor’s Note: the following may need further refinement.

The SF initiated sensing stop procedure is illustrated in Figure 8.y.1.

Figure 8.y-1: Message flow for sensing stop reporting

1.	The SF sends sensing stop to the gNB, which shall stop the sensing measurement and reporting.

Editor’s Note: FFS on class 1 or class 2 message.

The gNB initiated sensing failure indication procedure is illustrated in Figure 8.y.2.

Figure 8.y-2: Message flow for sensing failure indication

1.	The gNB sends sensing failure indication to the SF.

<<<<<<<<<<<<<<<<<<<< Change end >>>>>>>>>>>>>>>>>>>>

4.2	Discussion on open issues (if time allows)

SF-initiated sensing stop

| Options | Support companies |
| --- | --- |
| Class 1 | Huawei, NEC, Samsung, CMCC |
| Class 2 | China Telecom, BUPT, QC, Tejas Networks, ZTE, Google, Nokia, Ericsson |

Information in sensing request

According to the contributions, the following information is discussed by majority with some different preference on the naming and definitions.

| Information | Support companies |
| --- | --- |
| ID for a specific sensing request | Xiaomi: measurement ID InterDigital: Task ID Nokia, CMCC: sensing ID NEC, Samsung: sensing session ID Lenovo: service ID LGE: Sensing task ID |
| Target sensing area | Huawei (sensing TAC, sensing Cell) Nokia, Oppo (TAI, cell ID, GAD, refer to SA2) NEC (minimum a gNB ID list and a cell list) CATT (sensing area ID or Cell ID/TAI) Xiaomi, Ericsson, Samsung, CMCC |
| Report mode | InterDigital (on-demand, periodic, and event-triggered reporting) Huawei, Xiaomi, NEC, Ericsson (one-time, periodic) QC (on-demand, periodic) Nokia, CATT (one-time report, periodic report, or event-triggered report) Oppo (one-shot and periodical reporting. FFS on event-triggered and semi-persistent reporting) Lenovo CMCC (periodic, event-triggered, thresholds) |

Background information from SA2

- Sensing Entity can support one time, periodical Sensing Data to the Sensing Function. Editor's note: The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

Proposal, capture the following information in the sensing request, FFS on the details

- ID for a specific sensing request

- Target sensing area

- Report mode

Editor’s Note x3, FFS on other information

Editor’s Note x4, study the definition of sensing area.

Moderator summary:

5	Open issues for next meeting

For network architecture and protocols

List the candidate protocol stack(s) for sensing data transmission, based on RAN1 and SA2 progress, considering the transmission requirements (e.g., data volumes, efficiency).

Focus on the following existing candidate protocols:

- SCTP-based

- GTP-U-based

-Websocket-based (MDT-like)

For gNB selection

Discuss the information needed, whether signalling approach is needed. (captured in the editor’s note)

For sensing procedures

Discuss whether class 1 or class 2 is used for SF-initiated stop

Discuss interface connection setup if direct connectivity is agreed by SA2.

Discuss the need of sensing modify procedure

Discuss which procedure is used to transfer the sensing support information (e.g., supported sensing area), if signalling approach is needed.

For sensing signalling

Discuss the definitions of sensing area

Discuss information needed for the agreed procedures.

Reference

| - 13.1. General Work plan, BL pCR to TR38.765 |  |  |
| --- | --- | --- |
| R3-258088 | (pCR to TR 38.765) Study on Integrated Sensing and Communication (ISAC) for NR (China Telecom, Xiaomi) | pCR |
| R3-258357 | Work plan for study on Integrated Sensing And Communication (ISAC) for NR (China Telecom, Xiaomi) | Work Plan |
| - 13.2. Network architecture Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. From RAN3#129bis: Capture logical architecture for ISAC RAN3 to focus on sensing protocol in coordination with SA2 Continue the discussion on protocol stacks, particularly the protocol stack for sensing reporting, to be aligned with SA2 conclusions. |  |  |
| R3-258114 | (TP to pCR 38.765) ISAC general aspects and protocol stacks (Xiaomi) | other |
| R3-258147 | (TP to pCR 38.765) Further discussion on Network Architecture for ISAC (ZTE Corporation) | other |
| R3-258243 | (TP to BL pCR of TR38.765) The discussion on ISAC network architecture (NEC) | other |
| R3-258116 | (TP for pCR) Network architecture for ISAC (Huawei) | other |
| R3-258130 | Network Architecture and Protocol Aspects for NR Sensing Support (Qualcomm Incorporated) | other |
| R3-258137 | NR ISAC Network Architecture (InterDigital Inc.) | discussion |
| R3-258196 | Network Architecture and Protocol stack for NR ISAC (Tejas Network Limited) | discussion |
| R3-258207 | Discussion on architecture for ISAC (Nokia, Nokia Shanghai Bell) | discussion |
| R3-258289 | Discussion on protocol stack for sensing reporting (OPPO) | discussion |
| R3-258309 | (TP to BL pCR for TR38.765) Discussion on network architecture for ISAC (CATT) | other |
| R3-258358 | On protocol stack solution for ISAC (China Telecom) | discussion |
| R3-258371 | (TP to TR 38.765) Discussion on ISAC network architecture (Lenovo) | other |
| R3-258376 | Discussion on network architecture for ISAC (Samsung) | discussion |
| R3-258454 | ISAC Logical Architecture and Protocol Stack (Ericsson) | other |
| R3-258501 | Network architecture enhancements for NR ISAC (Hanbat National University) | discussion |
| R3-258591 | Discussions on the architecture design of ISAC (China Unicom) | discussion |
| R3-258612 | Discussion on Network Architecture for ISAC (CMCC) | discussion |
| R3-258630 | (TP to BL pCR to TR 38.765) Discussions for ISAC network architecture (LG Electronics Inc.) | other |
| R3-258682 | Discussion paper on ISAC network architecuture (CEWiT) | discussion |
| - 13.3. RAN-CN procedures and signaling Study the procedures, signaling between RAN and CN to support ISAC. From RAN3#129bis: Continue the discussion on the necessary interface functions and the detailed signaling procedures for ISAC. |  |  |
| R3-258115 | (TP to pCR 38.765) Sensing procedures and singalling (Xiaomi) | other |
| R3-258117 | (TP for pCR) RAN-CN procedures and signaling for ISAC (Huawei) | other |
| R3-258359 | (TP to 38.473) On RAN-CN functions and procedures for supporting ISAC (China Telecom, BUPT) | other |
| R3-258131 | Signalling and Procedures for NR Sensing Support (Qualcomm Incorporated) | other |
| R3-258197 | RAN-CN procedures and signaling for NR ISAC (Tejas Network Limited) | other |
| R3-258138 | NR ISAC RAN-CN Procedures and Signaling (InterDigital Inc.) | discussion |
| R3-258148 | (TP to pCR 38.765) Further discussion on RAN-CN Procedures and Signalling for ISAC (ZTE Corporation) | other |
| R3-258184 | Discussion on RAN-CN procedures and signaling for ISAC (Google) | discussion |
| R3-258208 | (TP to pCR) Discussion on RAN-CN procedures and signaling for ISAC (Nokia, Nokia Shanghai Bell) | other |
| R3-258244 | (TP to BL pCR of TR38.765)The discussion on ISAC RAN-CN procedure and signaling (NEC) | other |
| R3-258257 | Discussion on RAN-CN procedures and signaling for ISAC (KPN N.V.) | discussion |
| R3-258290 | Discussion on RAN-CN procedures and signalling for sensing (OPPO) | discussion |
| R3-258310 | (TP to BL pCR for TR38.765) RAN-CN procedures and signaling on ISAC (CATT) | other |
| R3-258372 | (TP to TR 38.765) Discussion on general procedures for gNB-based sensing (Lenovo) | other |
| R3-258377 | Discussion on RAN-CN procedures and signaling (Samsung) | discussion |
| R3-258533 | Discussion on RAN-CN procedures and signaling to support ISAC with TP to pCR (Ericsson) | other |
| R3-258592 | Standard impacts of RAN-CN procedures for ISAC (China Unicom) | discussion |
| R3-258613 | Discussion on RAN-CN Procedures Supporting ISAC (CMCC) | discussion |
| R3-258631 | (TP to BL pCR to TR 38.765) Discussions for ISAC RAN-CN aspects (LG Electronics Inc.) | other |
| R3-258683 | Discussion paper on ISAC procedures and signaling (CEWiT) | discussion |