3GPP TSG-RAN WG3 Meeting #130	R3-258769

# Dallas, US, 17 – 21 November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1348 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of handover for WAB-MT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei, ZTE, Samsung, CATT, China Telecom, Canon, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Agreements in RAN3#126 meeting: For HO, the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling. TS 23.501 defines - To prevent handover of a MWAB-UE towards a target MWAB-gNB, the target MWAB-gNB (i.e. during Xn handover or during N2 HO after target AMF slice control as described in step 4 in clause 4.9.1.3.2 of TS 23.502 [3]) fails the handover as specified in TS 38.401 [42] because the dedicated slices for BH PDU sessions of the MWAB-UE are not supported by the target MWAB-gNB. However, the target NG-RAN node behavior is missing in NG-based HO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that target NG-RAN node act as defined in TS 38.401, when S-NSSAI dedicated to WAB-MT’s backhaul PDU session(s) is received. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing behavior for target NG-RAN node when S-NSSAI dedicated to WAB-MT’s backhaul PDU session(s) is received. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.401 CR 0521 TS 38.423 CR 1630 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: update the co-signing companies and the cover page content. Rev 2: clarify the text is for target NG-RAN node is a WAB-node. Rev 3: update the TP to clarify the target NG-RAN node behavior reference to TS 38.401. |  |  |  |  |  |  |  |  |

Start of Change

8.4.2	Handover Resource Allocation

8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE. The procedure uses UE-associated signalling.

8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The AMF initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node.

----- Unmodified part skipped -----

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, the NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection. Upon reception of the Network Slice Area Scope of MDT IE, the NG-RAN node shall consider that the area scope for MDT measurement collection is defined only by the Network Slice Area Scope of MDT IE and the Area Scope of MDT IE.

If the S-NSSAI dedicated to WAB-MT’s backhaul PDU session(s) is included in the PDU Session Resource Setup List IE in the HANDOVER REQUEST message, the target NG-RAN node shall act as defined in TS 38.401 [2].

Interactions with RRC Inactive Transition Report procedure:

If the RRC Inactive Transition Report Request IE is included in the HANDOVER REQUEST message and set to "subsequent state transition report", the NG-RAN node shall, if supported, send the RRC INACTIVE TRANSITION REPORT message to the AMF to report the RRC state of the UE when the UE enters or leaves RRC_INACTIVE state.

8.4.2.3	Unsuccessful Operation

End of Change