3GPP TSG-RAN WG3 Meeting #132	R3-262501

# Dalian, China, 18-22 May 2026

| CR-Form-v13.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0541 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for timing information in AI/ML-based CCO signalling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, Nokia, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2026-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | In a GNB-CU CONFIGURATION UPDATE message, gNB-CU can send a predicted CCO issue to gNB-DU but without the time indicating when the predicted CCO issue will happen. In this case, the gNB-DU’s behaviour is unclear in stage 2 description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a description to clarify that if the predicted CCO issue is signalled without the associated time information, the gNB-DU assumes that the predicted CCO issue will occur within a short time. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact towards the previous version of the specification (same release) as it only impacts the stage2 description of AI/ML for NG-RAN in split architecture. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear stage2 description of AI/ML based CCO. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.473 CR1719 TS 38.423 CR1730 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| Forge related attachments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

### 7.11.3	Data Collection and Reporting

The following information can be configured to be reported by a gNB-DU:

-	Energy Cost (EC),

-	UE performance feedback.

The collection of EC is configured through the Resource Status Reporting Initiation procedure while the reporting is performed through the Resource Status Reporting procedure. Packet delay measurement collection for UE Performance from gNB-DU is triggered through the UE Context Setup procedure or the UE Context Modification (gNB-CU initiated) procedure. The collected packet delay measurements are reported from gNB-DU to gNB-CU-UP through the Transfer of Assistance Information procedure, specified in TS 38.425 [33].

The following information can be configured to be reported by a gNB-CU-UP:

-	UE performance feedback.

The collection and reporting of UE performance feedback from a gNB-CU-UP are configured through the Data Collection Reporting Initiation procedure, while the reporting is performed through the Data Collection Reporting procedure.

The collection of UE performance feedback is triggered at successful bearer context setup or successful bearer context modification.

Signalling by a split gNB of the RLC Average UE UL/DL Throughput over the Xn interface is not supported in this version of the specification.

At any given point in time, for a list of predicted affected cell(s) and optionally beam(s), there is only one predicted CCO issue generated by a gNB-CU.

A gNB-DU may receive, in a GNB-CU CONFIGURATION UPDATE message, predicted CCO assistance information, including a predicted CCO issue, the predicted affected cell(s) and optionally beam(s), and optionally the time when the predicted CCO issue will happen. If predicted CCO assistance information is received without the time when the predicted CCO issue will happen, the gNB-DU assumes that the predicted CCO issue will occur shortly after the reception of the GNB-CU CONFIGURATION UPDATE message. A gNB-DU may also receive, in a GNB-CU CONFIGURATION UPDATE message, the future coverage configurations of a list of cell(s) and optionally beam(s) not served by the gNB-DU with their corresponding future coverage cause and the time of application of the future coverage configuration. In response, the gNB-DU may indicate to the gNB-CU, in a GNB-DU CONFIGURATION UPDATE message, a future coverage configuration for its cell(s) and optionally beam(s), and an associated time and optionally cause.

A gNB-DU may receive, in a GNB-CU CONFIGURATION UPDATE message, a notification that a previously received predicted CCO issue, associated to the list of affected cell(s) and optionally beam(s) received with the predicted CCO issue, is cancelled. In response, the gNB-DU may also notify the gNB-CU, in the GNB-DU CONFIGURATION UPDATE message, that a previously notified non-applied coverage state change, for a list of affected cell(s) and optionally beam(s) corresponding to a cancelled predicted CCO issue, has been cancelled.

==============End of change==============