3GPP TSG-RAN WG3 Meeting #130	R3-258843

# Dallas, USA, 17th – 21st Nov, 2025

Agenda item:	12.2.2

Source: 	Samsung, Qualcomm

Title: 	AI/ML-based intra-CU LTM

Document for:	pCR

# Text Proposals

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Start of changes>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 4.2	AI/ML assisted Intra-CU LTM

### 4.2.1	Use case description

Editor’s Note: Capture the description of use case

L1/L2 Triggered Mobility (LTM) is specified in TS 38.300 [2].

Intra-CU LTM is specified in TS 38.401[3].

AI/ML can be used to optimise Intra-CU LTM procedures, e.g., to enhance Network and UE performance, optimize resource allocation and reduce mobility failures.

AI/ML-assisted L3 and L1 measurements based intra-CU LTM with inference in gNB-CU are studied.

### 4.2.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

For CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.2		Input data of AI/ML based Intra-CU LTM

For AI/ML optimization of intra-CU LTM the following information can be considered as input data:

-	L3 measurement results

-	UE mobility history

-	Measured/Predicted radio resource status per cell/SSB area

-	Measured/Predicted cell-based UE trajectory

-	Historical UE’s candidate LTM cell and beam list(s)

-	Measured TA values

Editor’s Note: To be updated

#### 4.1.2.3		Output data of AI/ML based Intra-CU LTM

For AI/ML optimization of Intra-CU LTM the following information can be considered as output data:

- Candidate cell and beam for LTM HO Preparation

- Target cell and beam selection for cell switch command

- Cell(s) and beam(s) for early synchronization

- Predicted TA value(s) for early UL synchronization

- Predicted validity time of measured TA value(s)

#### 4.1.2.4		Feedback of AI/ML based Intra-CU LTM

The following data is required as feedback data for intra-CU LTM:

-	LTM target cell

-	Measured TA value

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<End of changes>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>