3GPP TSG-RAN WG3 Meeting #130	R3-258791

Dallas, Texas, USA, 17-21 November 2025

Agenda Item:		12.2.1

Source:		NEC

Title:			(pCR for TR38.745) Multi-hop UE trajectory across gNBs

Document for:		Discussion and decision

# 1	Text Proposal

<Start of 1st change>

## 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: " NR; NR and NG-RAN Overall Description".

[3]	3GPP TS 38.401: "NG-RAN; Architecture description".

[4]	3GPP TS 38.423: "NG-RAN; Xn application protocol (XnAP)".

<Start of 2nd change>

## 4.1	Multiple-hop UE trajectory

### 4.1.1	Use case description

Editor’s Note: Capture the description of use case

In Rel-18, the cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to gNBs where the UE is expected to connect and these cells are listed in chronological order.

Multi-hop UE trajectory prediction is transferred to the target NG-RAN nodes via the Cell Based UE Trajectory Prediction IE in HANDOVER REQUEST message for the subsequent handovers. Once receiving the prediction, the behaviour of the receiving target NG-RAN nodes is referred to the Semantics Description of Cell Based UE Trajectory Prediction IE in section 9.2.3.180 in [4].

For the subsequent handovers, each target gNBs parallelly transmit the collected multi-hop measured UE trajectory back to the initial source gNB. In order to configure the measured UE trajectory for the subsequent hop, Data Collection Reporting Initiation procedure is in place between the initial source gNB and a subsequent gNB to request the reporting of the measured UE trajectory.

### 4.1.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions are considered for supporting multi-hop UE trajectory:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.2	Input data of multi-hop UE trajectory

To predict the multi-hop UE trajectory, a gNB may need the following information as input data:

From the UE:

-	UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

-	UE measurement report related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR

-	UE Mobility History Information

From the neighbouring RAN nodes:

-	UE Mobility History Information

-	Position, QoS parameters and the performance information of historical HO-ed UE (e.g., loss rate, delay, etc.)

-	UE handovers in the past that were successful and unsuccessful, including too-early, too-late, or handover to wrong (sub-optimal) cell, based on existing SON/RLF report mechanism

From the local node:

-	Multi-hop UE trajectory prediction

#### 4.1.2.3	Output data of multi-hop UE trajectory

The following information can be generated as output:

-	Multi-hop UE trajectory prediction, including cell identity of the cells that UE is predicted to be connected to and the associated Predicted Time UE Stays in Cell

#### 4.1.2.4	Feedback of multi-hop UE trajectory

To optimize the performance of multi-hop UE trajectory prediction, the following feedback can be considered to be collected from gNBs:

-	Legacy UE performance feedback for those UEs handed over from the source gNB

-	SON reports (e.g., RLF, CEF, RA)

<End of change>