3GPP TSG-RAN3 Meeting #130	R3-258809

# Dallas, US, 17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1579 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to OD-SIB1 Configuration Provision Status Update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, ZTE, Samsung, Huawei, CATT, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The “Else” in the OD-SIB1 Configuration Provision Status Update procedural text is unclear. The SIBx is described in section 9.1.3.30. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The text related to “Else” is restructured to avoid ambiguity. The SIBx in 9.1.3.30 is corrected to SIB26. The criticality for mandatory IEs is updated to “reject”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.18.2, 9.1.3.30 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGES START

### 8.4.18	OD-SIB1 Configuration Provision Status Update

#### 8.4.18.1	General

This procedure is initiated by an NG-RAN node2 to inform a neighbouring NG-RAN node1 about a previously admitted OD-SIB1 configuration provision being stopped.

The procedure uses non UE-associated signalling.

#### 8.4.18.2	Successful Operation

Figure 8.4.18.2-1: OD-SIB1 Configuration Provision Status Update procedure, successful operation

The NG-RAN node2 initiates the procedure by sending the OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE message to the NG-RAN node1.

Upon reception of the OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE message with the Provision Status IE set to "stopped", the NG-RAN node1 shall consider that the NG-RAN node2 has stopped the OD-SIB1 broadcasting for the previously admitted OD-SIB1 configuration for the cell identified by the NES Cell ID IE. If the Cell A ID IE is included in the OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE message, the NG-RAN node1 shall consider that a previously admitted OD-SIB1 configuration for the cell identified by the NES Cell ID IE is no longer transmitted in the cell identified by the Cell A ID IE. The NG-RAN node2 removes the stored OD-SIB1 configuration information that is no longer valid.

#### 8.4.18.3	Unsuccessful Operation

Not applicable.

#### 8.4.18.4	Abnormal Conditions

Void.

*************** Unchanged text omitted***************

#### 9.1.3.30	OD-SIB1 CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request to start or to stop the provisioning of an OD-SIB1 configuration.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Request Type | M |  |  |  | YES | reject |
| >Start |  |  |  |  |  |  |
| >>OD-SIB1 Configuration | M |  | OCTET STRING | Includes the OD-SIB1-Config IE as specified in SIB26 in TS 38.331 [10]. | – |  |
| >>NES Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>Cell A ID | O |  | NR CGI 9.2.2.7 |  | – |  |
| >Stop |  |  |  |  |  |  |
| >>NES Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

*************** Unchanged text omitted***************

#### 9.1.3.32	OD-SIB1 CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the OD-SIB1 configuration cannot be provided by the NG-RAN node2 as requested by the NG-RAN node1.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| NES Cell ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Cell A ID | O |  | NR CGI 9.2.2.7 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

*************** Unchanged text omitted***************

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

*************** Unchanged text omitted***************

-- **************************************************************

--

#### -- OD-SIB1 CONFIGURATION PROVISION REQUEST

--

-- **************************************************************

ODSIB1ConfigurationProvisionRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ODSIB1ConfigurationProvisionRequest-IEs}},

...

}

ODSIB1ConfigurationProvisionRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-RequestType								CRITICALITY reject	TYPE RequestTypeChoice							PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

RequestTypeChoice ::= CHOICE {

start							ODSIB1ConfigurationInformation,

stop							NR-CGI,

choice-extension				ProtocolIE-Single-Container { {RequestTypeChoice-ExtIEs} }

}

RequestTypeChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ODSIB1ConfigurationInformation ::= SEQUENCE {

oDSIB1Configuration					OCTET STRING,

nES-Cell-ID							NR-CGI,

cEll-A-ID							NR-CGI						OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ODSIB1ConfigurationInformation-ExtIEs} }	OPTIONAL,

...

}

ODSIB1ConfigurationInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- OD-SIB1 CONFIGURATION PROVISION FAILURE

--

-- **************************************************************

ODSIB1ConfigurationProvisionFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ ODSIB1ConfigurationProvisionFailure-IEs}},

...

}

ODSIB1ConfigurationProvisionFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-Cause									CRITICALITY ignore	TYPE Cause										PRESENCE mandatory}|

{ ID id-NES-Cell-ID								CRITICALITY reject	TYPE NR-CGI										PRESENCE mandatory}|

{ ID id-Cell-A-ID								CRITICALITY reject	TYPE NR-CGI										PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

CHANGES END