SA WG2 Meeting #S2-172	S2-2509864

17 - 21 November, 2025, Dallas, USA

3GPP TSG RAN WG1 Meeting #122bis                                       R1-2508184

Prague, Czech Republic, 13th – 17th October 2025

Title:	LS on traffic model study in RAN1

Response to:

Release:	Rel-20

Work Item:	FS_6G_Radio

Source:	RAN WG1

To:	SA WG4

CC:	RAN WG2, SA WG1, SA WG2

Contact Person:

#### Name:	Jinhuan Xia

E-mail Address:	jinhuan.xia@huawei.com

1. Overall Description:

RAN1 is studying the traffic model and reached the following agreements in RAN1#122bis meeting:

Agreement For the study traffic model(s) for 6GR AI/ML services: - A representative AI/ML service is the generative AI, e.g., as defined in TR22.870. Send LS to SA4 (cc RAN2, SA1, SA2) requesting input if any on traffic characteristics for AI/ML services.

Note: RAN1 is discussing the following options for the model:

  - Option-1a: The model is parameterized by Token, e.g., Token size, Token arrival rate, and Token delay budget.

    - Token is the minimum unit of data generated in the application layer.

    - How to associate Tokens to PHY layer packets.

    - How to reflect the variable importance of tokens.

    - Whether other parameters are additionally needed when tokens are encapsulated together into a packet, e.g., packet arrival rate, packet success rate, and packet delay.

  - Option-1b: The model is characterized by the parameters of PHY layer packet, including e.g., packet size, arrival rates, latency requirement, reliability requirement, etc.

  - Option-1c: reusing or extending the FTP-3/XR traffic model.

  - FFS other models/options need to be defined for other AI/ML services.

Agreement Study traffic modelling for evaluations related to immersive communication services including but not limited to advanced XR [e.g., TR22.870] and haptics services, - XR traffic models (in TR 38.838) are considered as starting point. - FFS the detailed modifications on the parameters to the XR traffic model, e.g., higher packet size, higher packet arrival rate, higher packet size deviation, PDB, etc. - FFS how many models need to be defined and the corresponding representative use cases. - FFS how to incorporate haptics traffic (TR26.854). Send LS to SA4 requesting input if any on the relevant traffic characteristics, RAN1 can continue the study before SA4 potential response.

2. Actions:

To SA4

ACTION: RAN1 respectfully requests input if any on the relevant traffic characteristics.

3. Date of Next RAN WG1 Meetings:

RAN WG1 Meeting#123			November 17th – 21st 2025	                                Dallas, USA

RAN WG1 Meeting#124  		February 9th – 13rd 2026	                                Gothenburg, Sweden