3GPP TSG-WG SA2 Meeting #172	S2-2509884

Dallas, US, 17 - 21 Nov 2025	(revison of S2-250)

Source:	OPPO

Title:	Proposed Interim Agreements and Conclusions for KI#5

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim Agreements and Conclusions for KI#5.

# 1	Discussion

The solutions for KI#5 are documented in Solution #1, #2, #3, #19, #23, #27, #28, #29, #30, #31, #32, #33, and #37.

A common theme of the solutions is the introduction of new Sensing Management and Sensing Processing functions to handle requests, configure sensing entities, and process sensing data. Sensing results are exposed to consumers (i.e. AF for this release) at one-time, periodically or event-triggered, often enriched with contextual information (e.g., location, velocity).

Below is a summary of the features addressed in the above solutions:

1. Sensing results and contextual information exposed to the consumer

Object profile with detail attributes for one or more objects. Attributes for each object include:

- Object ID (a unique identifier for tracking)

- Object Type/Classification (e.g., human, car, drone)

- Location (coordinates, cell ID, tracking area)

- Velocity/Speed and direction

- Detection Time

- Confidence Level for the detection/attributes

- Dimensions/Size

Simple detection indication: indicating the presence of an object in an area.

Environmental sensing: Some solutions include the capability to sense environmental conditions, such as rain detection.

Sensing QoS feedback: The result may include the actual achieved Sensing QoS (e.g., accuracy) if it differs from the requested values.

Result detail level: The consumer can request the level of detail, from a simple notification to a full report with associated information.

2. Manner of exposing sensing results to the consumer

One-Time (On-Demand): The sensing is performed once, and the result is delivered immediately in response to a single request.

Periodic: Sensing is performed and results are delivered at regular intervals specified by the consumer.

Event-Triggered: Sensing is continuous, but results are only delivered when a specific event occurs. Common events include:

- Object detection (a new object appears)

- Border crossing (object enters/exits a defined area)

- Velocity threshold (object speed exceeds a limit)

- Path deviation

3. Exposure path

Trusted AFs: Communicate directly with the Sensing Function via a new service-based interface (e.g., Nsf_SensingService).

Untrusted AFs: Interact via the NEF using existing frameworks like Nnef_EventExposure.

UE: A UE can be a consumer. Results can be delivered via:

- User Plane (data connection) via a PDU Session to a dedicated Sensing Function.

- Control Plane (signaling).

4. Transport mechanism

For large results, a data connection is preferred to avoid signaling load. For small results, Control Plane signaling is acceptable.

5. System architecture & functional split

A common architectural pattern emerges, splitting sensing responsibilities.

Sensing Management Function: A central function (called SeMF, SCF, SnCF, GSMF) that handles service requests, authorization, selection of sensing entities, and configuration. It is the control plane anchor for sensing.

Sensing Processing Function: A function (called SPF, SePF) responsible for aggregating and processing raw sensing data from multiple entities to generate the final sensing result. It may be co-located with the management function or separate.

Processing Chaining: Supports chaining multiple Sensing Processing Functions for complex, multi-step data processing (e.g., fusion, AI/ML models).

Sensing Result Storage: Introduces a storage function to cache sensing results, which can be reused for subsequent requests without triggering a new sensing procedure.

6. Sensing service request parameters

Target Area: Geographic area where sensing should be performed.

Sensing Service Type: High-level type of service requested (e.g., Object Detection, Border Crossing, Path Tracking).

Time Parameters: Start time, end time, and duration for the sensing operation.

Contextual Information: The consumer can provide information to aid sensing, such as expected object type, size, or mobility profile.

Sensing QoS: Quality requirements like accuracy, latency, confidence level, and detection probabilities.

Object Definition: Specific characteristics of the object to sense, including dimensions, material properties (e.g., Radar Cross Section), and mobility status.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-14 V1.1.0.

* * * * Start of Changes * * * *

### 7.1.Y	Agreed Principles for KI#5

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

Sensing results and contextual information exposed to the consumer may include the following information based on sensing service request:

- Object profile with detail attributes for one or more objects. Attributes for each object include:

  - Object ID (a unique identifier for tracking)

  - Object Type/Classification (e.g., human, car, drone)

  - Location (coordinates, cell ID, tracking area)

  - Velocity/Speed and direction

  - Detection Time

  - Confidence Level for the detection/attributes

  - Dimensions/Size

- Detection indication: indicating the presence of an object in an area.

- Sensing QoS feedback: The result may include the actual achieved Sensing QoS (e.g., accuracy) if it differs from the requested values.

Sensing results and contextual information may be exposed to the consumer on-demand, periodically, or triggered by event. The event trigger may include:

- Object detection

- Object in/out of a defined area

- Velocity threshold

- Path deviation

Sensing results and contextual information may be exposed to the consumer over control plane signaling or over a direct data connection between SF and the consumer.

The sensing result can be generated in multiple steps, i.e. the SF generating sensing data can be chained to process sensing data.

Sensing results stored in SF can be reused for subsequent requests without triggering gNB for sensing measurements.

The consumer sends the sensing service request to the SF, which include the following information:

- Target Area: Geographic area where sensing should be performed.

- Sensing Service Type: High-level type of service requested (e.g., Object Detection, Border Crossing, Path Tracking).

- Time Parameters: Start time, end time, and duration for the sensing operation.

- Contextual Information: The consumer can provide information to aid sensing, such as expected object type, size, or mobility profile.

- Sensing QoS: Quality requirements like accuracy, latency, confidence level, and detection probabilities.

- Object Definition: Specific characteristics of the object to sense, including dimensions, material properties (e.g., Radar Cross Section), and mobility status.

* * * * Next Change * * * *

## 8.5	Conclusions of KI#5

Sensing results and contextual information exposed to the consumer may include the following information based on sensing service request:

- Object profile with detail attributes for one or more objects. Attributes for each object include:

  - Object ID (a unique identifier for tracking)

  - Object Type/Classification (e.g., human, car, drone)

  - Location (coordinates, cell ID, tracking area)

  - Velocity/Speed and direction

  - Detection Time

  - Confidence Level for the detection/attributes

  - Dimensions/Size

- Detection indication: indicating the presence of an object in an area.

- Sensing QoS feedback: The result may include the actual achieved Sensing QoS (e.g., accuracy) if it differs from the requested values.

Sensing results and contextual information may be exposed to the consumer on-demand, periodically, or triggered by event. The event trigger may include:

- Object detection

- Object in/out of a defined area

- Velocity threshold

- Path deviation

Sensing results and contextual information may be exposed to the consumer over control plane signaling or over a direct data connection between SF and the consumer.

The sensing result can be generated in multiple steps, i.e. the SF generating sensing data can be chained to process sensing data.

Sensing results stored in SF can be reused for subsequent requests without triggering gNB for sensing measurements.

The consumer sends the sensing service request to the SF, which include the following information:

- Target Area: Geographic area where sensing should be performed.

- Sensing Service Type: High-level type of service requested (e.g., Object Detection, Border Crossing, Path Tracking).

- Time Parameters: Start time, end time, and duration for the sensing operation.

- Contextual Information: The consumer can provide information to aid sensing, such as expected object type, size, or mobility profile.

- Sensing QoS: Quality requirements like accuracy, latency, confidence level, and detection probabilities.

- Object Definition: Specific characteristics of the object to sense, including dimensions, material properties (e.g., Radar Cross Section), and mobility status.

* * * * End of Changes * * * *