SA WG2 Meeting #172	S2-2509940

17-21 November 2025, Dallas, Texas, USA	(was S2-250xxxx)

Source:	NEC, Thales

Title:	[WT#7] Work task and key issue for 6G satellite access

Document for:	Agreement

Agenda Item:	20.6.7

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract: This contribution proposes scope and key issue to Work Task 7 of the 6G study on NTN.

1. Justifications

This contribution discusses the scope and proposes a corresponding key issue to facilitate work on Work Task #7 of the approved 6G SID (SP-250806):

WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

NOTE 8:	The detailed scope for WT#7 will be coordinated and aligned with RAN

RAN#108 has agreed to include NTN in the 6G Radio SI (see RP-251881) and to design harmonized 6G Radio for TN and NTN.

This contribution builds on the baseline S2-2509796, which was postponed at the end of SA2#171.

Architectural Motivation: 6G NTN is expected to support enhanced mobility between TN and NTN, as well as multi-orbit satellite access, including mobility between LEO/MEO/GEO satellites. Furthermore, the Rel-20 RAN led Study item on 6G Scenarios and requirements (RP-25250810) aims at a harmonized 6G Radio design for TN and NTN, including their integration, while the objective in the 6G system architecture is to support 6G RAT and service continuity.

The satellite access technology has been specified in 4G and 5G/5G-Advanced, enabling a UE to use communication services from a single-orbit satellite access node with transparent or regenerative payload. Store-and-forward communication mode for NB-IoT has been added to support data transmission in cases of intermittent connectivity. UE-SAT-UE services for IMS and non-IMS service, as well as Voice for NB-IoT over a GEO satellite have also been included to support a wide range of use cases. Furthermore, idle and connected mode mobility between LTE and NR-NTN are now completing the mobility aspects in 5G-Advanced.

The range of use cases supported by 5G/5G-Advanced satellite access now also extends to providing backhaul services to base stations on board moving platforms with passengers (e.g. trains, buses, aircraft) as well as in areas where it is difficult to deploy terrestrial network links(wired or wireless).

Gap Analysis: While 5G/5G-Advanced support a single-orbit satellite communication, the lack of multi-orbit satellite access is notable. Users may experience limitations in mobility robustness, potentially leading to service interruptions due to handovers with lower-orbit deployments. Furthermore, there may be issues with congestion that a single-orbit constellation, i.e. a number of inter-connected satellites that belong to the same orbit, cannot handle, particularly in disaster-stricken areas. As a result, without multi-orbit satellite access, service availability, continuous coverage and efficient use of resources via NTNs is not fully realised.

A constellation may also refer to several inter-connected satellites that belong to different orbits, i.e. a multi-orbit constellation. Any constellation – intra- or inter-orbit, should provide a broader network coverage and a more persistent communication service than a single satellite. However, the functions on each satellite, the ground network and how to support service continuity considering multi-orbit constellation needs to be studied.  Furthermore, due to satellite and user mobility, the handover between the satellites may trigger signalling messages between the UE and the satellites, namely the source and the target satellite, as well as between the satellite and the ground network. Moreover, there are multiple ground stations; the handover procedure of a satellite in a constellation between two ground stations should also be considered.

Furthermore, while backhaul services using satellite access are nominally supported in 5G-A, there are aspects to be considered further in terms of resilience of backhaul, QoS, mobility and session management, and policy control.

Technical Impact: Support of multi-orbit satellite access and harmonization between NTN and TN, as well as enhanced connectivity via 6G NTN access in terms of reliability for backhaul purposes.

This WT and KIs would provide the following improvements:

- Ensuring resilient connectivity, efficient mobility and providing service continuity and adequate QoS support for UEs considering single- and multi-orbit constellation

- Ensuring resilient backhaul, which can provide adequate QoS support, as well as service continuity, with appropriate policy control.

2. Text proposal

It is proposed to agree the following changes to TR 23.801-01:

**** First Change (Same as S2-2509796) ****

A.X	Work Task 7

WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

NOTE 1:	The detailed scope for WT#7 will be coordinated and aligned with RAN, including multi-orbit NTN-NTN mobility handling, in Q1 2026. A checkpoint is set to Q1 2026 to revisit WT/KI descriptions if needed before discussing solutions.

NOTE 2:	This WT has IWK aspects that are scoped under WT#2.

NOTE 3:	This WT has mobility aspects that are scoped under WT#1.1.

**** Second Change (All new text) ****

# 5.X. Key Issue #X: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects

## 5.X. Key Issue #Y(7.1): Support 6G RAT for NTN in 6G system

This key issue will study how to support satellite access using 6G RAT for NTN in 6G system, considering transparent and regenerative payload architectures. The key issue will also study how to include support for:

- Store and Forward (S&F) mode.

- Policy/charging differentiation and (re)selection among TN and satellite accesses with different orbit types (LEO, MEO, GEO).

- Lawful Interception.

NOTE 1: The detailed scope on supporting 6G RAT for NTN will be coordinated and aligned with RAN.

NOTE 2:  Whether and how to re-use or enhance the 4G/5G satellite specific functions(e.g. earth fixed tracking areas, satellite specific RAT types etc.) should be evaluated.

NOTE 3:  The detailed scope of Lawful Interception will be coordinated and aligned with SA3-LI.

NOTE 4:  The deployment of CN functionalities on-board a satellite should support the optimization of the UP path (e.g. by the Inter-satellite link) and take the deployment complexity into consideration.

NOTE 5:  The mechanism design to tackle intermittent connectivity via NTN should follow the general principle designed to tackle intermittent connectivity via TN.

NOTE 6:  The policy/charging including the (re)selection among TN and satellite accesses with different orbit should be coordinated and aligned with general policy/charging framework in WT#1.2 and CT1/RAN2’s conclusions.

## 5.X. Key Issue #Z(7.2): Mobility and service continuity

This key issue will study how to support mobility and service continuity for NTN due to user or satellite mobility or other network conditions, including:

- Whether and how to enhance mobility and service continuity between TN and NTN, including support for simultaneous connectivity via TN and NTN (intra- and inter-operator cases).

- Whether and how to enhance mobility and service continuity between access links via satellitesin the same and different orbit types (LEO, MEO, GEO).

- Enhanced reliable connectivity via multiple 6G NTN access links (from different orbits) to support satellite backhaul.

- How to mitigate the effects from the use of dynamic satellite backhaul and/or feeder links on the user plane traffic between a UE and the CN and maintain system performance.

## 5.X. Key Issue #M(7.3): Interworking with legacy system/RAT

This key issue will study whether and how to support interworking for NTN with legacy system/RAT, especially the following aspects:

- Whether and how to support interworking with legacy system/RAT.

NOTE:	 How to handle interworking with legacy system/RAT will be coordinated and aligned with key issues related to WT#2.

**** End of Changes ****