3GPP SA WG2#172	S2-2509956

Dallas, US, 17-21 November, 2025	(revison of S2-250xxxx)

Source:	OPPO

Title:	Proposed Interim Agrements and Conclusions for KI#4

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim agreements update and conclusions for KI#4 for the FS_Sensing_ARC TR 23.700-14.

1	Discussion

Based on TR 23.700-14 V1.1.0, current interim conclusion only includes the basic principle on data processing, more details about data processing and sensing data transport are needed.

The following parts are missing:

1. The data transmission mechanism beteween the Sensing Entity and the Sensing Funcion.

2. Sensing data handling on data reuse for multiple sensing requests.

3. Sensing data handling when multiple Sensing Functions are involved for the same sensing request.

Besides, on the remaining EN:

Editor's note:	The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

It is already agreed that one time, periodical and event-triggered sensing service types should be supported in this release, the only thing that needs to coordinate with RAN WGs is how to configure the Sensing Entity to support these service types which is RAN WGs’ scope. Thus, it is proposed to remove the EN and update the bullet as:

“The Sensing Function can configure the Sensing Entity for data transmission to support one time, periodical and event-triggered sensing services” to avoid the overlap with RAN WGs.

This pCR is proposed to resolve the issues mentioned above.

2	Proposal

It is proposed to include the following changes in TR 23.700-14 V1.1.0.

* * * * Start of Changes * * * *

### 7.1.4	Agreed Principles for KI#4: Sensing Data and the Associated Information Collection and Transport

The following principles are agreed for KI#4:

-	Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data.

-	The Sensing Function can configure the Sensing Entity for data transmission to support one time, periodical and event-triggered sensing services

-	Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request.

-	The sensing data and the associated information are transmitted over a direct data connection from gNB to the Sensing Function for result generation.

Editor’s note: How the direct data connection is established is FFS, and will be coordinated with RAN WGs.

-	The sensing data can be reused for multiple service requests.

-	The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s).

-	One or multiple Sensing Function may be selected and chained for sensing result generation.

NOTE:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

* * * * Next Change * * * *

8.4	Conclusion for KI#4:

The following principles are concluded for KI#4:

-	Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data.

-	The Sensing Function can configure the Sensing Entity for data transmission to support one time, periodical and event-triggered sensing services

-	Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request.

-	The sensing data and the associated information are transmitted over a direct data connection from gNB to the Sensing Function for result generation.

Editor’s note: How the direct data connection is established is FFS, and will be coordinated with RAN WGs.

-	The sensing data can be reused for multiple service requests.

-	The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s).

-	One or multiple Sensing Function may be selected and chained for sensing result generation.

* * * * End of Changes * * * *