3GPP TSG-SA2 Meeting #172	S2-2509972

17 - 21 November, 2025, Dallas, USA                            (was S2-2508365)

Source:	Nokia

Title:	Proposed agreement of principles for Key Issues 6

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This document proposes to agree principles for KI#6

# 1.	Introduction

This document discusses aspects related to several KIs and proposes principles for conclusions by considering the scope, architectural assumptions and the solutions captured in the TR.

## 1.1 Discussion for KI#6:

This KI focuses on the parameters that are required to be configured at the SE by the SF and the corresponding parameters received from the SC. This KI also covers the aspect on how the concerned parameters are configured are provided to the SE. Various solutions (e.g., #27, 32) in the TR clearly specify the parameters that are required as part of the sensing configurations.

Proposal for KI#6:

Following content for KI#6 is proposed to be captured as principles for agreement.

- SC (AF) provides the sensing parameters through NEF or to the SF.

- SC (AF) uses the services exposed by NEF and/or SF based on the communication path as mentioned in the previous bullet.

- SF uses the service of AMF for providing sensing configuration to the corresponding SE; the request from SF also includes corresponding SE ID, SF ID, sensing operation ID and sensing session ID generated by the SF.

Note 1: Selection of SE is discussed in the conclusions of KI#3.

Note 2: For each sensing service request that a sensing client sends to the SF a Sensing Session ID is created by the SF. Each sensing service request is associated with only one Sensing Session ID.

Note 3: A Sensing Session ID consists of one or more Sensing Operation IDs to indicate the respective one or more sensing operations that are configured at the one or more BSs to support the respective Sensing Session ID.

- AMF includes the received request as a sensing container and uses the NGAP route to transfer the sensing container to the corresponding NG-RAN (i.e., SE ID) as specified by SF.

- Sensing request that is sent to the SE includes parameters such as sensing service type, sensing target area, sensing time parameters, sensing contextual information, requested Sensing QoS parameters (SQoS).

Note: The parameters that can be considered between the SF and the SE will be updated after alignment of some parameters (e.g., sensing service type) with RAN WGs as required.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change (all new text) * * * *

### 7.6.1	Agreed Principles for KI#6

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

The following principles are agreed for KI#6:

- SC (AF) provides the sensing parameters through NEF or to the SF.

- SC (AF) uses the services exposed by NEF and/or SF based on the communication path as mentioned in the previous bullet.

- SF uses the service of AMF for providing sensing configuration to the corresponding SE; the request from SF also includes corresponding SE ID, SF ID, sensing operation ID and sensing session ID generated by the SF.

Note 1: Selection of SE is discussed in the conclusions of KI#3.

Note 2: For each sensing service request that a sensing client sends to the SF a Sensing Session ID is created by the SF. Each sensing service request is associated with only one Sensing Session ID.

Note 3: A Sensing Session ID consists of one or more Sensing Operation IDs to indicate the respective one or more sensing operations that are configured at the one or more BSs to support the respective Sensing Session ID.

- AMF includes the received request as a sensing container and uses the NGAP route to transfer the sensing container to the corresponding NG-RAN (i.e., SE ID) as specified by SF.

- Sensing request that is sent to the SE includes parameters such as sensing service type, sensing target area, sensing time parameters, sensing contextual information, requested Sensing QoS parameters (SQoS).

Note: The parameters that can be considered between the SF and the SE will be updated after alignment of some parameters (e.g., sensing service type) with RAN WGs as required.

# * * * * End of change * * * *