SA WG2 Meeting #172	       S2-2509984

17 - 21 Nov, 2025, Dallas, US	             revision of S2-2509740

Source:	China Mobile

Title:	[WT#1.2, QoS]WT & KI for QoS Framework

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This document summarises the proposals for WT#1.2 QoS Framework. It is based on the final postponed version (S2-2509740) in SA2#171.

Discussion

This document summarises the proposals for WT#1.2 QoS Framework. It is based on the postponed version (S2-2509740) in SA2#171.

In R19, QoS differentiation for multiplexing traffic was developed and only applies to the limited types of service, i.e., RTP-based video and audio flow. However, nowadays the various applications are changing and emerging rapidly, there are much more types of applications with multiplexing traffic existing in the real network, which means that the related mechanism can not work very well so far. For the operators, it is beneficial to develop the common mechanism to support QoS differentiation for multiplexing traffic in the same application to cover as more scenarios as possible, e.g., after obtaining more granular service information, the 6G network can implement refined differentiated guarantees of service, thereby optimizing the perceived quality of service experience. The proposed text is highlighted in yellow.

**** First Change ****

Annex A.X WT#1.2.X: QoS Framework for 6G

Editor’s Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

Items that have consensus:

- Investigate if new functionality in 6G QoS framework is required, considering, e.g., the emerging new traffic characteristics and application needs.

- Whether and how to enhance QoS mechanisms to:

- support QoS targets fulfilling application requirements (e.g., dynamic QoS requirements) in a less resource intensive manner than the existing GBR, Delay Critical-GBR.

- Study whether and what enhancements are needed to adjust the QoS targets, in case the current QoS targets cannot be met, to minimize the impacts to application operation or user experience.

- Study whether and how to support the UE, application and network QoS collaboration to improve awareness in the 6GS of application traffic needs, e.g., traffic pattern, dynamic QoS requirements and awareness in the application/UE of what can be provided by the network, e.g., maximum bitrate.

NOTE 1: Applications typically consist of an Application Client in the UE and application server in the DN.

- Whether and how to enhance QoS monitoring, e.g., to determine the end-to-end packet delay between the PSA-UPF and the UE for the verification of the QoS target fulfillment.

- Study whether and what enhancements to traffic detection and identification are needed to enable QoS differentiation, e.g., in case of frequent IP address change of the server, data traffic with multiplexed media flows.

NOTES 1A: Aspects of traffic detection belongs to this WT.

- Study QoS related aspects of interworking with 5GS.

NOTE 2: Interworking aspects will be coordinated with WT#2.

NOTE 3: This WT will coordinate with SA WG4 and RAN WGs.

NOTE 4: The content of the 5GS QoS Framework that will be used as a starting point for discussion on 6G QoS will be determined by moderated discussion.

Items for further discussion:

- Study whether and how to minimize the impact from the network status fluctuations on the upper transport layer.

**** End of Changes ****