SA WG2 Meeting #S2-172	S2-2510026

17 - 21 November, 2025, Dallas, USA	(Revision of S2-2508467)

Source:	China Mobile

Title:	KI#5 Interim Agreements

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution proposes the interim agreements for Sensing_ARC / Rel-20.

- Discussion

In TR 23.700-14 V1.1.0 (2025-10), there are 13 solutions documented for KI#5. This contribution provides comprehensive analysis on all documented solutions on KI#5 and proposes the potential interim agreements for SI conclusions and normative work.

Based on the statement of Key Issue #5, the types of Sensing results, whether or not needing Sensing contextual information and the report modes should be analyzed.

-	Study what types of Sensing results with or without the Sensing contextual information defined in TS 22.137 [2] could be exposed from the Core Network to the Sensing Service Consumer.

-	How the Core Network exposes Sensing results to the Sensing Service Consumer (e.g. one time, periodic or event trigger).

Besides, throughout the whole sensing result exposure procedure, how to expose sensing result and sensing consumer should also be concluded.

| Solution | Types of Sensing results | Whether or not needing Sensing contextual information | Report modes (e.g. one time, periodic or event triggered) | Sensing consumer | How to expose sensing result |
| --- | --- | --- | --- | --- | --- |
| 1 | Based on sol of KI#5 | Based on sol of KI#5 | Based on sol of KI#5 | Based on sol of KI#5 | Based on sol of KI#5 |
| 2 | / | / | / | UE | User plane |
| 3 | / | / | Depend on the sensing service request from AF | AF | Control plane; Exposure via data connection is based on sol of KI#5 |
| 19 | / | / | / | A 3rd party AF or a UE | Control plane(via Exposure Function or directly or Mobility Function ) |
| 23 | / | / | Periodically or only once | / | / |
| 27 | Detection of an object type, tracking of an object, detection of environment conditions etc. | / | one time, periodic or event based | AF or NF | Control plane |
| 28 | Object Detection, Object Tracking | Result only or result with associated information based on AF request | One-time, event-triggered or periodic reporting | AF | Control plane |
| 29 | / | / | / | AF | data connection or signal connection |
| 30 | Object-specific information such as type, location, speed, direction, and geometric representation | Timestamp, accuracy estimation, confidence level (as defined in TS 22.137 [2]), and serving area | One-time, periodic, and event-triggered reporting | (un)trusted AF | Control plane exposure via NEF or directly |
| 31 | / | / | / | trusted AF | Control Plane or Data connection |
| 32 | Object Detection, Object Localization, Object Tracking | / | One time, Periodic, Event Triggered | AF | Control Plane |
| 33 | / | / | Periodic, event triggered | AF | Control Plane |
| 37 | Object Detection within a Target Sensing Service Area, Border Crossing Detection, Path Monitoring and Path Tracking | result ID and Object ID(s) | one time, periodic or triggered events | AF | Control plane exposure via NEF |

Observation 1: about report modes, over half of solutions propose to support the one-time, periodic, and event-triggered reporting based on AF request.

Observation 2: about report modes, the majority of solutions propose that AF acts as the sensing consumer.

Observation 3: about how to expose sensing result, the majority of solutions propose to support control plane exposure, and three solutions propose to support exposure with data connection.

Observation 4: For types of Sensing results and whether or not needing Sensing contextual information, few solutions have addressed this aspect, which is recommended to be reserved for further consideration.

- Proposal

It is proposed to adopt the following changes into TR 23.700-14.

*** Start of the change ***

7	Interim agreements

7.1	Agreed Principles

7.1.X	Agreed Principles for KI#5

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

- The one-time, periodic, and event-triggered reporting based on AF request should be supported.

- The sensing consumer should be AF in this release.

- Network could expose sensing result to AF via control plane or data connection.

7.2	Topics for further consideration

7.2.X	Topics for further consideration for KI#5

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#Z. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.Z) and can either be then marked as not pursued or postponed to a future release.

The following topics are for further consideration for KI#5:

- Types of Sensing results and whether or not needing Sensing contextual information are FFS

*** End of the change ***