SA WG2 Meeting #S2-172	S2-2510041

17 - 21 November, 2025, Dallas, USA	(revision of S2-250xxxx)

Source: 	LG Electronics

Title: 	KI#1, Interim Agreements updates and Conclusions

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This paper proposes to update the interim agreements and conclusions for KI#1 in TR 23.700-04.

1	Discussion

The part highlighted in yellow is updated on top of S2-2509295 (postponed, the latest version in SA2#171).

The text highlighted in green is based on RAN2 feedback (S2-2509869/ R2-2507928).

The text highlighted in blue is our additional proposal.

2	Text Proposal

It is proposed to agree the following changes into TR 23.700-04.

* * * * Start of 1st Change * * * *

7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

-	A new 5GC entity called DCF is defined as the UP path termination entity to support the transfer of data over UP for UE data. The DCF will further transfer the received data to the AF for UE side model training. The DCF can be standalone or co-located with an existing function in 5GC.

-	A UP path is used between the UE andthe DCF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	Traffic Differentiation

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level.

The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules.

-	UP path information

-	The DCF can send its user plane information (i.e. IP address or FQDN) to the UE via a DL NAS TRANSPORT message of the AMF.

-	The operator may provide DCF(s) address information in traffic descriptors and dedicated PDU session parameters (e.g., DNN and S-NSSAI) in Route Selection Descriptors to the UE as part of the URSP rule.

-	User consent

-	The DCF performs the user consent check from UDM by reusing the existing use consent check purpose.

-	UE data transfer

-	The DCF controls UE data transfer via the secured user plane connection between UE and DCF.

-	The UE is allowed to negotiate with the 5GC about UE data transfer (e.g., accept / reject / cancel data transfer request)

-	The data transfer of the collected data over the user plane should be continued during connected mode mobility

-	Configuration and trigger for UE data collection

-	There is no paging of UEs in IDLE or INACTIVE mode to provide data reporting configuration or to trigger data collection procedure.

-	The RAN can provide a list of candidate data configuration options, and the UE can indicate the preferred data configuration options from this list.

-	The UE-side server via the CN may provide some information that is used by the RAN to create the list of candidate data configuration options and when to start or stop the data collection for a UE. There may be some dependencies between a data collection request sent from the UE-side server and a data collection configuration procedure (measurement configuration) between a UE and RAN

-	The measurement configuration for UE-side data collection is provided per UE.

-	The UE data collection will be triggered on individual UEs by the UE model training server.

* * * * Start of 2nd Change * * * *

7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#1. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.1) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	How the 5GC NF checks the reported UE data to support the full visibility of standardized UE data contents.

* * * * Start of 3rd Change * * * *

Copy from clause 7.1.1

8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

8.1.1	Conclusions for KI#1

The conclusions for KI#1 to progress normative work are as follows:

-	A new 5GC entity called DCF is defined as the UP path termination entity to support the transfer of data over UP for UE data. The DCF will further transfer the received data to the AF for UE side model training. The DCF can be standalone or co-located with an existing function in 5GC.

-	A UP path is used between the UE and the DCF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	Traffic Differentiation

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level. The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules.

-	UP path information

-	The DCF can send its user plane information (i.e. IP address or FQDN) to the UE via a DL NAS TRANSPORT message of the AMF.

-	The operator may provide DCF(s) address information in traffic descriptors and dedicated PDU session parameters (e.g., DNN and S-NSSAI) in Route Selection Descriptors to the UE as part of the URSP rule.

-	User consent

-	The DCF performs the user consent check from UDM by reusing the existing use consent check purpose.

-	UE data transfer

-	The DCF controls UE data transfer via the secured user plane connection between UE and DCF.

-	The UE is allowed to negotiate with the 5GC about UE data transfer (e.g., accept / reject / cancel data transfer request)

-	The data transfer of the collected data over the user plane should be continued during connected mode mobility

-	Configuration and trigger for UE data collection

-	There is no paging of UEs in IDLE or INACTIVE mode to provide data reporting configuration or to trigger data collection procedure.

-	The RAN can provide a list of candidate data configuration options, and the UE can indicate the preferred data configuration options from this list.

-	The UE-side server via the CN may provide some information that is used by the RAN to create the list of candidate data configuration options and when to start or stop the data collection for a UE. There may be some dependencies between a data collection request sent from the UE-side server and a data collection configuration procedure (measurement configuration) between a UE and RAN

-	The measurement configuration for UE-side data collection is provided per UE.

-	The UE data collection will be triggered on individual UEs by the UE model training server.

* * * * End of Changes * * * *