SA WG2 Meeting #S2-172	S2-2510071

17 – 21 November, 2025, Dallas, USA

Source: 	Interdigital

Title: 	Proposal for KI#4 conclusion

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: proposal for KI#4 conclusion.

# 1	Introduction

This contribution provides the update on interim agreements and conclusion on KI#4 on Sensing Data and Associated Information Collection and Transport.

# 2 	Proposal

It is proposed to adopt the following changes into TR 23.700-14 V1.1.0.

# * * * * First change* * * *

### 7.1.4	Agreed Principles for KI#4: Sensing Data and the Associated Information Collection and Transport

The following principles are agreed for KI#4:

-	Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data.

-	Sensing Entity can support one time, periodical, and event triggered Sensing Data to the Sensing Function.

Editor's note:	The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

-	Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request.

-	The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s).

NOTE:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

-	The sensing operation ID, assigned after authorization of sensing service request, is used throughout the sensing operation and in reporting Sensing Data or results, serving as a reference to the corresponding sensing service request by a sensing service consumer.

-	A data connection between SF and SE is established for transferring sensing data based on a signalling for requesting and configuring a sensing operation associated to a sensing operation ID.

# * * * * Second change * * * *

# 8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

## 8.4 Conclusion on KI#4

-	Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data.

-	Sensing Entity can support one time, periodical, and event triggered Sensing Data to the Sensing Function.

Editor's note:	The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

-	Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request.

-	The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s).

NOTE:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

-	The sensing operation ID, assigned after authorization of sensing service request, is used throughout the sensing operation and in reporting Sensing Data or results, serving as a reference to the corresponding sensing service request by a sensing service consumer.

-	A data connection between SF and SE is established for transferring sensing data based on a signalling for requesting and configuring a sensing operation associated to a sensing operation ID.

# * * * * End of change* * * *