3GPP TSG-WG2 Meeting #172	S2-2510086

November 17 – 21, 2025, Dallas, USA		(revision of (unhandled) S2-2508859)

Source:	Ericsson

Title:	Interim agreement for KI#6

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution provides agreed principles and conclusions for KI#6

# 1. Discussion

This document proposes interim agreements for KI#6.

## 1.1	Discussing KI#6 Principles

Solutions #3,15, 20, 27, 34, 35 are proposed for KI#6. However, all of them are about how to send “Configuration Parameters” as a sensing service request to a Sensing Entity/gNB.

# 2. Text Proposal

It is proposed to agree the following principles to 3GPP TR 23.700-14.

>>>> Start of Changes <<<<

### 7.1.6	Agreed Principles for KI#6 on Configuration of Parameters for Sensing Entities

-	A Sensing Entity (gNB) can determine how to perform the relevant sensing measurements based on sensing service requirements received in the sensing service message that the Sensing Function sends to the Sensing Entity in relation to the request from the sensing service consumer.

-	The Sensing Function can include the following sensing service requirements in the sensing request message to the selected Sensing Entity(gNB): Target Sensing Area, sensing type (e.g. ‘Object Detection’, ‘Object Tracking’), reporting mode for sensing results (e.g. one-time or periodic reporting).

NOTE 1:	A final list of sensing service requirements that can be provided to Sensing Entities is subject of alignment with RAN WGs.

-	The core network does not need to provide any information to the Sensing Entities prior to a sensing service request to assist with the configuration of sensing measurements.

>>>> Next Change <<<<

# 8	Conclusions

Editor's note:			This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

## 8.6	Conclusions for KI#6 on Configuration of Parameters for Sensing Entities

TBD

>>>> End of Changes <<<<