SA WG2 Meeting #172	S2-2510133

17 – 21 November, 2025, Dallas, USA	(revision of S2-250xxxx)

Source:	CATT

Title:	Update the agreed principles and add conclusion for KI#1

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This pCR is proposed to update the agreed principles and add conclusion for KI#1.

- Introduction

This pCR is proposed to update the agreed principles and add conclusion for KI#1.

**** First Change ****

7.1.1	Agreed Principles for KI#1: System Architecture to Support Sensing

To support KI#1 System Architecture to Support Sensing, the following principles are concluded:

Principle 1: one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF).

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

Principle 2: NO dedicated storage NF is needed to store the Sensing data and Sensing result.

Principle 3: In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

NOTE  4:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

NOTE 5:	Other KI's interim conclusions will be aligned with KI#1 conclusions.

Principle x: The Nx interface located between SF and Sensing Entity is a P2P interface and is used to transfer the sensing signalling(s) between SF and gNB.

NOTE x:	The protocol stack of the Nx interface, the sensing signalling(s) and the information included in the sensing signalling(s) are in scope of RAN3.

Principle y: one new Network Function (i.e. Sensing Gateway Function, SGF) is defined to support authorisation of the AF’s sensing service request. The functionalities of the SGF are as follows:

-	Support authorisation of the AF’s sensing service request as described in clause 7.1.2. If the authorisation is success, the SGF invokes the sensing service request service operation towards the SF. Otherwise the SGF rejects the sensing service request.

-	SF selection as described in clause 7.1.3.

**** Second Change ****

8.1.x	Conclusions for KI#1: System Architecture to Support Sensing

The following conclusions for Key Issue #1 of system architecture to support sensing are made:

-	One new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. The SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF).

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

-	NO dedicated storage NF is needed to store the Sensing data and Sensing result.

-	In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

-	The Nx interface located between SF and Sensing Entity is a P2P interface and is used to transfer the sensing signalling(s) between SF and gNB.

NOTE x:	The protocol stack of the Nx interface, the sensing signalling(s) and the information included in the sensing signalling(s) are in scope of RAN3.

**** End of Changes ****