3GPP TSG-SA2 Meeting #172	S2-2510138

17 – 21 November, 2025, Dallas, USA 	(revision of S2-2506749)

Source:	CATT

Title:	Discussion on TEI20 WID of QoS Aspects of MWAB using satellite

Document for:	Information

Agenda Item:	30.1

Work Item / Release:	TEI20_QoSforMWAB / Rel-20

Abstract of the contribution: this discussion paper is proposed to introduce TEI20 WID of QoS aspects of MWAB using satellite.

1. Introduction

This contribution provides discussions for the proposed TEI20 WID of QoS aspects of MWAB using satellite, including Stage 1 progress on Vehicle-Mounted Relays Phase 3 (VMR_Ph3), gap analysis and solution to support the SA1 requirement.

2. Stage 1 Progress on VMR_Ph3

Rel-20 SA1 WID on Vehicle-Mounted Relays Phase 3 (SP-241821[1]) was approved in SA#106 meeting. And the WID captures the following service requirements in TS 22.261[2]:

-	Subject to regulatory requirements and operator policy, the 5G network shall support a mechanism to determine suitable QoS parameters for traffic relayed via a mobile base station when the connection quality of the serving mobile base station relay changes, e.g. during mobility between terrestrial access network and satellite access network.

3. Support of MWAB using satellite access and MWAB mobility between terrestrial access and satellite access

3.1	Scenario

The Scenario of Mobile gNB with Wireless Access Backhauling (MWAB) using satellite access and MWAB mobility between terrestrial access and satellite access is shown in Figure 1.

Figure 1: Scenario of MWAB using satellite access and MWAB mobility between terrestrial access and satellite access

3.2	Introduction of existing mechanisms to support the scenario

3.2.1	Introduction

To support the scenario in clause 3.1, the following existing mechanisms are required:

-	MWAB is specified in clause 5.49 in TS 23.501[3] and is introduced in clause 3.2.2.

-	Satellite backhaul is specified in clause 5.43 in TS 23.501[3] and is introduced in clause 3.2.3.

3.2.2	MWAB

Mobile gNB with Wireless Access Backhauling (MWAB) provides NR access link to UEs in proximity and connects to the 5GC serving the UE through an IP connectivity provided by a backhaul PDU Session, as illustrated in Figure 2.

Figure 2: MWAB architecture for 5GS

The MWAB consists of a gNB component (MWAB-gNB) and a UE component (MWAB-UE). The PDU Session of MWAB-UE acts as the backhaul links for the MWAB-gNB.

The serving PLMN of MWAB-UE may be the same or different with the serving PLMN of UE connecting to MWAB-gNB. When the two PLMNs are the same PLMN, the architecture is shown in Figure 3.

Figure 3: Architecture for MWAB operation support - non-roaming with one PLMN

When the two PLMNs are different PLMNs, the architecture is shown in Figure 4.

Figure 4: Architecture for MWAB operation support - non-roaming with two PLMNs

3.2.3	Satellite backhaul

The satellite backhaul represents the satellite is used as part of the backhaul between RAN and 5GC. Based on the type of the satellite, multiple satellite backhaul categories are supported including GEO, MEO, LEO or OTHERSAT, DYNAMIC_GEO, DYNAMIC_MEO, DYNAMIC_LEO and DYNAMIC_OTHERSAT.

The AMF determines the satellite backhaul category based on local configuration, e.g. based on Global RAN Node IDs associated with satellite backhaul. During the PDU Session Establishment procedure, the AMF report the satellite backhaul category to SMF. Furthermore, if the AMF is aware that satellite backhaul category changes (e.g. at handover), the AMF reports the current satellite backhaul category to SMF. When the backhaul network changes from satellite to terrestrial network, the AMF reports non-satellite backhaul network is used.

When SMF receives the satellite backhaul category from AMF, the SMF reports it to the PCF. The PCF takes this information into account for the policy decision. Furthermore, the PCF may use QoS monitoring to get reports for the packet delay which is used for the policy decision.

3.3	Gap analysis

For MWAB, there are three scenarios for MWAB-gNB to use satellite backhaul, as shown in Figure 5.

Figure 5: MWAB architecture using satellite access

The description of the scenarios in Figure 5 is as follows:

-	Scenario A: The BH NG-RAN supports transparent satellite access as specified in clause 5.1 in TR 38.821[4]. The MWAB-UE uses satellite access which could be LEO, MEO or GEO. The access type of MWAB-UE represents the satellite backhaul category of MWAB-gNB.

-	Scenario B: The BH NG-RAN supports regenerative satellite access as specified in clause 5.2 in TR 38.821[4]. The MWAB-UE uses satellite access which could be LEO, MEO or GEO. The access type of MWAB-UE represents the satellite backhaul category of MWAB-gNB.

-	Scenario C: The BH NG-RAN uses satellite backhaul which could be LEO, MEO, GEO, dynamic LEO, dynamic MEO or dynamic GEO. The MWAB-UE uses terrestrial access. In this case, the satellite backhaul category of BH NG-RAN represents the satellite backhaul category of MWAB-gNB.

Along with MWAB mobility, the source BH NG-RAN handovers the MWAB-UE to the target BH NG-RAN. The handover may lead to the change of satellite backhaul category of MWAB-gNB, as follows:

-	Scenarios A and B: The MWAB-UE’s access type may change, e.g. from LEO (source BH NG-RAN) to GEO (target BH NG-RAN), from LEO (source BH NG-RAN) to terrestrial access (target BH NG-RAN). As a result, the satellite backhaul category of MWAB-gNB changes accordingly.

-	Scenario C: the backhaul category of target BH-RAN is different from the backhaul category of source BH-RAN. As a result, the backhaul category of MWAB-gNB changes accordingly.

Based on the analyses above, the satellite backhaul of the MWAB-gNB changes dynamically in different scenarios. The 5GC NF (AMF, SMF, PCF) in Broadcasted PLMN needs to know such changes to support UE PCF to make policy decision or trigger QoS monitoring. However, based on the existing satellite backhaul mechanism in clause 3.2.3, the AMF determines the change of satellite backhaul category based on local configuration. So the existing mechanism does not support the change of satellite backhaul category of gNB. This leads to the existing satellite backhaul mechanism does not work for MWAB.

3.4	Solutions and analysis

3.4.1	Introduction

This clause describes the candidate solutions to solve the gap in clause 3.3 and analyses the solutions.

3.4.2	Sol#1: Network based solution

For scenarios A and B, the backhaul type of the MWAB-gNB is the access type of MWAB-UE. Considering the serving AMF of MWAB-UE knows the access type of UE, so the AMF in Broadcasted PLMN subscribes to the RAT type change event report of the MWAB-UE from the AMF in BH PLMN via UDM. Based on the RAT Type of the MWAB-UE, the AMF in Broadcasted PLMN knows the backhaul type of the MWAB-gNB.

For scenario C, the backhaul type of the MWAB-gNB is the backhaul type of the BH NG-RAN. Considering the serving AMF of MWAB-UE knows the satellite backhaul category of BH NG-RAN, so the AMF in Broadcasted PLMN subscribes to the satellite backhaul category change event report from the AMF in BH PLMN via UDM. Based on the satellite backhaul category of the BH NG-RAN, the AMF in Broadcasted PLMN knows the backhaul type of the MWAB-gNB.

To subscribe the events above, the AMF in Broadcasted PLMN obtains the MWAB-UE identifier (e.g. GPSI) from MWAB-gNB during NG Setup.

The solution is illustrated in Figure 6.

Figure 6: sol#1 illustration

Based on Figure 6, the steps 4-6 are existing functions. The step 2 is a new function.

The step 3 in Figure 6 requires the AMF to support a new event report, i.e. satellite backhaul category change. But no new procedure is required, because the procedure in clause 4.15.4.4 in TS 23.502[5] as follows can be reused.

Figure 6a: illustration of step 3 in Figure 6

3.4.3	Sol#2: UE based solution

For scenario A and scenario B, the access type of MWAB-UE represents the backhaul type of the MWAB-gNB. Based on the description in clause 3.2.3, in MWAB scenario, the AMF needs to know the backhaul category (e.g. LEO, MEO, GEO, non-satellite category) of MWAB-gNB. In this solution, the MWAB-UE determines and provides the access type (e.g. LEO, MEO, GEO, non-satellite category) to MWAB-gNB.

When MWAB-UE access a satellite cell, based on the satellite ephemeris in NTN-Config IE in SIB19 described in TS 38.331[6], the MWAB-UE can determine the satellite access type, i.e. LEO, MEO or GEO. The MWAB-UE notifies the satellite access type to MWAB-gNB which further notifies the information to AMF in Broadcasted PLMN.

The solution apply to both scenarios A and B is illustrated in Figure 7.

Figure 7: illustration of solution applies to scenarios A and B

Based on Figure 7, the steps 1, 2 and 3 are new functions.

However the solution in Figure 7 only works in scenarios A and B. For scenario C, based on the existing mechanism, the UE has no idea of the backhaul information of the NG-RAN, so the solution in Figure 7 needs to be enhanced to solve the issue. In this solution, the AMF in BH PLMN determines the backhaul information of BH NG-RAN based on local configuration and provides the backhaul information to MWAB-UE. The solution is illustrated in Figure 8.

Figure 8: illustration of solution applies to scenarios C

Based on Figure 8, the steps 1, 2 and 3 are new functions.

3.5	Evaluation and Proposal

Based on the solutions in clause 3.4, sol#1 mainly impacts 5GC NF. Sol#2 has more impacts on MWAB-UE and N2 interfaces, as follows:

-	Impacts to MWAB-UE: The MWAB-UE needs to determine the satellite access type and receive the backhaul information of BH NG-RAN.

-	Impacts to N2 interface: when the MWAB-UE access type or the backhaul of BH NG-RAN is changed, the MWAB-gNB needs to notify the new access type and new backhaul category to AMF in Broadcasted PLMN.

To reduce impacts on MWAB-UE and N2 interface, it is proposed to use the sol#1 as the wayforward.

Proposal#1: To determine the satellite backhaul category of the MWAB-gNB, the AMF in Broadcasted PLMN subscribes the following event from the AMF in BH PLMN via UDM:

-	The RAT Type change of the MWAB-UE.

-	The satellite backhaul category change of BH NG-RAN.

Proposal#2: To subscribe the event(s) above, the AMF in Broadcasted PLMN obtains the MWAB-UE identifier from the MWAB-gNB.

4. Support of Packet Delay Budget for MWAB using satellite access and terrestrial access

4.1	Introduction of PDB

The Packet Delay Budget (PDB) includes AN PDB and CN PDB.

The AN PDB is used by RAN to support the configuration of scheduling and link layer functions (e.g. the setting of scheduling priority weights and HARQ target operating points). The RAN determines the AN PDB by subtracting CN PDB from the PDB.

For CN PDB, two mechanisms are supported to determine the value of CN PDB, i.e. static CN PDB, dynamic CN PDB.

This clause analyses the gap to support PDB for MWAB.

4.2	Gap Analysis on supporting Static CN PDB for MWAB and solution

Based on the following description in clause 5.7.3.4 in TS 23.501[3],

The 5G Access Network Packet Delay Budget (5G-AN PDB) is determined by subtracting a static value for the Core Network Packet Delay Budget (CN PDB), which represents the delay between any N6 termination point at the UPF (for any UPF that may possibly be selected for the PDU Session) and the 5G-AN from a given PDB.

The CN PDB represents the delay between UPF and RAN, so it includes the backhaul delay and transmission delay in 5GC. When static CN PDB is used, no matter what kind of backhaul (e.g. satellite backhaul, non-satellite backhaul, wireless access backhaul in the case of MWAB) is used, the RAN always thinks the CN PDB is a static value and determines the AN PDB by subtracting the static value from PDB. So the static CN PDB mechanism can apply to any backhaul type. But the static value of CN PDB needs further studied in the case of MWAB, especially when the MWAB-gNB uses the satellite backhaul.

When MWAB-UE is access LEO satellite access, the illustration of real CN packet delay is shown in Figure 9.

Figure 9: illustration of real CN packet delay when MWAB-UE uses LEO satellite access

In Figure 9, the MWAB-UE connects to the BH NG-RAN which is on board of LEO satellite at 600km. The delay assumption is as follows:

-	The maximum delay between MWAB-UE and BH NG-RAN is 39ms, the minimum delay is 12ms. These delays are calculated based on the delay described in NOTE 17 in Table 5.7.4-1 in TS 23.501[3] and the delay described in Table 7.1-1 in TS 38.821[4].

-	The transmission delay between BH NG-RAN and BH UPF is 3ms.

-	The transmission delay between BH UPF and the UE UPF is 4ms.

-	The processing delay in MWAB-UE is 2ms.

-	The processing delay in BH NG-RAN is 2ms.

-	The processing delay in UPF is 1ms. At least two UPFs are involved in this example.

Based on the assumption above, the real CN packet delay is 52ms in maximum and 25ms in minimum.

Based on the Table 5.7.4-1 in TS 23.501[3], the static CN PDB could be 1ms, 2ms, 5ms and 20ms. Based on the analysis above, the real CN packet delay is larger than the static CN PDB, this may lead to the real packet delay is larger than the PDB. In this case, the user experiences lag.

-	For delay tolerant services, e.g. www, e-mail, chat, ftp, video, the user can tolerate lag. So the existing static value of CN PDB still can work.

-	For delay sensitive services, e.g. conversational voice, interactive gaming, the user cannot tolerate lag. So the RAN needs to know the correct CN PDB and then to deduce the AN PDB to ensure the end-to-end packet delay does not exceed the PDB. In this case, new static value of CN PDB needs to be specified.

Observation#1: When MWAB-UE connects to LEO, to support the delay sensitive services for UE connecting to the MWAB-gNB, new static value for CN PDB needs to be specified.

Proposal#3: add the following NOTEs on static value for CN PDB for MWAB using LEO satellite access in Table 5.7.4-1 in TS 23.501. The NOTEs shall apply to the 5QI= 1, 2, 7, 67, 71.

NOTE X: When UE is accessing MWAB-gNB and the MWAB-UE is accessing LEO satellite at 600km, a static value for the CN PDB of ~59ms for the delay between a UPF terminating N6 and the MWAB-gNB should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. The MWAB-gNB obtains the access type of MWAB-UE via implementation dependent internal interface.

NOTE Y: When UE is accessing MWAB-gNB and the MWAB-UE is accessing LEO satellite at 1200km, a static value for the CN PDB of ~83ms for the delay between a UPF terminating N6 and the MWAB-gNB should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. The MWAB-gNB obtains the access type of MWAB-UE via implementation dependent internal interface.

When the BH NG-RAN is in terrestrial network, the real packet delay will be much smaller than the example above, so in this scenario, the existing static value for CN PDB can be reused.

When the BH NG-RAN is on board of GEO satellite, the illustration of real CN packet delay is shown in Figure 10.

Figure 10: illustration of real CN packet delay when MWAB-UE uses GEO satellite access

The difference with the delay assumption in Figure 9 is as follows:

-	The maximum delay between MWAB-UE and BH NG-RAN is 810ms, the minimum delay is 720ms. These delays are calculated based on the delay described in NOTE 17 in Table 5.7.4-1 in TS 23.501[3] and the delay described in Table 7.1-1 in TS 38.821[4].

Based on the assumption above, the real CN packet delay is 823ms in maximum and 733 in minimum. In this scenario, the real CN packet delay is much larger than the static value of CN PDB. Based on the discussion in clause 3, the UE PCF can receive the satellite backhaul category of MWAB-gNB. When the satellite backhaul category indicates GEO, the UE PCF determines to use the 5QI=10. Based on the NOTE 18 in Table 5.7.4-1 in TS 23.501[3], for 5QI=10, the static CN PDB is 290ms for GEO satellite, 41ms for LEO at 1200km and 33ms for LEO at 600km. However in the scenario in Fgure 10, the real CN packet delay is much more than 290ms, to improve user experience, it is proposed to specify new static value for CN PDB.

Observation#2: When MWAB-UE connects to GEO, for UE connecting to the MWAB-gNB, the 5QI=10 could be used, but new static value for CN PDB needs to be specified.

Proposal#4: add the following NOTE on static CN PDB for MWAB using GEO satellite access in Table 5.7.4-1 in TS 23.501. The NOTEs shall apply to the 5QI=10.

NOTE Z: When UE is accessing MWAB-gNB and the MWAB-UE is accessing GEO satellite, a static value for the CN PDB of ~830ms for the delay between a UPF terminating N6 and the MWAB-gNB should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. The MWAB-gNB obtains the access type of MWAB-UE via implementation dependent internal interface.

4.3	Gap Analyse on supporting Dynamic CN PDB for MWAB and solution

Besides the static CN PDB, SA2 also supports the dynamic CN PDB for delay-critical GBR QoS flow, as follows:

For GBR QoS Flows using the Delay-critical resource type, in order to obtain a more accurate delay budget PDB available for the NG-RAN, a dynamic value for the CN PDB, which represents the delay between the UPF terminating N6 for the QoS Flow and the 5G-AN, can be used. If used for a QoS Flow, the NG-RAN shall apply the dynamic value for the CN PDB instead of the static value for the CN PDB (which is only related to the 5QI). Different dynamic value for CN PDB may be configured per uplink and downlink direction.

The existing mechanism to support the dynamic CN PDB is as follows:

The dynamic value for the CN PDB of a Delay-critical GBR 5QI may be configured in the network in two ways:

-	Configured in each NG-RAN node, based on a variety of inputs such as different IP address(es) or TEID range of UPF terminating the N3 tunnel and based on different combinations of PSA UPF to NG-RAN under consideration of any potential I-UPF, etc;

-	Configured in the SMF, based on different combinations of PSA UPF to NG-RAN under consideration of any potential I-UPF. The dynamic value for the CN PDB for a particular QoS Flow shall be signalled to NG-RAN (during PDU Session Establishment, PDU Session Modification, Xn/N2 handover and the Service Request procedures) when the QoS Flow is established or the dynamic value for the CN PDB of a QoS Flow changes, e.g. when an I-UPF is inserted by the SMF.

If the NG-RAN node is configured locally with a dynamic value for the CN PDB for a Delay-critical GBR 5QI and receives a different value via N2 signalling for a QoS Flow with the same 5QI, local configuration in RAN node determines which value takes precedence.

Based on the analysis in clause 4.2, the MWAB-UE accessing the satellite access network introduces very large CN PDB, for example, the CN PDB is 59ms when the MWAB-UE accesses the LEO satellite access. Based on the PDB for the delay-critical GBR QoS flow in Table 5.7.4-1 in TS 23.501[3], the maximum PDB is 30ms which is much smaller than 59ms. So when the MWAB-UE accessing the satellite access network, the MWAB cannot support the delay-critical service.

Observation#3: When MWAB-UE connects to satellite access network, the delay-critical service cannot be supported by MWAB.

The MWAB can support the delay-critical services in the scenario in Figure 11.

Figure 11: Delay-critical services supported scenario for MWAB

The dynamic CN PDB aims to calculate the more accurate CN PDB and AN PDB. In the scenario in Figure 11, the UE CN PDB includes the delay between MWAB-UE and BH NG-RAN and the delay between BH NG-RNA and BH 5GC, the delays are dynamic and transparent to the SMF in Broadcasted PLMN, so the existing configuration based mechanism does not work in this scenario.

For the MWAB scenario in Figure 11, to assist the SMF in Broadcasted PLMN to determine more accurate dynamic value for CN PDB, the SMF uses the QoS monitoring to get reports for the packet delay of the BH PDU Session. The SMF takes this reported packet delay information into account to determine the dynamic value of CN PDB. To support this solution, the SMF needs to obtain the MWAB-UE identifier and the IP address of BH PDU Session.

Furthermore, the static CN PDB for delay-critical GBR 5QI also needs to be updated, the existing static CN PDB does not consider the delay of the radio interface between MWAB-UE and BH NG-RAN.

Proposal#5: update the existing NOTE 4, NOTE 5 and NOTE6 to support the scenario that the delay critical GBR 5QI is used in MWAB.

NOTE 4:	A static value for the CN PDB of 1 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. When UE is accessing MWAB-gNB, a static value for the CN PDB of 2ms should be subtracted from a given PDB.

NOTE 5:	A static value for the CN PDB of 2 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. When UE is accessing MWAB-gNB, a static value for the CN PDB of 3ms should be subtracted from a given PDB.

NOTE 6:	A static value for the CN PDB of 5 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. When UE is accessing MWAB-gNB, a static value for the CN PDB of 6ms should be subtracted from a given PDB.

Proposal#6: when UE is accessing MWAB-gNB and the MWAB-gNB is using the terrestrial backhaul, to support the dynamic CN PDB for delay-critical resource type, the following enhancements are proposed:

-	The SMF in Broadcasted PLMN uses the QoS monitoring to get reports for the packet delay of the BH PDU Session. The SMF takes this reported packet delay information into account to determine the dynamic value of CN PDB.

-	To trigger the QoS monitoring, the SMF in Broadcasted PLMN obtains the MWAB-UE identifier and the IP address of BH PDU Session from the AMF in Broadcasted PLMN.

4.4	CN PDB adaptation based on implementation

Based on offline discussion, for CN PDB adaptation, another candidate solution is that the MWAB-gNB can adapt the Admission Control and QoS parameters (e.g. the estimated CN PDB) based on the awareness of whether the BH PDU session is using a Satellite link. How this adaptation is performed is outside the scope of 3GPP.

Proposal#7: the MWAB-gNB can adapt the Admission Control and QoS parameters (e.g. the estimated CN PDB) based on the awareness of whether the BH PDU session is using a Satellite link. How this adaptation is performed is outside the scope of 3GPP.

5. Analysis on the Discussion in RAN3

Based on the meeting report of RAN3#128 meeting, the following description is included in clause 12.2 in RAN3#128 minutes[7]:

If non-terrestrial link is used between WAB MT and BH gNB and/or between BH gNB and BH CN, the WAB-gNB informs UE’s CN that the BH includes a non-terrestrial link.

FFS how a WAB node know the BH-gNB is using a non-terrestrial link. Possible options include BH-gNB informs WAB-gNB via Xn.

It seems RAN3 is also discussing the satellite backhaul issue.

Based on the scenario described in clause 3.1, the scenario is the change of satellite backhaul category of MWAB-gNB.

However, SA2 has studied and standardized how to support the satellite backhaul including the change of satellite backhaul category in Rel-18. Based on the description in clause 3.2.3, the change of satellite backhaul category impacts the PCF to make QoS and policy decision. So for the scenario described in clause 3.1, it is in SA2 scope to study whether the existing mechanism can work or not. If the gap is identified, it is also in SA2 scope to discuss how to enhance the existing mechanism standardized in Rel-18.

Based on existing mechanism, the change of satellite backhaul category of gNB impacts the PCF to make QoS and policy decision which is out of scope of RAN3 Rel-19 NR_WAB_5GFemto WID (RP-243009[8]). Furthermore, as described in clause 2, it is SA1 Rel-20 requirement to support satellite backhaul category change of MWAB-gNB, so it makes no sense that RAN3 discusses this scenario in Rel-19.

Observation#4: it is Rel-20 SA1 requirement to support the change of satellite backhaul category for MWAB-gNB.

Observation#5: the change of satellite backhaul category has been studied and standardized in SA2 in Rel-18. For MWAB case required by SA1 Rel-20 requirement, whether the existing mechanism can be used or not is in SA2 scope. If the gap is identified, it is also in SA2 scope to discuss how to enhance the existing mechanism.

Observation#6: based on SA2 specification, the satellite backhaul category change impacts the QoS and policy decision in PCF which is neither in RAN3 scope nor in RAN3 R19 WAB scope. And there is SA1 Rel-20 WI on the requirements of QoS management for VMR using NTN backhaul. It makes no sense that RAN3 discusses this issue from Rel-19.

6. Summary

Based on discussion in clause 3 and clause 4, the proposals are as follows.

To support the MWAB using satellite access and MWAB mobility between terrestrial access and satellite access, the proposals are as follows:

Proposal#1: To determine the satellite backhaul category of the MWAB-gNB, the AMF in Broadcasted PLMN subscribes the following event from the AMF in BH PLMN:

-	The RAT Type change of the MWAB-UE.

-	The satellite backhaul category change of BH NG-RAN.

Proposal#2: To subscribe the event(s) above, the AMF in Broadcasted PLMN obtains the MWAB-UE identifier from the MWAB-gNB.

To support the Packet Delay Budget for MWAB using satellite access and terrestrial access, the proposals are as follows:

Proposal#3: add the following NOTEs on static CN PDB for MWAB using LEO satellite access in Table 5.7.4-1 in TS 23.501. The NOTEs shall apply to the 5QI=1, 2, 7, 67, 71.

NOTE X: When UE is accessing MWAB-gNB and the MWAB-UE is accessing LEO satellite at 600km, a static value for the CN PDB of ~59ms for the delay between a UPF terminating N6 and the MWAB-gNB should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface.

NOTE Y: When UE is accessing MWAB-gNB and the MWAB-UE is accessing LEO satellite at 1200km, a static value for the CN PDB of ~83ms for the delay between a UPF terminating N6 and the MWAB-gNB should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface.

Proposal#4: add the following NOTE on static CN PDB for MWAB using GEO satellite access in Table 5.7.4-1 in TS 23.501. The NOTEs shall apply to the 5QI=10.

NOTE Z: When UE is accessing MWAB-gNB and the MWAB-UE is accessing GEO satellite, a static value for the CN PDB of ~830ms for the delay between a UPF terminating N6 and the MWAB-gNB should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface.

Proposal#5: update the existing NOTE 4, NOTE 5 and NOTE6 to support the scenario that the delay critical GBR 5QI is used in MWAB.

NOTE 4:	A static value for the CN PDB of 1 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. When UE is accessing MWAB-gNB, a static value for the CN PDB of 2ms should be subtracted from a given PDB.

NOTE 5:	A static value for the CN PDB of 2 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. When UE is accessing MWAB-gNB, a static value for the CN PDB of 3ms should be subtracted from a given PDB.

NOTE 6:	A static value for the CN PDB of 5 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. When UE is accessing MWAB-gNB, a static value for the CN PDB of 6ms should be subtracted from a given PDB.

Proposal#6: when UE is accessing MWAB-gNB and the MWAB-UE is using the terrestrial access, to support the dynamic CN PDB for delay-critical resource type, the following enhancements are proposed:

-	The SMF in Broadcasted PLMN uses the QoS monitoring to get reports for the packet delay of the BH PDU Session. The SMF takes this reported packet delay information into account to determine the dynamic value of CN PDB.

-	To trigger the QoS monitoring, the SMF in Broadcasted PLMN obtains the MWAB-UE identifier and the IP address of BH PDU Session from the AMF in Broadcasted PLMN.

Proposal#7 (different solution compared with proposals 3-6): the MWAB-gNB can adapt the Admission Control and QoS parameters (e.g. the estimated CN PDB) based on the awareness of whether the BH PDU session is using a Satellite link. How this adaptation is performed is outside the scope of 3GPP.

7. Reference

[1]	SP-241821, New WID on Vehicle-Mounted Relays Phase3 (VMR_Ph3), SA#106.

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TR 38.821: "Solutions for NR to support non-terrestrial networks (NTN)".

[5]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[6]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[7]	RAN3#128 minutes, RAN3 Chair.

[8]	RP-243009, Revised WID on additional topological enhancements for NR, RAN#106.