3GPP TSG-WG SA2 Meeting #172	S2-2510147

17th – 21st November 2025, Dallas, USA	   (is revision of XXXX)

Source:	Lenovo

Title:	KI#2 Interim Agreements on Device registration and DO-A transmission

Document for:	Approval

Agenda Item: 	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: This paper proposes the interim agreement principles for Key Issue 2 on Device registration and DO-A data transmission.

1	Introduction

This paper proposes the interim agreements for the key issue 2, and mainly covers the following two aspects:

- AIoT Device registration: It is proposed that the AIoT Device can perform the active registration procedure towards the core network, and can include three different types of registration, namely a. initial registration, b. mobility registration and c. periodic registration.

- DO-A data transfer from the AIoT Device to the AF via the network:

  - The trigger for AIoT device sending the DO-A data: based on device own determination, or based on AF configuration

  - The selection of AIOTF: Based on either NG-RAN local configuration, or Temporary ID that contains the AIOTF information, or based on the AIOTF information indicated by the AIoT Device

  - The routing to AF from AIOTF: AIOTF obtains the target AF information from ADM, or indicated by the AIoT Device in the uplink AIOT NAS message.

  - The context retrieval for the AIoT Device and associated AIoT service.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-30.

# * * * * First change all new texts* * * *

7	Interim agreements

  - Agreed Principles

7.1.X  Agreed Principles for KI#2

The following principles are agreed for KI#2 to support DO-A capable Ambient IoT Devices.

7.1.X.1 DO-A Capable AIoT Device Registration

DO-A capable AIoT Device can perform active registration towards the core network without being triggered, and the registration contains the following three types:

- Initial Registration: AIoT Device performs the initial registration to inform the network of its presence autonomously.

- Periodic Registration: AIoT Device performs the periodic registration either based on the timer in the AIoT Device or based on the configuration from the reader and/or the AIOTF.

- Mobility Registration: AIoT Device performs the mobility registration when realizing that it has moved out of the current AIoT Registration area, based on the broadcasted area information from the reader.

- The registration message sent from the AIOT device includes at least, e.g., the security parameters, device identification information, registration type.

Editor’s note: Other parameters within the registration message can be decided during the normative phase.

- AIOTF retrieves the device context from the last serving AIOTF using the device Temporary ID, that includes the security parameters to authenticate the AIoT Device.

Editor’s note: The security protection and the design of the AIoT Device temporary ID depends on the SA3 study.

- AIOTF stores the device registration status (registered/unregistered), registration area, and the registration type, either locally, or at the ADM.

- The ADM will notify the last serving AIOTF to release the locally stored AIoT Device context.

Editor’s note: The support of periodic registration depends on RAN1 study.

Editor’s note: It depends on RAN2 about the broadcasting of the area information for device mobility registration.

7.1.X.2 DO-A Data Transfer from DO-A Capable Device to AF

- AIoT Device sends the DO-A data to the AIOTF using the AIOT NAS message, via the reader, either directly or indirectly (via AMF). The AIOT NAS message contains DO-A data, the temporary device ID, optionally the target AF information and the AIOTF information.

  - The trigger for the AIoT Device to send the DO-A data can either be device own determination or based on the configuration information sent by the AF.

- The NG-RAN reader selects the AIOTF either based on the local configuration of OAM, or based on the AIOTF information that contained in the AIoT Device Temporary ID, or based on the AIOTF information sent by AIoT device.

- The AIOTF selects the target AF for sending the DO-A data, either based on the AF information indicated by the AIoT Device, or by retrieving the target AF information from the ADM. The storing of target AF information in the ADM is based on the SLA between the 3rd party AF and the MNO, or based on the AF request.

- The AIOTF can retrieve the AIoT Device context from the last serving AIOTF, that further includes the service parameters associated with the AIoT Device, e.g., assistance information, AF information, transaction ID, target AIoT area.

# * * * * End of changes * * * *