3GPP SA WG2 Meeting #172	S2-2510235

November 17 – 21, 2025, Dallas, TX, USA

# Title:	[DRAFT] LS on the voice codec under stringent 1kpbs physical link rate

# Response to:	-

# Release:	Release 20

# Work Item:	FS_5GSAT_Ph4_ARC

Source:	Ericsson [to become SA2]

To:	SA4, SA1

Cc:	RAN1

Contact Person:

Name:	Stefan Rommer

Tel. Number:

E-mail Address:	stefan.rommer@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

1. Background:

Under the most stringent 1kbps physical link rate assumption (see TS 22.261, Table 7.4.2-1), the protocol overhead for transporting voice media payload is challenging.

In case of IP based solutions (similar to VoLTE) for transporting voice media in IMS voice over GEO NB-IoT satellite, protocol overhead comes from radio layer and the ROHC header (i.e. the compressed IP/UDP/RTP header). The calculation below shows that the maximum available voice codec rate in the IPv6 case has to go down to 300bps or 200bps, but that there is a better situation in case of IPv4 (500bps or 400bps), depending on the size of the MAC header (see R2-2507790).

-	1kbps physical link rate

-	ptime = 80ms

-	12.5 pkts per second = 1s voice samples / 80ms

-	1000 bits per second link rate / 12.5 pkt = 80 bits per pkt = 10 bytes per pkt

-	Radio layer overhead is assumed to be 4 bytes (assuming 2 byte MAC header)

-	ROHC overhead in SO state (minimum size of UO-0 packet):

-	IPv4 = 1 byte

-	IPv6 = 3 bytes (UDP checksum mandatory)

-	Remaining for Voice codec:

-	5 bytes = 10-4(Radio layer) -1(ROHC)  5*8*12.5 = 500bps for IPv4

-	3 bytes = 10-4(Radio layer) -3(ROHC)   3*8*12.5 = 300bps for IPv6

- If the MAC header is 3 bytes, the radio layer overhead will be 5 bytes, and the codec can use 400bps for IPv4 and 200bps for IPv6.

In case of a non-IP based solution (new/reduced RTP header, without IP/UDP header, e.g. as described in solution#28 in TR23.700-19, 1.1.0) for transporting voice media, the protocol overhead comes from the radio layer and the new/reduced RTP header. The existing RTP header has a minimum size of 12 bytes, which on its own cannot be compressed by existing ROHC profiles. The key difficulty of non-IP based solution lies in optimizing the size of the current RTP header. It will be up to SA4 to determine the new RTP header, depending on the answer from SA1 on whether certain IMS functions need to be supported (see S2-2507636 Question 11). The calculation below shows the maximum available voice codec rate under different new/reduced RTP header size assumptions.

- Still assuming radio layer protocol overhead is 4 bytes (2 bytes MAC header).

- If new RTP header is 4 bits, voice codec can use 10 - 4 - 0.5= 5.5 bytes = 550bps

- If new RTP header is 1 byte, voice codec can use 10 - 4 -1 = 5 bytes = 500bps

- If new RTP header is 2 bytes, voice codec can use 10 - 4- 2 = 4 bytes = 400bps

- If the radio layer overhead is 5 bytes due to 3 bytes MAC header, the above voice codec bitrate will become (450bps, 400bps, 300bps) respectively.

- For multiple IMS calls support, additional overhead is expected for the non-IP based solution, for example, keeping the SSRC (4 bytes) field in the new RTP header, or keeping UDP source/destination port number (4 bytes), which will consume additional 400bps. In that case the maximum available voice codec rate for the non-IP solution in above calculations will become (150bps, 100bps, 0bps) respectively. Other solutions with smaller overhead may be possible, however, the key point is that the more functionality required by IMS services, the less available bit-rate is left for the voice codec.

It can be observed that in both IP and non-IP cases, there are significant challenges to satisfy the stringent 1kbps link rate requirement.

If the physical link rate is 2 kbps, an additional 1 kbps can be allocated for the voice codec beyond the previously calculated values. Consequently, in both IP or non-IP solution cases, the challenge for voice codec design is similarly reduced.

To facilitate the commercial success of the IMS over GEO NB-IoT use case, it is important to understand the operating conditions for both IP and non-IP solutions, for example, whether both can function at 1 kbps physical link rate, or whether only IPv4 but not IPv6 could work at 1kbps, whether IP solution in general requires at least 2 kbps, whether non-IP solution cannot work at 1kbps if multiple IMS calls support is required, etc. All of this will depend on SA1’s requirements on what IMS services need to be supported and SA4’s work regarding the new voice codec and the new RTP header design.

When larger ptime is used, the maximum available voice codec is better (see table below), but overall the voice codec rate has to be below the physical link rate.

| ptime | number of pkts per second | number of bytes per pkt in 1kpbs link rate | radio OH per pkt (byte) | IPv6 rohc OH per pkt (byte) | voice payload per pkt (byte) | maximum available voice codec rate (bps) IPv6 |
| --- | --- | --- | --- | --- | --- | --- |
| 80 | 12.5 | 10 | 4 | 3 | 3 | 300 |
| 160 | 6.25 | 20 | 4 | 3 | 13 | 650 |
| 320 | 3.125 | 40 | 4 | 3 | 33 | 825 |

| ptime | number of pkts per second | number of bytes per pkt in 1kpbs link rate | radio OH per pkt (byte) | IPv4 rohc OH per pkt (byte) | voice payload per pkt (byte) | maximum available voice codec rate (bps) IPv4 |
| --- | --- | --- | --- | --- | --- | --- |
| 80 | 12.5 | 10 | 4 | 1 | 5 | 500 |
| 160 | 6.25 | 20 | 4 | 1 | 15 | 750 |
| 320 | 3.125 | 40 | 4 | 1 | 35 | 875 |

| ptime | number of pkts per second | number of bytes per pkt in 1kpbs link rate | radio OH per pkt (byte) | new RTP header size (byte) | voice payload per pkt (byte) | maximum available voice codec rate (bps) non-IP |
| --- | --- | --- | --- | --- | --- | --- |
| 80 | 12.5 | 10 | 4 | 2 | 4 | 400 |
| 160 | 6.25 | 20 | 4 | 2 | 14 | 700 |
| 320 | 3.125 | 40 | 4 | 2 | 34 | 850 |

Question to SA4: Will the new voice codec and new RTP header (for non-IP case) work with the maximum available bit rate calculated above, in the most stringent 1kbps physical link?

Question to SA1: Is the 1kbps physical link rate requirement in TS 22.261, Table 7.4.2-1, really needed?

2. Actions:

To SA1 and SA4:

ACTION:

- SA2 kindly requests SA1 and SA4 to answer the questions above and provide additional feedback if necessary.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #173		09-13 Feburary, 2026	Goa, India

TSG-SA2 Meeting #174		13-17 April, 2026	Malta