SA WG2 Meeting #172	S2-2510284

17 - 21 October, 2025, Dallas, US                              (revision of S2-2508694)

Source: 	InterDigital Inc

Title: 	KI #3, Update to solution #29 for Energy Saving State and Schedule of Energy Savings State

Document for: 	Approval

Agenda Item: 	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution is a Key Issue #3 solution update for solution #29 for N3IWF and TNGF Reselection to add that Energy Saving State and Schedule of Energy Savings State can be considered for reselection. The solution update also clarifies how N3IWF and TNGF profiles can be provisioned in the NRF.

# 1	Discussion

TS 28.310 includes the following statements about the Energy Saving State:

energySaving state: state in which a cell or network function is powered-down for energy saving purposes.

Note 1: In energySaving state, the cell or network function is still controllable.

Note 2:	Void.

notEnergySaving state: state when no energy saving is in progress.

Note 3:	Void.

Conceptually, a cell or a network element or network function may be on one of these two states with respect to energy saving:

-	notEnergySaving state

-	energySaving state

When a cell is in energy saving state it may need candidate cells to pick up the load.

A cell in energySaving state is not considered as a cell outage or a fault condition.

TS 32.551 includes the following statements about the Energy Saving State:

notEnergySaving state: the default state in peak-traffic situation, with no specific energy saving in progress.

energySaving state: in an off-peak-traffic situation, some functions of a cell or a network element are powered-off or restricted in resource usage in other ways, whereas the cell or network element is still controllable.

The energySaving state represents the maximum energy saving effect on the cell level. Hardware components shall be switched off for energy saving purpose as far as possible.Which hardware components are switched off is an issue specific to NE implementation.

A key point from the SA5 text is that an NF that is in an energy savings state is still controllable but is preferably not to be selected. Thus, it is preferable to for an AMF to select an N3IWF or a TNGF that is not in the energySavings state and/or is not scheduled to soon be in the energySavings state.

This p-CR proposes to update Solution #29 so that EnergySaving state and Schedule of EnergySaving state can be considered in N3IWF and TNGF Selection.

This p-CR also clarifies that the N3IWF and TNGF profiles can be registered by the N3IWF or TNGF or via OAM.

# 2	Proposal

It is proposed to capture the following solution update in TR 23.700-67 v1.1.0.

Stars of changes

## 6.29	Solution #29: N3IWF/TNGF Reselection Based on Energy Related Information

### 6.29.0	High-level solution principles

This Key Issue #3 solution proposes that the AMF can take Energy Saving State, Schedule of Energy Savings State, Energy Priority Information or the Schedule of Energy Priority Information into account for N3IWF or TNGF reselection. The AMF can then use the Rel-19 mechanism of sending a Registration Reject message to the UE to redirect the UE to a more energy efficient N3IWF or TNGF.

NOTE 1:	Energy Priority Information and the Schedule of Energy Priority Information are defined in Solution #20. The Energy Priority Information indicates the priority to select a NF relative to other NFs of the same type considering the energy characteristics of the NF. The Schedule of the Energy Priority Information includes the value of Energy Priority information which applies at each time interval in the schedule.

NOTE 2:	Energy Saving State is defined in TS 28.310 [9] and TS 32.551 [x].

### 6.29.1	Description

This is a Key Issue #3 solution.

The solution proposes that when the AMF receives a registration request from a UE, via an N3IWF or TNGF, the AMF can obtain Energy Saving State, Schedule of Energy Savings State, Energy Priority Information or a Schedule of Energy Priority Information for N3IWF(s) or TNGF(s) from the NRF (e.g. based on solution #20) and can use that information to decide if the AMF should redirect the UE to a different N3IWF or TNGF. The redirection would use the Release 19 mechanism which is to send a Registration Reject message that identifies the N3IWF or TNGF that the UE should register though.

The solution proposes that N3IWF and TNGF profiles are stored in the NRF. The N3IWF and TNGF profiles include Energy Saving State, Schedule of Energy Savings State, Energy Priority Information or a Schedule of Energy Priority Information based on Solution #20. The AMF may obtain the profiles during the registration procedure or the profiles may be obtained before registration procedure and the information can be cached in the AMF.

The N3IWF and TNGF can provide their profiles to the NRF by invoking the Nnrf_NFManagement_NFRegister service or the N3IWF and TNGF profiles can be provided to the NRF via OAM.

### 6.29.2	Procedures

#### 6.29.2.1	N3IWF/TNGF Reselection Considering Energy Related Information

Figure 6.29.2.1-1 shows a procedure for how the AMF can redirect the UE to a more energy efficient N3IWF or TNGF.

Figure 6.29.2.1-1: N3IWF/TNGF reselection considering energy related information

1.	The UE sends a Registration Request to the AMF via a TNGF or N3IWF.

2.	The AMF queries the NRF for the NF profiles of N3IWF(s) or TNGF(s) that can serve the slices that UE is allowed to use. The AMF may have already cached the NF profiles for the N3IWF(s) or TNGF(s) from earlier queries. Based on the Energy Saving State, Schedule of Energy Savings State, Energy Priority Information or a Schedule of Energy Priority Information in the NF profiles, the AMF decides that it would be more energy efficient if the UE used a different N3IWF or TNGF.

3.	The AMF sends a Registration Reject message. The Registration Reject message provides target N3IWF information (FQDN and/or IP address) to the UE within the Registration Reject message (per clause 4.12.2.2 of TS 23.502 [3]) or provides target TNAN information (SSID, TNGF ID) to the UE within the Registration Reject message indicating to the UE to build the NAI based on the TNGF ID (per clause 4.12a.2.2 of TS 23.502 [3]).

4.	Based on the information in the Registration Reject message, the UE sends a new Registration Request to the AMF via the more energy efficient N3IWF or TNGF.

### 6.29.3	Impacts on existing services, entities and interfaces

AMF:

-	obtains NF Profile information from the NRF for N3IWF(s) and/or TNGF(s).

NRF:

-	stores NF Profile information for N3IWF(s) and/or TNGF(s) and the NF Profiles include Energy Saving State, Schedule of Energy Savings State, Energy Priority Information or a Schedule of Energy Priority Information.

-	sends NF Profile information to the AMF for N3IWF(s) and/or TNGF(s).

Next Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 22.261: "Service requirements for the 5G system".

[6]	3GPP TS 29.519: "5G System; Usage of the Unified Data Repository Service for Policy Data, Application Data and Structured Data for Exposure; Stage 3".

[7]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[8]	3GPP TS 32.255: " Telecommunication management; Charging management; 5G data connectivity domain charging; Stage 2".

[9]	3GPP TS 28.310: "Energy efficiency of 5G".

[10]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[11]	IETF RFC 2782: "A DNS RR for specifying the location of services (DNS SRV)".

[12]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[13]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[14]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[15]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[16]	3GPP TS 29.522:"5G System; Network Exposure Function Northbound APIs; Stage 3".

[17]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[18]	3GPP TS 23.527: "5G System; Restoration procedures".

[x]	3GPP TS 32.551: "Telecommunication management; Energy Saving Management (ESM); Concepts and requirements".

End of Changes