SA WG2 Meeting #172	S2-2510301

Dallas, USA, 17-21 November, 2025

Source: 	Interdigital Inc.

Title: 	TR 23.700-04: [KI#1] Agreed Principles for KI#1

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes interim agreement on principles for KI#1.

# 1	Introduction

This contribution proposes interim agreements on principles related to selection and configuration of UE to enable transfer of UE data collection. The proposal is based on the postponed proposal for interim agreement on principles for KI#1 as captured in S2-2509295.

The additional changes on top of what is captured in S2-2509295 are highlighted in yellow.

# 2 	Proposal

It is proposed to update the following contents into TR 23.700-04 V1.1.0.

# * * * * First change* * * *

### 7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

-	A new 5GC entity called DCF is defined as the UP path termination entity to support the transfer of data over UP for UE data. The DCF will further transfer the received data to the AF for UE side model training. The DCF can be standalone or co-located with an existing function in 5GC.

-	A UP path is used between the UE and a 5GC NF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	Traffic Differentiation

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level.

-	The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules, or

-	Optionally, the packet filter set (e.g., IP address / IPv6 prefix or port number of the new 5GC entity) is used in the QoS rule and the PDR to identify the packet flow used for UE data transfer.

-	DCF discovery

-	NEF discovers the DCF based on NF discovery procedure from NRF or local configuration.

-	UP path information

-	The FQDN / IP address of the DCF is self-constructed by the UE according to the URSP rule for UP path establishment. The trigger for UE establishing a UP path to the DCF is based on AF’s request or UE side implementation, or

-	Optionally, The DCF is discovered from NRF to support the UE data transfer to different AFs that need to receive data for UE side model training. DCF sends the FQND / IP address to UE.

-	DCF performs the user consent check from UDM by reusing the existing use consent check purpose.

-	The UE is allowed to negotiate with the 5GC about transfer data (e.g., accept / reject / cancel data transfer request)

-	Selection and configuration of UE for transfer of UE data collection

-	Based on the request from AF and UE’s registered capability, the DCF sends UE Data Collection configuration information (e.g. information about event for UE data collection, data type and format for the requested data collection) to the candidate UEs.

-	The DCF selects UEs for transfer of UE data collection among the UE that accepted configuration of data collection transfer. The DCF allows the data collection transfer to the AF based on user consent

-	The DCF may perform data verification by comparing the data received from the UE against the data collection transfer configuration sent to the UE. The UE may anonymize the collected data once it has been verified, prior to transferring the collected data to the AF.

-	UE may reject UE data collection during configuration of UE data Collection.

# * * * * Second change* * * *

### 7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#1. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.1) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

.

-

-	Whether and how to select UEs for UE data collection.

-	How does DCF sends UP path information to UE is FFS.

-	Whether DCF performs further processing on the transferred data from the UE (e.g., data aggregation,) as per the configuration provided by the AF (UE side model training server) is FFS.

# * * * * End of change* * * *