3GPP SA WG2 Meeting #172	S2-2510371

Dallas, US, 17 – 21 Nov, 2025

Title:	    [DRAFT] Reply LS on Clarification on 5G VN group

Response to:	LS on Clarification on 5G VN group (S2-2508180 / C4-253576)

Release:	Rel-19

Work Item:	GMEC, FS_PAIDC_UPF(CT4)

Source:	SA2

To:	CT4

Cc:

# Contact Person:

Name:	Sang-Jun Moon

Tel. Number:

E-mail Address:	moonst(at)samsung(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 would like to thank CT4’s LS (S2-2508180 / C4-253576) on Clarification on 5G VN group.

Regarding the following, SA2 would like to provide feedback.

Q1: Can a UE, that is not member (temporary/permanently) of any 5G VN group, have an SM subscription (in DNN configuration) with an S-NSSAI/DNN that is associated with a 5G VN group? Q2: Assuming an enterprise deployment with many UEs subscribed to an enterprise specific S-NSSAI/DNN, is it allowed to provision only a subset of these UEs into one 5G VN group with the same aforementioned S-NSSAI/DNN? E.g., 100 UEs are subscribed to the enterprise S-NSSAI/DNN and only 10 UEs are required to be provisioned to the S-NSSAI/DNN associated 5G VN group. (still keep 1:1 mapping between the S-NSSAI/DNN and 5G VN group that one and only one 5G VN group is associated with this specific S-NSSAI/DNN) Q3: If the answer to Q1 and Q2 is no, before the subset of UEs (of Q2) is added to a 5G VN group, this subset of UEs have no S-NSSAI/DNN subscription for the 5G VN group, does it imply that this subset of UEs needs to be re-configured with the S-NSSAI/DNN associated with 5G VN group when they are added to the 5G VN group by the AF/NEF invoking Nudm_pp service(such that these UEs start using the S-NSSAI/DNN associated with the 5G VN Group when they establish PDU sessions)? Q4: If the answer to Q3 is yes, how are the UEs re-configured? NOTE: This LS relates to the completion of the CT4 work within Rel-19 timeframe. CT4 kindly asks SA2 to reply during their #171 meeting.

Answer to Q1: No. A UE, that is not member (temporary/permanently) of any 5G VN group, cannot have an SM subscription (in DNN configuration) with an S-NSSAI/DNN that is associated with a 5G VN group.

Answer to Q2: No. For example, 100 UEs are subscribed to the enterprise S-NSSAI/DNN1 and only 10 UEs are required to be provisioned to the S-NSSAI/DNN2 associated 5G VN group.

Answer to Q3: Yes.

Answer to Q4: By OAM, the subset of UEs needs to be re-configured with the S-NSSAI/DNN associated with 5G VN group when they are added to the 5G VN group by the AF/NEF invoking Nudm_pp service.

2. Actions:

CT4:

ACTION: 	SA2 kindly asks CT4 to consider the above information.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #173		09 - 13 February, 2026	India, IN

TSG-SA2 Meeting #174		13 - 17 April, 2025	Malta, MT