SA WG2 Meeting #S2-172	S2-2510386

17 - 21 November, 2025, Dallas, USA

Title:	[DRAFT] Reply to LS on a new measurement related to the maintainability of a QoS flow

Response to:	Response to LS (S2-2509876 / S5-254627) addressing the new measurement related to the maintainability of a QoS flow

Release:	Rel-19

Work Item:	eNA_Ph3, TEI19

Source:	SA2

To:	SA5

Cc:	-

Contact person:	Adrian Buckley			adrian.buckley@verizon.com

Uri Baniel			uri.baniel@oracle.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:  TS 23.288 CR 1537

# 1	Overall description

SA2 thanks SA5 for the incoming LS and related CRs that address the new requested measurement related to the maintainability of a QoS flow (S5-254627, S2-2509876).

Degraded QoS flow is an active QoS flow that is not fulfilling the requested QoS profile attributes. As such SA2 agrees with SA5 regarding the use of the definitions in TS 23.501 clause 5.7.1.2a and clause 5.7.2.4 with respect to the GFBR, PDB, and PER attributes of the requested QoS profile to define performance measurements and KPIs for QoS flows fulfilment.

SA2 believes that the new QoS Flow Maintainability new measurement, as captured in CR 0251 (S5-254235), can address the original request for a new measurement provided the term “QF.ModNtfNbrNotFulfilled.QoS” , as defined in TS28.552 subclause 5.1.1.13.4.y (i.e. Number of QoS flows non-fulfillment notification), is further defined and clarified per the explanation below:

The non-fulfillment count must be incremented just one time when the RAN is signaling a non-fulfillment when a change from the requested QoS profile to one of the Alternate QoS Profiles (AQP) takes place.

A change from one AQP to another AQP can happen several times in the direction of QoS downgrade and also in the direction of QoS upgrade. All of these AQP to AQP changes must not be counted as non-fulfilment incidents. However, if these AQP to AQP transitions are counted as non-fulfilment, the new measurement can wrongly indicate a high number of non-fulfilment situations although there was just a small number of UEs/QoS Flows for which the RAN was adjusting the AQPs to match the best possible alternate one(s).

# 2	Actions

To SA5

ACTION:  SA2 kindly asks SA5 to address the request to have the term “QF.ModNtfNbrNotFulfilled.QoS” incremented just once when the original QoS profile is transitioned from the requested QoS profile to an Alternate QoS Profile and suppress increments of AQP to AQP transitions.

# 3	Dates of next TSG SA WG 2 meetings

SA2 #173		9 Feb – 13 Feb 2026		Goa, India

SA2 #174		13 April – 17 April 2026		Malta, MT