SA WG2 Meeting #S2-172	S2-2510388

17 - 21 November, 2025, Dallas, USA	(revision of S2-250abcd)

Source:	vivo

Title:	Conclusions on KI#2: Authorization and Revocation to Support Sensing Service

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Add new conclusion for KI#2.

1. Introduction

The contribution proposes to add new conclusion for key issue#2 based on interim agreement in 7.1.2.

More principles for KI#2 are proposed:

For authorization: the sensing authorization functionality’s action on three scenarios including authorised, partially authorised and not authorised are provided.

For revocation: the sensing authorization functionality’s action on two scenarios including current status changes, partially authorised are provided.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

8.X Conclusions for KI#2 Authorization and Revocation to Support Sensing Service

To resolve the KI#2 Authorization and Revocation to Support Sensing Service, the following conclusions are adopted for normative work:

a)	The Sensing Service authorization consists of two steps:

1)	Authorisation of the AF for Sensing Service request is performed by the NEF, if the AF is outside the trusted domain, as defined in TS 33.501 [9];

2)	Authorisation of the AF's Sensing Service Request performed by the Sensing authorization functionality;

NOTE 1:	In this clause, Sensing authorization functionality may refer to Sensing Function, Sensing Control Function or Sensing Gateway, and the wording may need alignment according to the conclusion of key issue 1.

b)	Sensing authorization functionality determines whether the Sensing Service Request from the AF is authorised, considering the example of Sensing authorisation information described in table 7.1.2-1. Additional parameters, if any, and further details on Sensing authorization information can be determined during the normative phase.

1)	If Sensing Service Request from the AF is authorised, the sensing authorization functionality performs sensing configuration based on requested sensing requirements.

2)	If Sensing Service Request from the AF is partially authorised, e.g. the Target sensing service area is partially within the Allowed area for sensing, the sensing authorization functionality performs sensing configuration based on authorized sensing requirements, e.g. within authorized Target sensing service area.

3)	If Sensing Service Request from the AF is not authorised, the sensing authorization functionality rejects the Sensing Service request with appropriate cause value, e.g. unauthorized AF, area for sensing not allowed, time period for sensing not allowed, sensing service type not allowed, etc..

Table 7.1.2-1: Sensing Authorization information for Sensing Service

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed/Not allowed area for sensing | Indicate the allowed/not allowed area for the indicated AF to trigger the sensing services operations. (Note 1) |
| Allowed/Not allowed time period for sensing | Indicate the allowed time period within which AF can trigger a particular sensing service operation |
| (Allowed) sensing service type | Indicate the allowed sensing service type (e.g., object tracking, detection) for the indicated AF to trigger the sensing services operations. |
| Note 1: the Allowed area and the Not allowed area may be both present or only one of them can be present, for example if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area only the not allowed area can be present. If both are not present all area is allowed. |  |

c)	The Sensing Authorization information is pre-configured or configured by OAM, and stored in Sensing authorization functionality for authorisation of the sensing service request;

d)	Sensing authorization functionality can revoke the Sensing Service Request when the authorisation criteria are no longer met. The sensing authorization functionality may pause the sensing service when the current status changes, e.g. current time exceeds the allowed time period for sensing.

e)	The AF can cancel the on-going Sensing Service Request when needed.

f)	There is no need for authorisation for the gNB as the Sensing Entity, as the gNB is deployed by operator.

g)	The Sensing authorization functionality may re-configure the sensing service when the authorisation criteria are partially met, e.g. the ongoing sensing service type 1 is not allowed, while the ongoing sensing service type 2 is still allowed.

# * * * * End of changes * * * *