SA WG2 Meeting #S2-172	S2-2510390

17 - 21 November, 2025, Dallas, USA

Source: 	vivo

Title: 	Conclusions on KI#4: Sensing Data and the Associated Information Collection and Transport

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution provides Conclusion on KI#4: Sensing Data and the Associated Information Collection and Transport.

1	Discussion

According to the TR 23.700-14 V1.1.0, there are 12 solutions (#1, #3, #5, #18, #21 - #26, #34, #39) related to the KI#4 Sensing Data and the Associated Information Collection and Transport.

|  | AMF or not | How to transfer sensing request | How to transfer sensing data | Who generate result | associated information for Sensing result generation |
| --- | --- | --- | --- | --- | --- |
| #1 | No | Direct interface over direct connection | Direct interface over direct connection (by UP address, transport protocol and required security information over CP or UP connection) | Sensing Function |  |
| #3 | No | Direct interface? | the path for 3GPP sensing data reporting. (establishment is triggered by SE) | SF (incl. control & processing) |  |
| #5 | Yes | CP (via AMF) | CP (via AMF) | SF |  |
| #18 | Yes & No | Direct data connection? | direct data connection (by connection info [address or end point info of SF] and binding information [e.g. Sensing Entity ID, connection ID or specific binding ID] over CP or direct interface) | SF |  |
| #21 | No | Direct interface | Direct interface | SF | correlation ID |
| #22 | No | Direct interface | Data tunnel (by exchange [SF tunnel info and sensing service ID] and [SE tunnel info and operation execution result indication]) | SF | the Sensing service ID |
| #23 | No | Direct interface? | Direct interface? | SePF (SeMF) | Sensing Service Request (SSR) Identifier, response code |
| #24 | Yes | CP | Data connection (by SDCF endpoint information (e.g. SDCF ID/ FQDN over CP) | SSMF (SDCF) |  |
| #25 | No | Direct interface? | Direction connection (by SCF identifier and address, sensing session ID, sensing service identification and SPF connection configure information) | SPF (SCF) Can be one |  |
| #26 | No | new interface defined between the SCF and the gNB. | data reporting tunnel (e.g. the IP address of the SPF, the Sensing Task ID) | SPF (SCF) Can be one | the Sensing Task ID |
| #34 | No | New interface | data connection | SPF (SCF) Can be one |  |
| #39 | Yes | CP (an enhanced version of NGAP or a new protocol for sensing defined between the endpoints as AMF and gNB can be used.) | CP | SF |  |

Based on above summary, it proposes the following conclusion for KI#4:

- Sensing data and the associated information, if any, are transferred from the Sensing Entity to the Sensing Function via the dedicated data tunnel. The data tunnel is established by exchange of tunnel information (e.g. IP address, terminal endpoint info) between the Sensing Entity and the Sensing Function. Sensing service request is transferred from the Sensing Function to the Sensing Entity to trigger sensing data and the associated information collection and transport.

- The associated information is required by the Sensing Function for sensing result generation. The required associated information includes correlation ID (e.g. sensing service ID) to bind the generated sensing result and the requested sensing service.

2 	Proposal

It is proposed to capture the following contents into TR 23.700-14.

# * * * * First change* * * *

# 8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

8.X Conclusions for KI#4 Sensing Data and the Associated Information Collection and Transport

To resolve the KI#4 for Sensing Data and the Associated Information Collection and Transport, the following conclusions are adopted for normative work:

-	Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data.

-	The Sensing Function and the Sensing Entity (gNB) have different endpoints for Sensing control messages and for Sensing Data.

-	A data tunnel is established by exchange of tunnel information (e.g. IP address, tunnel endpoint info) between the Sensing Entity and the Sensing Function.

-	Based on sensing service requirements, Sensing Entity can support one time, periodical Sensing Data and associated information, if any, transport to the Sensing Function via the dedicated data tunnel.

NOTE 1:	The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

-	Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request.

-	The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s).

-	The associated information is required by the Sensing Function for sensing result generation. The required associated information includes correlation ID (e.g. sensing service ID) to bind the generated sensing result and the requested sensing service.

NOTE 2:	Other associated information required by the Sensing Function for sensing result generation will be determined in the normative phase.

NOTE 3:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

# * * * * End of change* * * *